TSG-RAN Working Group 4 (Radio) meeting #112	R4-2411193

Maastricht, Netherlands, 19th to 23rd  August 2024

Source:				Ericsson

Title:		IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range

Agenda item:		8.2.3

Document for:		Approval

# Introduction

In last RAN#103 meeting, a new SI ([1]) was agreed triggered by an ITU-R WP5D LS ([2]) to study the IMT parameters  for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz. RAN4 had some initial discussion in last RAN4#110bis meeting [9]) and made further agreements in RAN4#111 meeting ([12]).

Based on those agreements, this contribution is further discussing the remaining open IMT parameters to be shared with ITU-R, focusing on the 7125 to 8400 MHz frequency range.

# Discussion

The following table captures the previous agreements related to IMT parameters to be answered in the LS to ITU-R, agreements made during RAN4#110b ([9]) and RAN4#111 ([12]) meetings.

| No. | Parameter | BS | UE |
| --- | --- | --- | --- |
| 1 | Duplex Method | TDD |  |
| 2 | Channel bandwidth (MHz) | 100 MHz typical To be agreed if “wider bandwidths may be possible in future” should be added |  |
| 3 | Signal bandwidth (MHz) | To be agreed |  |
| 4 | Transmitter characteristics |  |  |
| 4.1 | Power dynamic range (dB) | To be agreed | To be agreed |
| 4.2 | Spectral mask (dB) | Same as n104 for AAS To be confirmed: same as n104 for non-AAS to be agreed: ΔfOBUE | Same as n104 |
| 4.3 | ACLR (dB) | Same as n104 | To be agreed |
| 4.4 | Spurious emissions /out of band emissions | Same as n104 | Same as n104 |
| 4.5 | Maximum output power (dBm) |  | 23 dBm typical Other output powers are not precluded and refer to TR |
| 5 | Receiver characteristics |  |  |
| 5.1 | Noise figure (dB) | 6dB WA, 11dB MR, 14dB LA | To be agreed |
| 5.2 | Sensitivity (dBm) | Same as n104 | To be agreed |
| 5.3 | Blocking response | Same as n104 To be agreed: ΔfOOB | To be agreed |
| 5.4 | ACS | Same as n104 | To be agreed |
| 5.5 | SINR operating range (dB) | To be agreed | To be agreed |

## Common parameters for 7125-8400 MHz

### Channel bandwidth and signal bandwidth

The RAN4#111 Way Forward ([12]) has captured the following agreement:

As captured in the way forward, 100 MHz will be the typical channel bandwidth for this frequency range. Nevertheless, we also propose to add that wider channel bandwidth(s) are not precluded, considering this feature might be useful for such wide band. Wider channel bandwidth would need further study, depending also on the established regulations and this frequency range allocation.

To keep consistent with the previous LS response sent to ITU-R ([13]), we propose to refer to TS 38.104 sub-clause 5.3.2 and to add the formula to derive the signal bandwidth.

## BS parameters for 7125-8400 MHz

### Power dynamic range

Similar to the 3GPP response for the 6425-7125 MHz and 10.0-10.5 GHz frequency ranges, we propose to reply with the same information, i.e. 0 dB for BS.

### Emissions mask - ΔfOBUE and ΔfOOB

The RAN4#111 Way Forward ([12]) has captured the following agreement:

ΔfOBUE and ΔfOOB have already been increased to 100 MHz for band n104, considering filtering challenges for those frequencies.

As already mentioned in our past contribution (XX), we should not further relax parameters shared with ITU-R, this would make coexistence with existing services more challenging, possibly ending up with stringent requirements imposed by WP5D.

Our proposal is then to consider 100 MHz for both ΔfOBUE and ΔfOOB, as specified for band n104.

Also, we haven’t seen any issue with n104 unwanted emissions for non-AAS BS and then confirm our proposal to consider n104 unwanted emissions mask for non-AAS BS as well.

## UE parameters for 7125-8400 MHz

### Power dynamic range

Similar to the 3GPP response for the 6425-7125 MHz and 10.0-10.5 GHz frequency ranges, we propose to reply with the same information, i.e. 56 dB typical for UE (considering that 23dBm is typical output power).

### ACLR

The RAN4#110b Way Forward ([9]) has captured the following agreement:

The study on 6425-7125 GHz ([6]) concluded that UE ACLR value could be relaxed to 26dB. Nevertheless, to align with all FR1 bands, RAN4 finally decided to specify a value of 30 dB for n104 ACLR, no company had concern on feasibility.

As BS ACS value is equal to 42 dB, UE ACLR value has a major impact on the ACIR value used for coexistence study. Based on this observation, to ease coexistence study, we then propose to consider 30 dB for ACLR UE, which is consistent with band n104.

### Noise figure

The RAN4#110b Way Forward ([9]) has captured the following agreement:

Option 3 would be consistent with both option 1 and option 3. Nevertheless, it would be preferrable to indicate a single value to WP5D instead of a range (they would anyway pick most likely the highest value in their assumptions).

Considering RAN4 agreed on a 12 dB UE noise figure for band n104, we would propose to reply to ITU-R LS with that value for 7125-8400 MHz frequency range.

### Sensitivity

To avoid lengthy discussion on this topic, we propose to reply that sensitivity is “to be specified” but we could capture in the TR that the limits would be the same ones as those for band n104.

We would then also propose to align BS sensitivity information, saying it’s “to be specified” but referring to n104 sensitivity if ITU-R would need more detailed information.

### ACS

The RAN4#110b Way Forward ([9]) has captured the following agreement:

According to the upper 6GHz study ([6]), the UE ACS could have a 32 dB value in the upper 6GHz frequency range. Nevertheless, RAN4 agreed to align the band n104 ACS with the value from any other FR1 band.

Considering BS ACLR would be equal to 38 dB, the UE ACS is influencing the most the ACIR value used for coexistence. Based on this observation, to ease coexistence with adjacent services, we propose to reply to ITU-R WP5D that UE ACS should be equal to 33 dB.

### Blocking

For UE blocking, we propose to reuse the in-band blocking, out of band blocking and spurious response requirements specified in TS 38.101-1 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz (Tables 7.6.2-2 and 7.6.2-4, 7.6.3-2 and 7.6.3-4 and 7.7-2)

## SINR operating range

To keep consistent, we propose to reply to ITU-R LS with the same information we replied for 4400-4800 MHz frequency range ([13]).

## IMT parameters

Proposal1: Agree with the following Table 1 summarizing our proposals (highlighted in green) for IMT parameters in the 7125-8400 MHz frequency range.

Table 1: IMT parameters for the 7125-8400 MHz frequency range

| No. | Parameter | BS | UE |
| --- | --- | --- | --- |
| 1 | Duplex Method | TDD |  |
| 2 | Channel bandwidth (MHz) | 100 MHz typical Add a note that wider bandwidths may be possible in future |  |
| 3 | Signal bandwidth (MHz) | Will be derived from Channel Bandwidth, see TS 38.108, sub-clause 5.3.2. Signal bandwidth = NRB x SCS x 12 |  |
| 4 | Transmitter characteristics |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB | 56 dB typical |
| 4.2 | Spectral mask (dB) | Same as n104 for AAS and for non-AAS ΔfOBUE = 100 MHz | Same as n104 |
| 4.3 | ACLR (dB) | Same as n104 | Same as n104 (30 dB) |
| 4.4 | Spurious emissions /out of band emissions | Same as n104 | Same as n104 |
| 4.5 | Maximum output power (dBm) |  | 23 dBm typical Other output powers are not precluded and refer to TR |
| 5 | Receiver characteristics |  |  |
| 5.1 | Noise figure (dB) | 6dB WA, 11dB MR, 14dB LA | 12 dB |
| 5.2 | Sensitivity (dBm) | “To be specified” based on n104 | Mention “To be specified” |
| 5.3 | Blocking response | Same as n104 ΔfOOB = 100 MHz | Reuse requirements from Tables 7.6.2-3, 7.6.2-4, 7.6.3-3 and 7.6.3-4 for blocking levels and requirements from Tables 7.7-2 for spurious response |
| 5.4 | ACS | Same as n104 | Same as n104 (33 dB) |
| 5.5 | SINR operating range (dB) | See [13] |  |

## Antenna parameters

To achieve a reasonable network coverage, compromising with an acceptable antenna size, we would propose the following antenna parameters (Table 2) for the 7125-8400 MHz frequency range. A larger antenna would be needed to maintain a cell grid like 3.5 GHz one.

Proposal2: Consider the following antenna parameters (Table 2) when answering ITU LS on IMT parameters for the 7125 to 8400 MHz frequency range.

Table 2: Antenna parameters for the 7125-8400 MHz frequency range

|  |  | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (To be further discussed) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 1.2 | Element gain (dBi) | 6.4 | 6.4 | 6.4 |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |  |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |  |
| 1.6 | Antenna array configuration (Row × Column) | 8 x 16 elements | 8 x 16 elements | 8 × 8 elements |  |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |  |
| 1.7a | Number of element rows in sub-array, Msub | 3 | 3 |  |  |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub | 0.7l m | 0.7l m | 0.7l m |  |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) | 3 | 3 | 0 |  |
| 1.8 | Array Ohmic loss (dB) | 2 | 2 | 2 |  |
| 1.9 | Conducted power per antenna element/sub-array (before Ohmic loss) (dBm) | 22 | 22 | 16 |  |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 |  |
| 1.11 | Base station vertical coverage range (degrees) | 90-100 | 90-100 | 90-100 |  |
| 1.12 | Mechanical downtilt (degrees) | 6 | 6 | 6 |  |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) | 78.3 | 78.3 | 66.2 |  |

## Other

It might be difficult for RAN4 to conclude by November 2024 on all questions asked in the LS from ITU-R, as initially planned. Depending on the progress during this meeting, it might be good then to early notify ITU-R WP5D that RAN4 might need an additional meeting to conclude on its final LS Reply. ITU-R WP5D would be able to early plan for an extension of its work program on this topic.

Proposal3: Early notify ITU-R WP5D that RAN4 might only complete its study by February 2025 instead of November 2024 as initially communicated.

# Conclusion

In this contribution, we discussed the ITU-R LS and related RAN4 SI, focusing on the 7125-8400 MHz frequency range. We proposed a pragmatic approach to answer to ITU-R on time.

We made the following and proposals:

Proposal1: Agree with the following Table 1 summarizing our proposals (highlighted in green) for IMT parameters in the 7125-8400 MHz frequency range.

Proposal2: Consider the following antenna parameters (Table 2) when answering ITU LS on IMT parameters for the 7125 to 8400 MHz frequency range.

Proposal3: Early notify ITU-R WP5D that RAN4 might only complete its study by February 2025 instead of November 2024 as initially communicated.

Our companion contribution ([14]) is the draft LS Reply to ITU-R for the 7125-8400 MHz frequency range based on the past agreements and our proposals made in this contribution.

# Reference

- RP-240787, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.

- R4-2400333, ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

- INTERNATIONAL TELECOMMUNICATION UNION – RADIOCOMMUNICATION SECTOR ITU-R FAQ on INTERNATIONAL TELECOMMUNICATIONS (IMT), ITU-R FAQ on International Mobile Telecommunications (IMT)

- Recommendation ITU-R M.2150-2 (12/2023), Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)

- Report ITU-R M.2483-0, The outcome of the evaluation, consensus building and decision of the IMT-2020 process (Steps 4 to 7), including characteristics of IMT-2020 radio interfaces

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

- R4-2016973, Email discussion summary for [97e][132] FS_6425_10500MHz _NR

- R4-2406615, WF on IMT parameters for other frequency ranges, CATT

- R4-2008927, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Ericsson, ZTE

- R4-2405874, Work Plan for SI on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges, Ericsson

- R4-2410741, WF on IMT parameters study, Ericsson

- R4-2410576,  LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson

- R4-2411194, LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, Ericsson