3GPP TSG-RAN WG4 Meeting # 112	R4-2411195

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New NTN S-band – SAN RF requirements overview

Agenda item:		7.17.3

Document for:		Discussion

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new a new NTN S-band.

This contribution is giving an overview of the impact on the existing SAN RF requirements as specified in TS 38.108.

# Discussion

## General requirements – system parameters

Our proposals for the system parameters of this new NTN S-band are detailed in our other contribution [2].

The following table list the updates needed on general requirements when introducing the new NTN S-band support in TS 38.108.

| Requirements | Expected impact |
| --- | --- |
| 4. General |  |
| 4.1 Relationship with other core specifications | No impact |
| 4.2 Relationship between minimum requirements and test requirements | No impact |
| 4.3 Requirement reference points | No impact |
| 4.4 Satellite Access Node classes | No impact |
| 4.5 Regional requirements | To be updated based on considered Regulations and impacted requirements |
| 4.6 Applicability of minimum requirements | No impact |
| 5. Operating bands and channel arrangement |  |
| 5.1 General | No impact |
| 5.2 Operating bands | The FR1-NTN band tables should be updated to consider the new NTN S-band. |
| 5.3 Satellite Access Node channel bandwidth |  |
| 5.3.1 General | No impact |
| 5.3.2 Transmission bandwidth configuration | No impact |
| 5.3.3 Minimum guardband and transmission bandwidth configuration | No impact |
| 5.3.4 RB alignment | No impact |
| 5.3.5 SAN channel bandwidth per operating band | The FR1-NTN channel bandwidth table should be updated to add the new S-band and supported channel BWs. |
| 5.4 Channel arrangement |  |
| 5.4.1 Channel spacing | No impact |
| 5.4.2 Channel raster | The channel raster ranges should be added for the new NTN S-band in subclause 5.4.2.3 |
| 5.4.3 Synchronization raster | The sync raster ranges should be added for the new NTN S-band in subclause 5.4.3.3 |

## Tx requirements

The following table list the updates needed on Tx requirements when introducing the new NTN S-band support in TS 38.108.

| Requirements | Expected impacts |
| --- | --- |
| 6.1 General | No impact |
| 6.2 SAN output power | No impact |
| 6.3 Output power dynamics | No impact |
| 6.4 Transmit ON/OFF power | No impact |
| 6.5 Transmitted signal quality | No impact |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No impact |
| 6.6.2 Occupied bandwidth | No impact |
| 6.6.3 Adjacent Channel Leakage Power Ratio | No impact |
| 6.6.4 Out-of-band emissions | No impact |
| 6.6.5 Transmitter spurious emissions | No impact |
| 6.7 Transmitter intermodulation | No impact |

| Requirements | Expected impacts |
| --- | --- |
| 9.1 General | No impact |
| 9.2 Radiated transmit power | No impact |
| 9.3 OTA Satellite Access Node output power | No impact |
| 9.4 OTA output power dynamics | No impact |
| 9.5 OTA transmit ON/OFF power | No impact |
| 9.6 OTA transmitted signal quality | No impact |
| 9.7 OTA unwanted emissions |  |
| 9.7.1 General | No impact |
| 9.7.2 OTA occupied bandwidth | No impact |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio | No impact |
| 9.7.4 OTA out-of-band emissions | No impact |
| 9.7.5 OTA transmitter spurious emissions | No impact |
| 9.8 OTA transmitter intermodulation | No impact |

So far, no impact on SAN Tx RF requirement has been identified, pending on further study of FCC and ISED regulations.

## Rx requirements

The following table list the updates needed on Rx requirements when introducing the new NTN S-band support in TS 38.108.

| Requirements | Expected impact If NTN Ku-band(s) are part of FR1-NTN |
| --- | --- |
| 7.1 General | No impact |
| 7.2 Reference sensitivity level | No impact |
| 7.3 Dynamic range | No impact |
| 7.4 In-band selectivity and blocking |  |
| 7.4.1 Adjacent Channel Selectivity (ACS) | No impact |
| 7.4.2 In-band blocking | No impact |
| 7.5 Out-of-band blocking | No impact |
| 7.6 Receiver spurious emissions | No impact |
| 7.7 In-channel selectivity | No impact |

| Requirements |
| --- |
|  | Expected impact |
| 10.1 General | No impact |
| 10.2 OTA sensitivity | No impact |
| 10.3 OTA reference sensitivity level | No impact |
| 10.4 OTA dynamic range | No impact |
| 10.5 In-band selectivity and blocking |  |
| 10.5.1 Adjacent Channel Selectivity (ACS) | No impact |
| 10.5.2 OTA in-band blocking | No impact |
| 10.6 OTA out-of-band blocking | No impact |
| 10.7 OTA teceiver spurious emissions | No impact |
| 10.8 OTA in-channel selectivity | No impact |

As mentioned in our other contribution [2],MSS receivers operating in the 2000-2020 MHz frequency range shall accept interference from in-band power operations in the 1955-2000 MHz and from out of band emission in the 2000-2005 MHz (FCC CFR 47 25.265).

So far, no other impact on SAN Rx RF requirement has been identified, pending on further study of FCC and ISED regulations.

# Conclusion

In this contribution, we made a first evaluation of the impacts on the SAN RF requirements when introducing the new NTN S-band in TS 38.108.

# References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2411196, NTN S-band – System parameters and UE RF requirements, Ericsson