3GPP TSG-RAN WG4 Meeting # 112	R4-2411196

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New NTN S-band – System parameters and UE RF requirements

Agenda item:		7.17.2

Document for:		Approval

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify new NTN S-band.

This contribution is giving an overview of the impact on the existing UE RF requirements as specified in TS 38.101-5.

# Discussion

## General aspects

Based on the WID justification ([1]) and the ITU Radio Regulations, it seems this new NTN S-band is only targeting Region 2 (the frequency range 2010-2025 is allocated to MSS only in Region 2). Moreover, the WID only refers to FCC and ISED regulations, which tends to highlight this band would only target USA and Canada.

Proposal1: Clarify that the new NTN S-band is only targeting USA and Canada.

The new NTN S-band would occupy the 2000-2020 MHz frequency range in UL and 2180-2200 MHz in DL. The adjacent and overlapping bands are shown in Figure 1.

Figure 1: Adjacent and overlapping bands to the new NTN S-band

The following observations could be made:

Observation1: The new S-band UL overlaps NTN band n256, TN TDD band n34, TN band n70 DL, TN band n65 UL and TN SUL band n95.

Observation2: The new S-band UL is only 5 MHz away from TN band n25 DL and 10 MHz away from TN band n2 DL.

Observation3: The new S-band DL overlaps NTN band n256, TN bands n65 and n66 DL.

Following the same assumptions used when introducing any new NTN band (the existing TN bands shall not be impacted by the introduction of a new NTN band), TN bands n2 and n25 should then protected using the usual -50dBm/MHz UE coexistence requirement. Further discussion would be needed for the protection of TN band n70 DL, the new NTN S- band overlapping this band n70.

Proposal2: Consider the usual -50dBm/MHz UE coexistence requirement for the new S-band when protecting TN bands n25 and n2. Further discuss how to address coexistence with band n70.

RAN4 made some initial studies of the 2000-2020 MHz UL and 2180-2200 MHz DL frequency ranges in scope of the definition of a LTE band (band 23), the outcomes have been captured in TR 36.811 (5).

During this study, A-MPR simulations were made using -50dBM/MHz to protect band n2 and -40dBm/MHz to protect the “G-block” which corresponds to the last 5 MHz of band n25, also reusing the additional emission mask specified with NS_03. Further discussion would be needed to understand how to reuse those results.

## Regulations

According to FCC 47 CFR, at least the EIRP density limit as specified in 25.216 (e) requirements should be considered for this new NTN S-band. Further study on FCC and ISED regulations would be needed if other regulatory requirement shall be considered.

Proposal3: Consider the EIRP density limit (FCC 47 CFR 25.216 (e)) and further study FCC and ISED regulations.

It should also be noted that FCC mandate (25.265) the acceptance of interference in the 2000-2020 MHz for MSS receivers operating in that frequency range:

## System parameters

### Band numbering

As band 253 was specified for IoT NTN in TS 36.108, our proposal would be to specify the new NTN S-band with band number n252.

Proposal4: Specify the new NTN S-band with band number n252.

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |

With the following channel BW and SCS:

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 | 10 | 15 | 20 |  |
| n252 | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |

### Channel raster

The WID ([1]) explicitly excludes the 100 kHz channel raster from this band, only the 10 kHz channel raster should then be specified:

Based on this objective:

Proposal5: Specify the following NR-ARFCN for band n252 in table 5.4.2.3-2:

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 | Yes |

### Synchronization raster

Proposal6: Specify the following GSCN for band n252 in table 5.4.3.3-1:

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |

# Conclusion

In this contribution, we made a first evaluation of the impacts on the UE RF requirements when introducing NTN S-band in TS 38.101-5. We made the following observations and proposals:

Proposal1: Clarify that the new NTN S-band is only targeting USA and Canada.

Observation1: The new S-band UL overlaps NTN band n256, TN TDD band n34, TN band n70 DL, TN band n65 UL and TN SUL band n95.

Observation2: The new S-band UL is only 5 MHz away from TN band n25 DL and 10 MHz away from TN band n2 DL.

Observation3: The new S-band DL overlaps NTN band n256, TN bands n65 and n66 DL.

Proposal2: Consider the usual -50dBm/MHz UE coexistence requirement for the new S-band when protecting TN bands n25 and n2. Further discuss how to address coexistence with band n70.

Proposal3: Consider the EIRP density limit (FCC 47 CFR 25.216 (e)) and further study FCC and ISED regulations.

Proposal4: Specify the new NTN S-band with band number n252.

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |

With the following channel BW and SCS:

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 | 10 | 15 | 20 |  |
| n252 | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |

Proposal5: Specify the following NR-ARFCN for band n252 in table 5.4.2.3-2:

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) | Mandatory support |
| --- | --- | --- | --- | --- |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 | Yes |

Proposal6: Specify the following GSCN for band n252 in table 5.4.3.3-1:

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |

# References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TR 36.811, Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America