3GPP TSG-RAN WG4 Meeting #112	R4-2411200

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New band n68 – BS RF impacts

Agenda item:		7.16.3

Document for:		Approval

# Introduction

In last RAN#104, a new WI has been approved to specify the new NR band n68, re-farmed from LTE band 68.

This contribution is discussing the core BS RF impacts when introducing this band.

# Discussion

## Introduction

The NR band n68 is re-farmed E-UTRA band 68.

The WI objectives are:

- Specify a new NR FDD operating band n68 (the uplink band for this new band is 30 MHz, 698-728 MHz; the downlink band is 30 MHz, 753-783 MHz; the duplex gap is 25 MHz, 728-753 MHz) to support subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz and 30 kHz for 10 and 15 MHz channel bandwidths.

  - Only symmetric bandwidths are supported.

- Specify system parameters and RF characteristics of the new band.

- Address potential BS and UE co-existence issues, if any.

- Update the related technical specifications to include support for the new band .

- Specify the UE RF requirements for the new bands based on 2 Rx operation.

## General

### Band definition

The band n68 shall be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n68 | 698 MHz – 728 MHz | 753 MHz – 783 MHz | FDD |

### BS channel bandwidth

Following channel BWs shall be added to table 5.3.5-1:

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  | 5 | 10 | 15 Note |  |  |  |  |  |  |  |  |  |  |  |  |
| n68 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With the note: For the 15 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 705.5 MHz or 710.5-720.5 MHz.

The note on the 15 MHz placement restriction was motivated by the considered UE reference design when band 68 was specified (assuming reusing a band 28 duplex filter and a 15 MHz duplex duplex filters), assuring DTT protection below 694 MHz. Except if a different architecture would be considered (e.g. a single full band 68 duplex filter), this restriction would remain for band n68.

### Channel arrangement

NR-ARFCN for n68 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 100 | 139600 – <20> – 145600 | 150600 – <20> – 156600 |

And sync raster information shall be added to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n68 | 15 kHz | Case A | 1888 – <1> – 1951 |

## Tx requirements

### OBUE

Band n68 shall be added to the list of bands mentioned for Wide Area cat A and cat B option 1 limits for bands below 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2:

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n31, n68, n71, n72, n85, n105, n106, basic limits are specified in tables 6.6.4.2.1-1 and 6.6.4.2.1-1a.

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n31, n67, n68, n71, n72, n85, n105, the basic limits are specified in tables 6.6.4.2.2.1-1 and 6.6.4.2.2.1-1a.:

### Spurious

Band n68 limits shall also be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1:

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n20, n67, n28 or n68. |
|  | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. |
| E-UTRA Band 68 or NR band n68 | 753 -783 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n28 or n68. |
|  | 698-728 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n68, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.5.2.2. |
| NR Band n83 | 703 – 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. |
| NR Band n109 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93, n94 or n109 |
|  | 703 –733MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.2.2. For BS operating in Band n68, it applies for 728 MHz to 733 MHz. |

Band n68 limits shall also be added to the colocation spurious emissions limits, table 6.6.5.2.4-1:

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| E-UTRA Band 68 or NR Band n68 | 698 – 728 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

## Rx requirements

The blocking requirements for co-located BS are band agnostic.

There is so no impact on Rx requirement.

## Other BS specifications

Other BS Technical Specifications shall be updated including band n68 in the coexistence and colocations requirements tables.

# Conclusion

In this contribution, we analyzed the BS RF impacts when introducing band n68.

Proposal: Agree with the changes listed in this contribution.

Our proposal is reflected in our companion draft CRs ([2], [3] and [4]).

The impacts on other BS specifications are on our companion draft CRs ([5], [6], [7]and [8]).

# References

- RP-241664, New WID on introduction of NR band n68, Ericsson

- R4-2411205, draft CR to TS 38.104, introduction of band n68, Ericsson

- R4-2411207, draft CR to TS 38.141-1, introduction of band n68, Ericsson

- R4-2411208, draft CR to TS 38.141-2, introduction of band n68, Ericsson

- R4-2411202, draft CR to TS 36.104, introduction of band n68, Ericsson

- R4-2411203, draft CR to TS 37.104, introduction of band n68, Ericsson

- R4-2411204, draft CR to TS 37.105, introduction of band n68, Ericsson

- R4-2411209, draft CR to TS 38.133, introduction of band n68, Ericsson