3GPP TSG-RAN WG4 Meeting #112	R4-2411210

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New band n87-n88 – BS RF impacts

Agenda item:		7.15.3

Document for:		Approval

# Introduction

In last RAN#104, a new WI has been approved to specify the new NR bands n87 and n88, re-farmed from LTE bands 87 and 88.

This contribution is discussing the core BS RF impacts when introducing this band.

# Discussion

## Introduction

The NR bands n87 and n88 are re-farmed E-UTRA band 87 and 88.

The WI objectives are:

The core objectives of this WI are:

- Standardization of two new NR FDD bands n87 and n88 in the 410 MHz range based on

  - E-UTRA band 87 (UL 410 - 415, DL 420 – 425 MHz)

  - E-UTRA band 88 (UL 412 - 417, DL 422 – 427 MHz)

- During the work evaluate potential impacts of NR. If any impact on performance or any changes w.r.t. existing requirements of band 87 and 88. are found, address those during the work.

- Specify band numbering (i.e. n87 and n88) system parameters and RF characteristics for 3 and 5 MHz channel bandwidth of the new bands.

  - 15 kHz SCS for 3MHz and 5MHz channel bandwidth

  - Only symmetric bandwidths are supported.

- Address potential BS and UE co-existence issues, if any.

- Specify the UE RF requirements for the new bands based on 2 Rx operation.

- Update the related technical specifications to include support for the new band

Note: These 2 new NR bands will be introduced in a REL-independent way starting from REL-15.

## General

### Band definition

The bands n87 and n88 shall be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n87 | 410 MHz – 415 MHz | 420 MHz – 425 MHz | FDD |
| n88 | 412 MHz – 417 MHz | 422 MHz – 427 MHz | FDD |

On top of this, n87 and n88 should be added to the list of NR bands supporting NB-IoT operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n87, n88, n90, n106 which are defined in Table 5.2-1.

### BS channel bandwidth

Following channel BWs shall be added to table 5.3.5-1:

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n87 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n88 | 15 | 3 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel arrangement

NR-ARFCN for n87 and n88 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 100 | 82000 – <20> – 83000 | 84000 – <20> – 85000 |
| n88 | 100 | 82400 – <20> – 83400 | 84400 – <20> – 85400 |

And sync raster information shall be added to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 1055 – <1> – 1057 |
| n88 | 15 kHz | Case A | 1061 – <1> – 1062 |

As 3 MHz should also be supported, table 5.4.3.3-3 should also be updated:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n87 | 15 kHz | Case A | 28743 – <1> – 28754 |
| n88 | 15 kHz | Case A | 28752 – <1> – 28764 |

## Tx requirements

### OBUE

Band n68 shall be added to the list of bands mentioned for Wide Area cat A and cat B option 1 limits for bands below 1GHz, in clauses 6.6.4.2.1 and 6.6.4.2.2:

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n31, n71, n72, n85, n87, n88, n105, n106, basic limits are specified in tables 6.6.4.2.1-1 and 6.6.4.2.1-1a.

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n31, n67, n71, n72, n85, n87, n88, n105, the basic limits are specified in tables 6.6.4.2.2.1-1 and 6.6.4.2.2.1-1a.:

### Spurious

Bands n87 and n88 limits shall also be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1:

| System type for NR to co-exist with |  | Frequency range for co-existence requirement |  | Basic limits |  | Measurement bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band n87 |  | 420 - 425 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band n87 or n88. |  |
|  |  | 410 – 415 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band n87, since it is already covered by the requirement in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n88. |  |
| NR Band n88 |  | 422 - 427 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band n87 or n88. |  |
|  |  | 412 - 417 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band n88, since it is already covered by the requirement in clause 6.6.5.2.2. This requirement does not apply to BS operating in band n87. |  |

Bands n87 and n88 limits shall also be added to the colocation spurious emissions limits, table 6.6.5.2.4-1:

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| E-UTRA Band 87 or NR Band n87 | 410 – 415 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 88 or NR Band n88 | 412 – 417 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

Note that coexistence and co-location requirements were not specified for E-UTRA bands 87 and 88, they should most likely be added here.

## Rx requirements

The blocking requirements for co-located BS are now band agnostic.

There is so no impact on Rx requirement.

## Other BS specifications

Other BS Technical Specifications shall be updated including bands n87 and n88 in the coexistence and colocations requirements tables.

# Conclusion

In this contribution, we analyzed the BS RF impacts when introducing bands n87 and n88.

Proposal: Agree with the changes listed in this contribution.

Our proposal is reflected in our companion draft CRs ([2], [3] and [4]).

The impacts on other BS specifications are in our companion draft CRs ([5], [6], [7]and [8]).

# References

- RP-241663, New WID on introduction of NR bands n87 and n88, Nokia

- R4-2411214, draft CR to TS 38.104, introduction of band n87-n88, Ericsson

- R4-2411216, draft CR to TS 38.141-1, introduction of band n87-n88, Ericsson

- R4-2411217, draft CR to TS 38.141-2, introduction of band n87-n88, Ericsson

- R4-2411212, draft CR to TS 36.104, introduction of band n87-n88, Ericsson

- R4-2411213, draft CR to TS 37.104, introduction of band n87-n88, Ericsson

- R4-2411218, draft CR to TS 38.133, introduction of band n87-n88, Ericsson