3GPP TSG-RAN WG4 Meeting # 112	R4-2411219

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New NR band 1.4 Hz – System parameters

Agenda item:		7.13.2

Document for:		Approval

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify a new NR band in the 1.4 GHz frequency range.

This contribution is discussing and proposing the system parameters for this new band.

# Discussion

## System parameters

### Band numbering

Keeping NR band numbering and LTE band numbering aligned, as the highest band number used so far is number n109, considering also we proposed band number 110 for the new LTE band in the 1800-1830 MHz frequency range, our proposal would be to specify the new NR band 1.4 GHz with band number n111.

Proposal1: Specify the new NR band 1.4 GHz with band number n111.

| NR Operating Band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit /UE receive FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n111 | 1390 – 1395 | 1432 – 1435 | FDD |

With the following channel BW and SCS:

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n111 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel raster

Proposal2: Specify the following NR-ARFCN channel number for band n111 in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 100 | 278000 – <20> – 279000 | 286400 – <20> – 28700 |

### Synchronization raster

Proposal3: Specify the following SS raster entries for band n111 in table 5.4.3.3-4:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 15 kHz | Case A | 33802 – <1> – 33804 |

# Conclusion

In this contribution, we discussed the system parameters for the new NR band 1.4 GHz. We made the following proposals:

Proposal1: Specify the new NR band 1.4 GHz with band number n111.

| NR Operating Band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit /UE receive FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- | --- |
| n111 | 1390 – 1395 | 1432 – 1435 | FDD |

With the following channel BW and SCS:

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n111 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal2: Specify the following NR-ARFCN channel number for band n111 in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 100 | 278000 – <20> – 279000 | 286400 – <20> – 28700 |

Proposal3: Specify the following SS raster entries for band n111 in table 5.4.3.3-4:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n111 | 15 kHz | Case A | 33802 – <1> – 33804 |

# References

- RP-241648, Introduction of the 1.4 GHz Band, MidWave Wireless