3GPP TSG-RAN WG4 Meeting # 112	R4-2411220

# Maastricht, Netherland, August 19 – 23, 2024

Source:			Ericsson

Title:			New LTE band 1800-1830 MHz – System parameters

Agenda item:		7.14.2

Document for:		Approval

# Introduction

In last RAN#104, a new WI ([1]) has been approved to specify a new LTE band covering the 1800-1830 MHz frequency range.

This contribution is discussing and proposing the system parameters for this new band.

# Discussion

## System parameters

### Band numbering

Keeping NR band numbering and LTE band numbering aligned, as the highest band number used so far is number n109, our proposal would be to specify the new LTE band 1800-1830 MHz with band number 110.

Proposal1: Specify the new LTE band 1800-1830 MHz with band number 110.

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| 110 | 1800 MHz – 1810 MHz | 1820 MHz – 1830 MHz | FDD |

With the following channel BW and SCS:

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 110 | Yes | Yes | Yes | Yes |  |  |

### Channel raster

Proposal2: Specify the following E-UTRA channel number for band 110 in table 5.7.3-1:

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 106 (NOTE 6) | 935 | 70656 | 70656 - 70705 | 896 | 134292 | 134292 - 134341 |
| 107 (NOTE 7) | 612 | 70706 | 70706 – 71105 | N/A |  |  |
| 108 (NOTE 7) | 470 | 71106 | 71106 – 73385 | N/A |  |  |
| 110 | 1820 | 73386 | 73386 – 73485 | 1800 | 134342 | 134342 – 134441 |

# Conclusion

In this contribution, we discussed the system parameters for the new LTE band 1800-1830 MHz. We made the following proposals:

Proposal1: Specify the new LTE band 1800-1830 MHz with band number 110.

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| 110 | 1800 MHz – 1810 MHz | 1820 MHz – 1830 MHz | FDD |

With the following channel BW and SCS:

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 110 | Yes | Yes | Yes | Yes |  |  |

Proposal2: Specify the following E-UTRA channel number for band 110 in table 5.7.3-1:

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 106 (NOTE 6) | 935 | 70656 | 70656 - 70705 | 896 | 134292 | 134292 - 134341 |
| 107 (NOTE 7) | 612 | 70706 | 70706 – 71105 | N/A |  |  |
| 108 (NOTE 7) | 470 | 71106 | 71106 – 73385 | N/A |  |  |
| 110 | 1820 | 73386 | 73386 – 73485 | 1800 | 134342 | 134342 – 134441 |

# References

- RP-241619, New WID: Introduction of LTE FDD band in 1800 – 1830 MHz for Canada, Novamint, Sequans, Semtech, Telit, Ubiik