3GPP TSG-RAN WG4 Meeting #112                                           R4-2411227

Maastricht, Netherland, Aug 19th – 23th, 2024

Source: 	Huawei, HiSilicon

Title: 	Further consideration on general aspects for Rel-19 LP-WUS

Agenda Item:	8.22.1

Document for:	Approval

Introduction

General aspects for LP-WUS/WUR have been discussed in last RAN4 meeting, and some of them have been concluded in the WF [1].

This contribution provides further consideration on some aspects which should be further discussed.

Discussion

Set of requirements

On one set or multi-sets of requirements has not been determined yet.

Way forward in RAN4#110bis:

  - Discussions could be separated for OOK-based receiver and OFDM-based receiver. And latter stage to discuss whether a generic requirement could be defined, or not.

    - Further discuss the possibility of a dedicated set of requirements for OOK LP-WUR

    - FFS whether there is any other factors should be considered for LP-WUR requirements discussion

  - SNR and NF may be different for these two types.

It is known that three candidate UE architectures for LP-WUR have been discussed in Rel-18, of which the NF are expected to be different. To accommodate implementation flexibility, e.g. OOK-based and OFDM-based receivers, we think at least different NF should be considered.

Regarding SNR, the methodology adopted by RAN1 is targeted to the achievable coverage of PUSCH of MSG3.

Conclusion in RAN1#116bis: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement in RAN1#117: For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

If the target is to have the same coverage of MSG3, it may result in worse SNR in conjunction with better NF, which should not be the target pursued by RAN4. As SNR in determining REFSENS is the required SNR from performance perspective, if LP-WUR can achieve better SNR together with better NF, that should be what RAN4 strive to. In that sense, same or different SNR both could be considered, but no limitation that two sets of requirements can only achieve the same coverage of MSG3, in other words, similar coverage of MSG3 is the minimum demand for specifying the requirement, which is not the upper bound for determining combination of SNR and NF.

According to the latest progress on SNR evaluation by RAN1, it can be seen that the SNR values could be differ in several dBs for OFDM based and envelop detection based receivers. Given the discrepancy of NF for different LR architectures, a single set of requirements could be too pessimistic for the UE performance. From simplicity perspective, even we could use unique SNR to accommodate different implementations, we feel two different NF values as well as separate Rx requirements on top of the NF in conjunction of REFSENS for LP-WUR would be the proper way to define the corresponding requirements. Therefore, we continue to consider the option of two sets of Rx requirements for LP-WUR.

Proposal 1: To accommodate different UE architectures, two sets of requirements at least with different NF should be considered for LP-WUR.

Performance metric

For performance metric, the following agreement has been reached in last meeting [1]

Issue 1-1-11: Metric for LP_WUR requirements

Agreements:

- MDR is used as the metric for LP_WUR Rx RF requirements

  - FFS MDR values.

- FAR will be defined as demodulation requirements

  - FFS FAR values.

Issue 2-1-2: Performance metric (MDR/BLER value) for LLS simulation (apply to ACS/ASCS and REFSENS)

Agreement:

- For LLS simulation (apply to ACS/ASCS and REFSENS), the metric includes

  - 1% MDR/BLER as baseline and 5% MDR/BLER as optional

  - The following false alarm rate can be considered

    - 1%

    - 5%

    - Providing the information whether the false alarm rate is considered or not

- Further down-select the performance metric for the requirements and testing

In principle, miss-detection rate is considered as performance metric for LP-WUS requirements. Though it was agreed in last meeting that FAR is considered as demodulation requirement, however, we are concerned that whether MDR itself would be enough to avoid that during the test MR rather than LR is involved in the measurement which fails the verification of RF ability of LR. Given there is no clear conclusion on the testability issues for LP-WUR, we think it’s too early to draw a conclusion that FAR is excluded from the RF conformance test.

Proposal 2: Whether FAR is only considered for demod test should be revisited once there is a clear vision for testability issues for LP-WUR.

It is noted that both miss-detection rate and false-alarm rate have been considered by RAN1 for the LLS simulation assumption [2].

| MDR/FAR assumption | The miss-detection rate (MDR) of LP-WUS 1%, The false-alarm rate (FAR) of LP-WUS - 0.1%, 1% - Other values are not precluded for studying, reported by companies Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly The above values are applied in both RRC CONNECTED and IDLE/INACTIVE mode. FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead. The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group. For evaluation purpose, FAR target is determined across a reference time duration T of one or multiple LP-WUS attempts/trials, - UE has N attempts within T, - Company to report (FAR target, T, N) - For example, - if UE makes a single decision based on multiple correlations for a sequence in the monitor occasion, these correlations are considered as UE implementation in ONE trial/attempt. - if UE performs decoding in a monitor occasion, a single decoding is considered as ONE trial/attempt. - If UE performs N non-overlap attempts within the reference time duration, the false alarm event for the attempts are assumed as independent. Companies to provide the assumed side conditions to attain the used FAR over T or per one attempt e.g. CRC/sequence length in LP-WUS design. |
| --- | --- |

Two values of false-alarm rate have been evaluated by RAN1 during the SI study, i.e. 0.1% and 1%. Extremely lower false-alarm may require unnecessarily stringent SNR requirement for LP-WUR. We think that 1% for both miss-detection rate and false-alarm rate would be an appropriate choice.

Proposal 3: Both 1% miss-detection rate and false-alarm rate are considered for LP-WUR test.

Conclusion

This contribution provides further consideration on general aspects for LP-WUS/WUR. With above analysis, we have the following observations and proposals:

Proposal 1: To accommodate different UE architectures, two sets of requirements at least with different NF should be considered for LP-WUR.

Proposal 2: Whether FAR is only considered for demod test should be revisited once there is a clear vision for testability issues for LP-WUR.

Proposal 3: Both 1% miss-detection rate and false-alarm rate are considered for LP-WUR test.

- References

[1] R4-2410569, WF on NR LP-WUS UE requirements, vivo

[2] TR 38.869, Study on low-power wake-up signal and receiver for NR