3GPP TSG-RAN WG4 Meeting #112                                           R4-2411228

Maastricht, Netherland, Aug 19th – 23th, 2024

Source: 	Huawei, HiSilicon

Title: 	On UE REFSENS, ACS requirements for Rel-19 LP-WUS

Agenda Item:	8.22.2.2

Document for:	Approval

Introduction

Based on the last meeting discussion for Rel-19 LP-WUS, the issues to be further studied for REFSNES, ACS and ASCS are captured in WF [1]. The following are important agreements or directions for further discussion:

Issue 2-2-3: How to decide SNR value (not requirement) for REFSENS

Way forward:

  - After concluding WUS design in RAN1, the SNR to specify REFSENS requirements should be decided in RAN4

Issue 2-2-3: SNR value (not requirement) for REFSENS

Agreement:

- Stick to last meeting agreements: Wait for RAN1 SNR progress

Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB)

Agreement:

- RAN4 further discuss potential NF for OOK-based and OFDM-based receiver.

- Linked to resolution of Issue 2-2-8

Issue 2-2-6: IM value for REFSENS

Agreement:

- RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver.

Issue 2-4-2: ACS requirements value

Agreement:

- RAN4 consider the how to specify ACS requirements. FFS same as MR.

  - Decision will be made based on LLS outcome

Issue 2-4-3: Required guard RB for ACS case

Agreement:

- RAN4 further discuss required number of guard RB for ACS based on LLS simulation outcome next meeting.

Issue 2-3-2: ASCS requirements value

Agreement:

- RAN4 further discuss whether ASCS requirements value should be specified, or just the test parameter are specified, i.e., LP_WUS and NR signal bandwidths and power levels, and guard RBs.

Issue 2-3-3: Required number of guard RB for ASCS

Agreement:

- RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting.

This contribution provides our further analysis on REFSENS, ACS and ASCS.

Discussion

REFSENS

As discussed in previous meetings, legacy approach would be utilized to derive the REFSENS requirement, of which the most important parameters are SNR and NF.

Noise Figure

The targeted coverage for LP-WUS is achieve the comparable value of PUSCH for Msg3. To fulfill the target, some NF are considered by RAN1 in the evaluation, which could be conceived as a reference. It is noticed that 7dB rather than 9dB was adopted by RAN1 as the basis for MR.

Conclusion in RAN1#116bis: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

In RAN4 spec, the NF adopted for REFSENS varies across different bands, it could be ranged from 9dB to 13dB or even worse. Given the three major UE architectures discussed by RAN4 in Rel-18, we think that +2dB option could be ruled out. For the rest of +5dB and +8dB, we think that they can be considered tentatively for OFDM-based receiver and OOK-based receiver respectively.

Proposal 1: It is proposed to adopt +5dB and +8dB on top of 9dB basis as NF for OFDM-based receiver and OOK-based receiver respectively as starting point for REFSENS.

SNR

The SNR evaluation in RAN1 is based on TDL model, while the usual model adopted by RAN4 in SNR simulation for purpose of RF requirements is AWGN based. It is foreseen that the OFDM based receiver has better SNR compared to OOK envelope detection receiver, and OOK-4 with chip rate M=4 is worse than other cases, we focus on the simulation for OOK4-M4. Figure below shows the required SNR value for envelope detection. It can be seen that for both 3rd order or 5th order Butterworth filter and 4-bit or 8-bit ADC, -2dB SNR could be achieved for 1% BLER or MDR.

Figure 1: SNR evaluation for LP-WUS (envelop detection based receiver)

It is expected that SNR for OFDM based receiver would be better than that of envelop detection receiver, but for sake of simplicity, we may just define single SNR for both receiver types as we already have different NF assumption for these receiver types.

Proposal 2: It is proposed to define single SNR value with -2dB for both OFDM based and envelop detection based LP-WUR receivers.

ACS

Guard RBs for ACS

According to the study outcome in Rel-18, RAN4 had the following conclusion for guard RBs for ACS:

Conclusions for RAN4 ACS guard RB analysis:

Based on the analysis above, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ACS is in the range of 1RB ~ 6RBs for 30kHz SCS, or 2RBs ~12RBs for 15kHz SCS. For lower power, i.e., lower order filters, the required guard RBs may increase. There is no recommendation on which filter order should be considered to the baseline in SI phase.

During the SI study, we have evaluated the guard RBs for ACS, and our observation based on the simulation results with consideration of ADC bits as well as frequency error is that guard RB with the size of one 30kHz SCS RB could provide necessary protection of LP-WUS from interference of adjacent NR carrier. We think 11 RBs in terms of 30kHz SCS is feasible for 5MHz channel bandwidth. In addition, as discussed in the following section that the adjacent interferer should be relaxed with lower power imbalance with LP-WUS signal, we see no additional GB is needed for ACS case.

Proposal 3: It is proposed to adopt 11 PRB for LP-WUS with 30kHz SCS for 5MHz channel bandwidth with no additional guard RBs. In case of 15kHz SCS, the guard band should be similar to that of 30kHz. Specific PRB number can be further discussed based on RAN1 progress.

ACS requirement

Generally, adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s). The said receiver filter usually is the baseband channel filter. While for LP-WUR, the filter attenuation mainly comes from the analog filter, e.g. 3rd and 5th order Butterworth filters implemented in the Rx chain before ADC, but it does not preclude the possible implementation of digital domain channel filter.

The requirement of ACS comes from the co-existence study, it is derived from ACIR with also consideration of ACLR from the BS side. The assumed BS ACLR is 45dBc for the co-existence study, however, the normal BS implementation for ACLR is better than the assumed value, in addition, the performance metric for LP-WUR ACS is not throughput as considered for the MR, it’s possible to relax the ACS requirement in exchange of smaller guard RB size for LP-WUS.

In current specification ACS is defined as 33dBc for NR at least for 5MHz CBW, by which together with REFSENS degradation, it results the Pinterferer, i.e. REFSENS + 45.5dB. The power imbalance of 31.5dBc is the level considering the ACS value as well as the implementation margin. Apparently, 5th order Butterworth filter cannot provide very high rejection to the adjacent interferer. The evaluated 5th order Butterworth filter can only provide less than 20dB rejection for the adjacent channel with 5MHz frequency offset. Furthermore, if component tolerance is considered for the filter implementation, the suppression capability of this kind of filter could be further degraded. In addition, as pointed out in [2] that with process and temperature variations, the metal–oxide–metal capacitor value in 28-nm CMOS can vary by ± 20%, and poly resistors vary by ±15%. All the factors make the filter rejection much less than the theoretical analysis. The real implementation may just provide ~10dB suppression for the adjacent interferer, which is much lower than the MR.

Figure 2: Frequency response of Butterworth filter

Supposing 10dB filtering rejection is provided, and still 14dB REFSENS degradation is allowed, with rough estimation, the interferer level LR could handle is about LR REFSENS + 22.5dB or less with IM considered. Alternatively, if keep the same interferer level as MR, the power imbalance between the interferer and LR wanted signal should consider the REFSENS degradation due to the larger NF for LR. If 8 dB NF increase is considered, which also means the power imbalance is decreased by 8dB, i.e. 23.5dB. It could be interpreted as either larger filter rejection is needed with less guard RBs, or with the certain filter rejection capability considering real implementation, e.g. 10dB, more guard RBs would be expected.

Figure 3: Illustration of ACS requirement

It is known that two cases have been specified for ACS in the current specification. Absolute values rather than relative value in case 1 are defined for case 2. The interferer is fixed -25dBm, while the wanted signal is -56.5dBm for MR. For case 2, the dynamic range is 31.5dBc. Whether the LR can meet the dynamic range, it depends on two major factors, i.e. the ADC bits as well as the dynamic range of AGC. The ADC dynamic range can be roughly estimated as 6dB times bit number, thus a 4-bit ADC can only provide 24dB dynamic range and an 8-bit ADC can provide 48dB dynamic range. If the requirements are defined with consideration of 4-bit ADC implementation, definitely we need to decrease the interferer level. On the other hand, owing to the low power consumption design, it is expected that the dynamic range for AGC is not very large. For instance, assume dynamic range of AGC is 30dB (the higher the dynamic range, the larger the power consumption), if the REFSENS of LR is 8dB higher than MR, it can be assumed as -89dBm/5MHz. For simplicity, we consider the AGC dynamic range also includes the Rx gain, and to avoid to saturate the chain, the LNA can be bypassed. The amplified LR signal should be set to appropriate level in the dynamic range of ADC, and REFSENS should be above the noise floor of the ADC. With above assumption, we can have some simple calculation. Normally, around 10dB backoff is considered from 0dBFS for ADC due to the PAPR for the modulated signal, no matter it is wanted signal or interfering signal, which in turn further reduce the dynamic range of an ADC. An 8-bit ADC can merely comply with the dynamic range if -25dBm is considered for ACS case 2, however, if the AGC dynamic range is lowered, 8-bit ADC could also fail the required dynamic range. It’s a tradeoff between the power consumption and ability of LP-WUR to handle large interferer.

ACS value reflects the adjacent channel rejection of the filter, and it is derived from the co-existence study, however, LR has somehow a different co-existence scenario compared to normal UE, which should not be constraint by the value of 33dBc, in other words, no need to keep the same interferer level in both case 1 and case 2 if we do need to consider two cases for LP-WUR ACS requirement. If the dynamic range of LR is supposed not as good as MR, the interferer level could be relaxed.

Observation 1: With worse REFSENS for LR, if keep the same REFSENS degradation level, i.e. 14dB, the ACS value range would be decreased even with the same interferer level as MR.

Observation 2: The filter evaluated by RAN4 in SI stage cannot provide sufficient suppression compared to the level defined in current spec for MR.

With above analysis, we propose to consider the requirement as well as parameters for ACS case 1 and case 2 as follows:

Table 1: Adjacent channel selectivity

| Rx Parameter | Units | 5 MHz |
| --- | --- | --- |
| ACS | dB | 10.0 |

Table 2: Test parameters for Adjacent channel selectivity, Case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + [22] dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. |  |  |

Table 3: Test parameters for Adjacent channel selectivity, case 2

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Power in transmission bandwidth configuration | dBm | -56.5 |
| Pinterferer | dBm | [-31.5] |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. |  |  |

Proposal 4: It is proposed to relax ACS requirement for LP-WUR from co-existence and performance perspective. The proposed ACS is 10dB given the poor filtering capability of LP-WUR.

Proposal 5: It is proposed to relax dynamic range for ACS requirement in case 2 while keep the same wanted signal level as MR. The proposed dynamic range for ACS case to is relaxed to 25dB.

ASCS

Guard RBs for ASCS

According to the study outcome in Rel-18, RAN4 had the following conclusion for guard RBs for ASCS:

Conclusion for RAN4 ASCS guard RB analysis:

Based on the following analysis, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0RB ~ 1RBs for 30KHz SCS, or 0RBs ~2RBs for 15KHz SCS. Similar number of guard RBs could be applied also with lower filter orders as performance was observed to remain similar with 3rd order filter.

From the SI study, it is seen that performance loss is not severe in presence of the adjacent NR signal with the same PSD as LP-WUS. Given the possible degradation of the filter suppression capability for real implementation, guard RB is preferred for ASCS. One RB with 15kHz SCS could be considered. In case of 30kHz SCS, one RB can still be considered.

Proposal 6: It is proposed to adopt 1 RB as the size of guard RB for LP-WUS ASCS regardless of the applied SCS.

ASCS requirement

Compared to ACS requirement, ASCS just reflects the performance demand for LP-WUS in presence with adjacent interferer. In SI evaluation of ASCS GB, the interferer is the NR signal in the channel BW for in-band operation with same PSD. From all simulations presented in SI, 1 RB for 30kHz SCS would be sufficient to guarantee the demodulation for LP-WUS in-band operation. Unlike ACS, the requirement with strong interferer also could verify the Rx chain linearity as well as dynamic range, while ASCS may just reflects the filter ability, which could be verified by ACS as well. In that sense, if guard RB is specified for ASCS, specifying ASCS requirement may not be necessary.

Proposal 7: If guard RB is specified for ASCS scenario, there is no need to define specific requirement for ASCS.

Conclusion

This contribution provides our further analysis of REFSENS, ACS and ASCS requirements for LP-WUR. With the analysis, we have the following observations and proposals:

Proposal 1: It is proposed to adopt +5dB and +8dB on top of 9dB basis as NF for OFDM-based receiver and OOK-based receiver respectively as starting point for REFSENS.

Proposal 2: It is proposed to define single SNR value with -2dB for both OFDM based and envelop detection based LP-WUR receivers.

Proposal 3: It is proposed to adopt 11 PRB for LP-WUS with 30kHz SCS for 5MHz channel bandwidth with no additional guard RBs. In case of 15kHz SCS, the guard band should be similar to that of 30kHz. Specific PRB number can be further discussed based on RAN1 progress.

Observation 1: With worse REFSENS for LR, if keep the same REFSENS degradation level, i.e. 14dB, the ACS value range would be decreased even with the same interferer level as MR.

Observation 2: The filter evaluated by RAN4 in SI stage cannot provide sufficient suppression compared to the level defined in current spec for MR.

Proposal 4: It is proposed to relax ACS requirement for LP-WUR from co-existence and performance perspective. The proposed ACS is 10dB given the poor filtering capability of LP-WUR.

Proposal 5: It is proposed to relax dynamic range for ACS requirement in case 2 while keep the same wanted signal level as MR. The proposed dynamic range for ACS case to is relaxed to 25dB.

Proposal 6: It is proposed to adopt 1 RB as the size of guard RB for LP-WUS ASCS regardless of the applied SCS.

Proposal 7: If guard RB is specified for ASCS scenario, there is no need to define specific requirement for ASCS.

- References

[1] R4-2410569, WF on NR LP-WUS UE requirements, vivo

[2] A Digital Filtering ADC With Programmable Blocker Cancellation for Wireless Receivers, Qiwei Wang, etc., IEEE JOURNAL OF SOLID-STATE CIRCUITS, VOL. 53, NO. 3, MARCH 2018