3GPP TSG-RAN WG4 Meeting #112                                           R4-2411229

Maastricht, Netherland, Aug 19th – 23th, 2024

Source: 	Huawei, HiSilicon

Title: 	On UE RF other Rx requirements for Rel-19 LP-WUS

Agenda Item:	8.22.2.3

Document for:	Approval

Introduction

Based on last meeting discussion for Rel-19 LP-WUS, the issues to be further studied on Rx requirements other than REFSNES, ACS and ASCS are captured in WF [1].

This contribution provides further analysis on other Rx requirements including IBB/OBB, intermodulation, spurious emissions and spurious response.

Discussion

IBB and OBB

Regarding IBB and OBB, the issues to be further studied in the WF including:

Issue 3-1-1: IBB and OBB assumption

Agreement:

  - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR.

    - With assumption that Rx dynamic range for LR may not be as good as MR

Issue 3-1-2: IBB and OBB requirements

Agreement:

  - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS IBB and OBB requirements

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. Noted that spurious response is a measure to avoid failure for blocking test due to instinct character of some UE architectures design, e.g. IF architecture. In that sense, it is reasonable to keep the spurious response requirement for LP-WUR as well.

In the specification, the absolute interferer levels are adopted for both in-band blocking (IBB) and out-of-band blocking (OBB). IBB refers to interference that occurs within the second adjacent and next adjacent channels, while OBB occurs outside a frequency range 15 MHz below or above the UE receive band. The interference of IBB and OBB are higher than ACS. The interferers relevant to the wanted signal are illustrated in the figure below:

Figure 1: Interference of ACS, IBB and OBB

The main challenges for receiver design in presence of strong interferer are the Rx chain dynamic range as well as the filter suppression capability, which could be the channel filters to alleviate interfering by ACS or IBB, or could be the band filter/duplexer to suppress OBB.

Regarding the dynamic range, the main determining components are ADC and AGC. The more ADC bits, the larger dynamic range for the receiver. However, the ADC bits cannot be increased without limitation. The relationship between ADC bit depth and power consumption is typically inversely proportional. Higher bit-depth ADCs generally offer greater precision and resolution but tend to consume more power due to the increased complexity of circuitry required for finer quantization and digitalization. Conversely, lower bit-depth ADCs consume less power but sacrifice resolution and dynamic range.

In the context of 5G smartphones, ADCs with 12 bits or higher are commonly employed. These higher precision ADCs are favored to accommodate the demanding requirements of 5G communication, such as higher data rates and broader spectrum ranges. While some 5G smartphones may opt for even higher bit-depth ADCs like 14 or 16 bits for enhanced performance, the general trend revolves around 12 bits.

However, there's a trade-off to consider. LP-WUS which aims to minimize power consumption in smartphones, often utilize lower bit-depth ADCs to reduce power consumption. While this approach yields power-saving benefits, it may compromise the dynamic range and overall performance of the ADC. During the SI evaluation, 4 bits, 6 bits and 8 bits are considered for LLS simulation. Apparently, if low power consumption is the main target for LP-WUR design, it may not be possible that a LP-WUR could adopt the same ADC as MR, which means the dynamic range for LP-WUR could be sacrificed to some extent.

Apart from ADC, the dynamic range of AGC is another main factor which could have impact on the possibly large interferer handled by LR. As we discussed in [2], the AGC dynamic range could be much lower compared to that of MR for sake of power consumption design.

On the other hand, due to the limited suppression capability of the LP-WUR filters, it may not guarantee the same level of resistance against IBB and OBB interference as the main receiver (MR).

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption.

From requirements perspective, if the dynamic range and filter suppression capability for LR is not as good as MR, which end should be compromised for specifying the requirement? We think that better REFSENS to guarantee the coverage should have higher priority, as the NF is already expected much worse than MR. In this sense, probably the interference level should be relaxed to make it feasible for LR implementation. It is noted that the in-band blocking is from the non-coordinated NW and out-of-band blocking may come from other systems, if MR experiences such strong interference, LR will also be affected by these strong interferences. In such scenario, whether LR can work well in presence of strong interference may need further consideration.

Proposal 1: In order to guarantee the coverage of LP-WUR in blocking scenarios, the interference levels for IBB could be relaxed compared to the values defined for MR. FSS whether OBB could be relaxed.

Table 1: In-band blocking parameters

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |

Table 2: In-band blocking

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-61] | [-49] |
| Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |

In case the LR encounters larger in-band blocking interferer which could not handle, MR should take over the monitoring of the PO or PDCCH no matter in in IDLE/INACTIVE mode or CONNECTED mode. It is worth noting that blocking is just a statistical event, the LR may not always experience the blocking scenarios, which is just a best effort target for LP-WUR design comparted to the low power consumption purpose.

Regarding OBB, with suppression of the out-of-band interferer by band filter and the analog filter before ADC, the blocking could be suppressed to relatively low level even with less dynamic range of LR, unless band filter is not considered for better coverage with less insertion loss for LR design. In that case, whether OBB should be relaxed can be further considered.

Intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

This requirement is designed to verify the linearity of the Rx chain. Similar to the analysis of blocking requirement, the implementation of LR, specifically the linearity of the receiver may not as good as that for MR, thus, if the interferer level of blocking is relaxed, similar relaxation should be considered for intermodulation as well.

Proposal 2: If the blocking interferer is relaxed to accommodate the low power design, intermodulation requirement could also be relaxed for LP-WUR.

Table 3: Wide band intermodulation parameters

| Rx parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5 |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | [-51] |
| PInterferer 2 (Modulated) | dBm | [-51] |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset) | MHz | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |

Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. In current specification, the power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table below:

Table 4: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |

For receiver, the emissions at the antenna connector usually come from the reverse LO leakage. Since the emissions could be considered as kind of regulatory requirements, the same levels for MR should also be defined for LR.

Proposal 3: The same level of receiver spurious emissions for MR shall be defined for LR as well.

Spurious response

The source of generating spurious response includes mirror frequency interference, intermediate frequency interference, high-order mixing products of mixers, and other Nyquist zone aliasing of ADCs that happen to fall within the receiving band. These interferences cannot be processed by the receiver, thus causing a decrease in sensitivity. The only way to address this is by selecting appropriate intermediate frequency points, ADC sampling frequencies, and mixers with high-order suppression to reduce the number of spurious response points.

Spurious response reflects situations where the UE receiver architecture generates response points that cannot be adequately processed by the receiver. Blocking requirements cannot pass the test on these scattered points, therefore, spurious response can serve as a remedial measure for blocking tests.

When encountering spurious response, the level of the interfering signal is uniformly relaxed to -44dBm, while the wanted signal remains the same as that for out-of-band blocking.

Proposal 4: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR.

Similar to that of IBB, if the interferer level is relaxed several dBs, the spurious response should be relaxed as well.

Table 5: Spurious response parameters for LP-WUR

| RX parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 5 MHz |
| Power in transmission bandwidth configuration | dBm | PREFSENS + 6dB |

Table 6: Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | [-49] |
| FInterferer | MHz | Spurious response frequencies |

Conclusion

This contribution provides further consideration on IBB/OBB, intermodulation, spurious emissions and spurious response requirements for LP-WUR.

Observation 1: LP-WUR may have less dynamic range and capability to resist against strong blocking interference as MR could do owing to the tradeoff for the implementation of low power consumption.

Proposal 1: In order to guarantee the coverage of LP-WUR in blocking scenarios, the interference levels for IBB could be relaxed compared to the values defined for MR. FSS whether OBB could be relaxed.

Proposal 2: If the blocking interferer is relaxed to accommodate the low power design, intermodulation requirement could also be relaxed for LP-WUR.

Proposal 3: The same level of receiver spurious emissions for MR shall be defined for LR as well.

Proposal 4: spurious response as a remedial measure for blocking tests needs to be considered for LP-WUR.

- References

[1] R4-2410569, WF on NR LP-WUS UE requirements, vivo

[2] R4-2411228, Further consideration on UE RF REFSENS, ACS, ASCS requirements for Rel-19 LP-WUS, Huawei, HiSilicon