# 3GPP TSG-RAN WG4 Meeting # 112	R4-2411235

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	Delta Ppowerclass and Delta TRxSRS for 4Tx

Agenda Item:	5.9.1

Document for:	Approval

# Introduction

This contribution discusses issues raised in [1] for ΔPPowerClass and ΔTRxSRS for 4Tx based on an approved WF [2] with consideration of discussion papers [3, 4]. Simply put, in order to take things a step beyond, we propose to follow an approach that we have taken for 2Tx.

# Discussion

## Overview

In our view, many of the technical observations in [3, 4] are well aligned, though the proposed solutions are different.

Observation 1: The solution in [3] is UE implementation oriented and allows the UE to achieve the best performance possible, while it theoretically gives the UE more chances to perform Antenna Virtualization than the solution in [4].

Observation 2: The solution in [4] is more oriented to prevent the UE from performing AV than the solution in [3] at the cost of achieving the best possible performance, while it still cannot perfectly prevent the UE from performing AV.

Apart from above those observations, one of the biggest problems is that the current specification doesn’t allow PC1.5 UE with 4Tx to perform SRS antenna switching (AS) for some patterns like t1ry and t2ry, since the UE cannot achieve the same power as that of the declared PC with 1Tx and 2Tx during SRS AS, since there are no ΔPPowerClass values depending on the SRS AS patterns. Thus, at least that aspect shall be fixed.

Observation 3: At least 38.101-1 shall need specific values for ΔPPowerClass depending on configured SRS AS (AS) patterns. Otherwise, UE with 4Tx is not allowed to perform SRS AS other than t4ry.

As mentioned in [3, 4], there is no perfect solution. Thus, in order to move forward, we propose to follow an approach of 2Tx to develop the requirements for SRS AS for UE supporting 4Tx.

Proposal 1: For 4Tx SRS AS related requirements, apply an approach of PC2 2Tx. Then, refine the requirements later if needed.

## PC1.5 with 4Tx with txDiversity4Tx-r18

For 2Tx discussion, if txDiversity-r16 is indicated by a UE, RAN4’s assumed that the UE’s PA configuration is 23 dBm x 2. Thus, when a UE indicates txDiversity-r16, if its PC is 2 and the UE is configured with t1ry pattern(s), ΔPPowerClass is set to 3 dB in 38.101-1. ΔPPowerClass in accordance with SRS AS patterns become as summarized in Table 2-2.-1.

Table 2.2-1: Relationship between ΔPPowerClass and configured AS pattern for UE with txDiversity4Tx-r18

Proposal 2: Apply following ΔPPowerClass values in accordance with configured SRS AS pattern(s) if the UE indicates txDiversity4Tx-r18 as shown in Table 2.2-1.

## PC1.5 4Tx without txDiversity4Tx-r18

A point for this case is how to handle cases that a UE doesn’t use PA(s) with the highest power ability for SRS AS. For this point, we apply the approach for PC2 2Tx without txDiversity-r16 to that for PC1.5 4Tx without txDiversity4Tx-r18. To make the discussion more specific, we analyse following cases.

Figure 2.3-1: PC2 2Tx for t1r4: Relationship between ΔPPowerClass, ΔTRxSRS and support of txDiversity-r16

Observation 4: For PC2 2Tx with ΔPPowerClass and without txDiversity-r16, even if a UE implements 26 dBm x 2 PA configuration, additional 3 dB relaxation is allowed on top of a case that ΔPPowerClass = 3 dB or PC2 with txDiversity-r16.

Hence, when it comes to analysing 4Tx, we consider 23 dBm x 4 PA configuration and 26 dBm x 2 + 23 dBm x 2 PA configuration and don’t consider 26 dBm x 4 PA configuration, since as far as the former two types of PA configurations are covered, 26 dBm x 4 PA configuration can be automatically covered.

Observation 5: For PC1.5 4Tx, in accordance with the principle of Observation 4, 23 dBm x 4 and 26 dBm x 2 + 23 dBm x 2 PA configurations are considered to derive relaxation value, since if both configurations are considered into 38.101-1, 26 dBm x 4 PA configuration is automatically covered.

Figure 2.3-2: PC1.5 4Tx for t1r4: Relationship between ΔPPowerClass, ΔTRxSRS and support of txDiversity4Tx-r18

Figure 2.3-2 shows cases that a PC1.5 UE supporting 4Tx is configured with t1r4 SRS AS pattern. For Case 4, additional 3 dB relaxation is needed from the 2nd SRS port transmission by following the principle of Case 2 in Figure 2.3-1. From implementation perspective, that applicability is reasonable, since UE does not always switch PA in the order of PA with higher power.

Observation 6: For PC1.5 UE supporting 4Tx configured with t1ry, if ΔPPowerClass = 3 dB and the UE doesn’t not indicate txDiversity4Tx-r18, ΔTRxSRS shall be increased by 3 dB from the 2nd SRS port transmission compared to ΔTRxSRS for PC1.5 UE indicating txDiversity4Tx-r18 or its ΔPPowerClass = 6 dB.

Figure 2.3-3: PC2 2Tx for t2r4: Relationship between ΔPPowerClass, ΔTRxSRS and support of txDiversity-r16

Figure 2.3-3 shows cases that a PC2 UE supporting 2Tx is configured with t2r4 SRS AS pattern. No additional 3 dB relaxation to ΔTRxSRS for Case 6 on top of ΔTRxSRS for Case 5 has been introduced, since any PA pairs can achieve PC2.

Observation 7: For PC2 UE supporting 2Tx configured with t2ry, ΔTRxSRS for a UE implementing 26 dBm + 23 dBm or 26 dBm x 2 PA configuration, i.e., ΔPPowerClass = 0 dB and no txDiversity-r16 indication is the same as that for a UE implementing 23 dBm x 2 PA configuration, i.e., indicating txDiversity-16 or ΔPPowerClass = 0 dB. This is because any PA pairs can achieve PC2 during performing the AS pattern.

Figure 2.3-4: PC1.5 4Tx for t2r4: Relationship between ΔPPowerClass, ΔTRxSRS and support of txDiversity4Tx-r18

Figure 2.3-4 shows cases that a PC1.5 UE supporting 4Tx is configured with t2r4 SRS AS pattern. Regarding Case 8, since the UE can deliver 29 dBm at least for the 1st SRS port pair transmission so that ΔPPowerClass = 0 dB. If the UE doesn’t use PA pair with 26 dBm x 2 for the 2nd SRS port pair transmission, i.e., two 26 dBm PAs are fixed to the respective physical antenna connectors, the highest power for the 2nd SRS port pair transmission is 26 dBm (23 dBm x 2) so that ΔPPowerClass becomes 3 dB. Then, if we apply the principle for the Case 2, ΔTRxSRS for the 2nd port pair transmission should be increased by 3 dB compared to ΔTRxSRS for Case 7.

Observation 8: When a PC1.5 UE supporting 4Tx is configured with t2r4 SRS AS pattern, the UE’s ΔPPowerClass = 0 dB and the UE doesn’t indicate txDiversity4Tx-r18, ΔTRxSRS for SRS port pair transmission from the 2nd up to 4th port pair should be increased by 3 dB compared to ΔTRxSRS for a case that the UE’s ΔPPowerClass = 3 dB and the UE indicates txDiversity4Tx-r18

Proposal 3: Specify the requirements related to SRS AS for PC1.5 4Tx without txDiversity4Tx-r18 indication based on the Observation 4 – 8.

# Summary

We obtained following observations and proposals. A companion CR based on this contribution is also submitted [5].

Observation 1: The solution in [R4-2407682] is UE implementation oriented and allows the UE to achieve the best performance possible, while it theoretically gives the UE more chances to perform Antenna Virtualization than the solution in [R4-2409772].

Observation 2: The solution in [R4-2409772] is more oriented to prevent the UE from performing AV than the solution in [R4-2407682] at the cost of achieving the best possible performance, while it still cannot perfectly prevent the UE from performing AV.

Observation 3: At least 38.101-1 shall need specific values for ΔPPowerClass depending on configured SRS AS (AS) patterns. Otherwise, UE with 4Tx is not allowed to perform SRS AS other than t4ry.

Proposal 1: For 4Tx SRS AS related requirements, apply an approach of PC2 2Tx. Then, refine the requirements later if needed.

Proposal 2: Apply following ΔPPowerClass values in accordance with configured SRS AS pattern(s) if the UE indicates txDiversity4Tx-r18.

Observation 4: For PC2 2Tx with ΔPPowerClass and without txDiversity-r16, even if a UE implements 26 dBm x 2 PA configuration, additional 3 dB relaxation is allowed on top of a case that ΔPPowerClass = 3 dB or PC2 with txDiversity-r16.

Observation 5: For PC1.5 4Tx, in accordance with the principle of Observation 4, 23 dBm x 4 and 26 dBm x 2 + 23 dBm x 2 PA configurations are considered to derive relaxation value, since if both configurations are considered into 38.101-1, 26 dBm x 4 PA configuration is automatically covered.

Observation 6: For PC1.5 UE supporting 4Tx configured with t1ry, if ΔPPowerClass = 3 dB and the UE doesn’t not indicate txDiversity4Tx-r18, ΔTRxSRS shall be increased by 3 dB from the 2nd SRS port transmission compared to ΔTRxSRS for PC1.5 UE indicating txDiversity4Tx-r18 or its ΔPPowerClass = 6 dB.

Observation 7: For PC2 UE supporting 2Tx configured with t2ry, ΔTRxSRS for a UE implementing 26 dBm + 23 dBm or 26 dBm x 2 PA configuration, i.e., ΔPPowerClass = 0 dB and no txDiversity-r16 indication is the same as that for a UE implementing 23 dBm x 2 PA configuration, i.e., indicating txDiversity-16 or ΔPPowerClass = 0 dB. This is because any PA pairs can achieve PC2 during performing the AS pattern.

Observation 8: When a PC1.5 UE supporting 4Tx is configured with t2r4 SRS AS pattern, the UE’s ΔPPowerClass = 0 dB and the UE doesn’t indicate txDiversity4Tx-r18, ΔTRxSRS for SRS port pair transmission from the 2nd up to 4th port pair should be increased by 3 dB compared to ΔTRxSRS for a case that the UE’s ΔPPowerClass = 3 dB and the UE indicates txDiversity4Tx-r18

Proposal 3: Specify the requirements related to SRS AS for PC1.5 4Tx without txDiversity4Tx-r18 indication based on the Observation 4 - 8.

# References

[1] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[2] R4-2406590, WF on 4Tx SRS issues, vivo, RAN4#110bis

[3] R4-2407682, Delta Ppowerclass and Delta TRxSRS for 4Tx for SRS antenna switching, Huawei, HiSilicon, RAN4

#111

[4] R4-2409772, On DeltaP_PowerClass for SRS antenna switching for 4Tx, Ericsson, RAN4#111

[5] R4-2411236, Introduction of Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#112