3GPP TSG-RAN WG4 Meeting # 112	R4-2411237

# Maastricht, Netherlands, August 19th – 23rd, 2024

Title: 	Technical wording ambiguities and Table modifications handling

Source: 	Huawei, HiSilicon

Agenda item:	10.1.1.1

Document for:	Approval

# Introduction

This contribution discusses simplification of ΔTIB,c and ΔRIB,c tables, wording ambiguities and handling of NOTEs in a table.

- Simplification of ΔTIB,c and ΔRIB,c tables

The approved WF of [1] listed following three options.

Among the options, our view is that at least Option 2 should not be adopted due to following reasons. First, the proposed values are smaller than those in the existing tables for ΔTIB,c and ΔRIB,c in most of the cases. RAN4 requirements, however, are minimum requirements to accommodate various UE implementation choices with consideration of various aspects such that cost, power consumption, supported features etc. In fact, the proponent of Option 2 also observed that “The real values are meanwhile heavily dependent on the RF Frontend architecture of the UE, the vendor of the frontend, the number of supported bands and band combinations etc., at least they can vary significantly from one UE to another for the same band combination” in [2].

Observation 1: RAN4 requirements are minimum requirements to accommodate various UE implementation options and the requirements should be reasonable for the industry including aspects such that power consumption, cost etc.

Though so called “shared pain” approach’s been used as pointed out in [2], it is also true that these values are from some technical analysis. Specifically, requirements for BCs, e.g., low-low, low-low-low, BCs including higher bands like n77, n79 and n102, or BCs with simultaneous RxTx etc., are based on careful technical analysis. Some can be seen in below Table 1-1.

Table 1-1: Some values from Table 6.2A.4.2.3-1 for NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n41 | 0.5 | 0.5 |
| CA_n1-n77 | 0.6 | 0.8 |
| CA_n2-n41 | 0.5 | 0.45/0.96 |
| CA_n28-n71 | 1.1 | 1.1 |
| CA_n41-n66 | 0.86 / 1.37 | 0.5 |
| CA_n41-n70 | 0.5 | 0.5 |
| CA_n71-n85 | 1 | 1 |
| CA_n74-n77 | 0.4 | 0.8 |
| CA_n77-n102 | 1.5 | 1.5 |
| CA_n78-n79 | 0.5 / 1.58 | 0.5 / 1.58 |
| CA_n78-n102 | 1.5 | 1.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Observation 2: Option 2 tightens the existing requirements which may preclude some implementation choices.

Since the proposed values themselves are not from technical analysis, there is no justification to change the current values as well as those don’t guarantee possible challenges that we may face in the future. For instance, if went with Option 2, band combination with seven bands would have 0.6 dB relaxation values. However, more bands in a certain frequency range(s) make frequency distance(s) between bands smaller and it leads to more challenging implementation.

Observation 3: Option 2 has no technical justification to tighten the existing requirements as well as does not guarantee future challenges that RAN4 may face, e.g., larger number of bands in a certain frequency range may require more relaxations.

Removing specifically table for band combinations with two bands loses some essential information like simultaneous Rx/Tx and accompanied values.

Observation 4: Adopting Option 2 means no tables for relaxation values and it leads to a situation that specifications lose some essential information like simultaneous Rx/Tx and accompanied values specifically for BCs with two bands.

Overall, only for the sake of the simplification of standards and procedure, tightening the requirements is not justified. Moreover, though Option 2 makes specifications simpler, it doesn’t necessarily mean that the quality is improved given that values in the specifications also are one of the key factors of the quality.

Observation 5: Having reasonable values in specifications is also one of the key factors for the quality of the specifications and the values in the current specifications are the ones that we have been able to find out as the most reasonable values.

Proposal 1: RAN4 shall not adopt Option 2 or similar approaches and not tighten the existing requirements for the sake of specification quality improvement.

As one possible alternative, we propose a following approach.

Proposal 2: Adopt the following approach.

      - Apply followings to band combinations (BCs) with two bands,

        - Maintain the values for ΔTIB,c and ΔRIB,c for BCs in the tables for BCs with two bands.

        - Put 0 dB being handled as zero (i.e., not listed in the tables for two bands) with respective BCs in the tables for BCs with two bands.

      - Apply followings to BCs with more than two bands, values for ΔTIB,c and ΔRIB,c for BCs with more than two bands are derived by taking maximum among all the values for the corresponding respective bands from all the fallback BCs only with two bands.

        - For example, regarding CA_n1-n3-n78, ΔTIB,c for CA_n1-n3, CA_n1-n78 and CA_n3-n78 are (0.3 dB, 0.3 dB), (0.3 dB, 0.8 dB) and (0.6 dB, 0.8 dB), respectively.

        - Take max for each band like max (0.3 dB, 0.3 dB) for n1, max (0.3 dB, 0.6 dB) for n3 and max (0.8 dB, 0.8 dB) for n78. Then, the values for the respective bands for CA_n1-n3-n78 become (0.3 dB, 0.6 dB, 0.8 dB) = (n1, n3, n78).

        - It’s noted that the values currently specified are (0.6 dB, 0.6 dB, 0.8 dB) = (n1, n3, n78). Thus, this does not always give the same values in the current specifications.

        - Thus, some exceptions are needed for specifically BCs whose degree of implementation challenges drastically changes compared to fallback BCs with two bands

      - If a BC with more than two bands has more than two bands at least in any one of the frequency ranges, below 1 GHz, 1.4 - 2.7 GHz and above 2.7GHz, RAN4 shall discuss values on case-by-case basis instead of applying the 2nd main bullet in the proposal 2 to it and list the values in the corresponding tables as exception

        - For a BC with more than three bands, values for ΔTIB,c and ΔRIB,c for bands group(s) consisting of more than two bands which falls into one of the frequency ranges, the values are derived based on the 3rd main bullet (i.e., case by case analysis is needed) in the proposal 2, while values for ΔTIB,c and ΔRIB,c for the remaining band(s) are derived based on the 2st main bullet in the proposal 2 .

        - E.g., A+B+C+D+E where A+B+C falls into one of the frequency ranges, then, values derived based on the 3rd main bullet in the proposal 2 applies, while the 2nd main bullet applies to D and E to derive values.

        - For another example, if a completely new band combination with four bands whose three of the four bands are below 1GHz, then, the proponent of this band combination shall provide technical analysis on values in their TP.

      - Those above shall be allowed to be revisited whenever necessary.

        - This is because there may be aspects that we have not been aware of at the writing of this contribution

Some specific examples are shown in Figure 2-1.

Figure 2-1: Some examples for applying the proposal 2 to some band combinations

- Wording ambiguities

## CA requirement applicability

In our view, CA requirement applicability requires technical discussion. In fact, there have been a discussion on if “scheduled” should be used or not to allow higher power transmission during dual uplink configuration. Hence, this exceeds the scope under this task so that this should be discussed under HPUE CA power class agenda item (10.2 in RAN4#112).

Observation 6: The discussion on CA requirement applicability requires technical discussion.

Proposal 3: CA requirement applicability should be handled in HPUE CA power class agenda item.

## “dual TX” and “2Tx”

In our understanding, they are mutually exchangeable. The next step is which either dual Tx or 2Tx we should take. In our view, we should take 2Tx due to following reasons.

- If we selected dual Tx, i.e., replaced 2Tx with dual Tx, could we change e.g., 2Tx-2Tx into dual Tx - dual Tx for uplinkTxSwitching? If we select 2Tx, 2Tx for uplinkTxSwitching can stay as they do.

- For 4Tx, we haven’t used quad Tx thus far. If we used dual Tx instead of 2Tx, we would need to use quad Tx for consistency.

Observation 7: From following current situations, if we use one of dual Tx and 2Tx, using 2Tx is rational.

- If we selected dual Tx, i.e., replaced 2Tx with dual Tx, could we change e.g., 2Tx-2Tx into dual Tx - dual Tx for uplinkTxSwitching? If we select 2Tx, 2Tx for uplinkTxSwitching can stay as they do.

- For 4Tx, we haven’t used quad Tx thus far. If we used dual Tx instead of 2Tx, we would need to use quad Tx instead of 4Tx for consistency.

Proposal 4: Replace “dual Tx” with “2Tx”.

## modifiedMPR-Behaviour

A contribution of [3] raised a following question that what “If the bit is not set” mean and it listed following three possible interpretations with an example of n30 and n39 in Table L.1-1 of TS 38.101-1:

- Interpretation 1: modifiedMPR-Behavior should be present and this bit is set to 0 → From the perspective of English grammar

- Interpretation 2: modifiedMPR-Behavior is absent → Given it is not necessary for UE to specifically indicate this IE if only leftmost bit is utilized which is a waste of signaling

- Interpretation 3: Including both Interpretation 1 and 2. → Most reasonable for example a legacy Rel-16 UE supporting n39 may not indicate this IE

The interpretation 3 would be a rational one, given that n30 and n39 have only one definition per band in the Table. In our understanding, “If the bit is not set” would be captured in case one band has more than one definition for modifiedMPR-Behaviour. For example, n41 has four definitions. If a UE supports such a band, the UE may need to set the former 3 bits to 1 and the other 5 bits to 0. In this case, the UE has no choice but indicating a parameter of modifiedMPR-Behaviour where the bit string is 11100000. Probably, the text would exist for this situation, since the 4th bit has definition with two options, i.e., a definition for 1 and the other definition for 0 in the row, i.e., bit index 3. And a similar text has been inherited to other bands.

Overall, if a band has multiple definitions in modifiedMPR-Behaviour, there is not ambiguity to use “if the bit is not set”. If a band has single definition in modifiedMPR-Behaviour, it’s still understandable, but if we pursue more clarification, one possible way is to modify it in a way of “if the bit is not set, i.e., 0, or a parameter of modifiedMPR-Behaviour in BandNR is not present,”.

Observation 8: “If the bit is not set” has no ambiguity if a band has multiple definitions in modifiedMPR-Behaviour

Observation 9: “If the bit is not set” is still understandable even if a band has single definition in modifiedMPR-Behaviour if readers are aware of Observation 8.

Proposal 5: If RAN4 pursues more clarification, modify the text in a way of “if the bit is not set or a parameter of modifiedMPR-Behaviour in BandNR is not present,”

In our view, CRs for the above discussion are beyond editorial CRs. Thus, this should be discussed in maintenance agenda item. Therefore, we submitted companion CRs [4. 5] in agenda 4.2.

- Handling of NOTEs in a table

As a new aspect, we’ve observed that handling NOTEs in a table isn’t consistent across a specification or over the specifications.

Observation 10: handling NOTEs in a table is not consistent across a specification or over the RAN4 specifications

In addition, granularity of NOTEs applicability in a table may become even finer than originally MCC expected. For more specific, Table 5.5A.3.1-1a to Table 5.5A.3.1-1n in 38.101-1 have 16 NOTEs. As can be seen in the below example, the granularity of NOTEs applicability can be e.g., per UL band/band configuration.

Figure 4-1:  A part of Table 5.5A.3.1-1m in TS38.101-1

When readers try to find a specific CA configuration, it may not be a big problem, since if there is NOTE(s) for the CA configuration in the corresponding row(s), the readers can further check the definitions of the NOTE(s) in the bottom of the table. If, however, the readers want to know for which band configurations a certain NOTE(s) applies, it’s quite challenging to find out which band, band configuration, channel bandwidth etc has the specific NOTE(s). It also makes specifications maintenance more difficult, in case corrections are made for NOTEs in a table. In order to know what the recommended handling of NOTEs in a table is, we take a look at the below Figure 4-2, which is a part of Table 2 in TR21.801.

Figure 4-2:  A part of Table 2 in TR21.801

If we exactly followed this approach for all the tables in all 38.101-1 series, some table would look good, but some other tables like Table 5.5A.3.1-1I look not good, though good or not good is very subjective.

Figure 4-3:  A part of Table 5.5A.3.1-1I in TS38.101-1 to which the approach in 21.801 is applied.

One possible alternative approach is as shown below Figure 4-4. With this, if readers want to know e.g., which CA configurations have note 6, the readers can use search function of Microsoft word and the information is obtained from the NOTE column. Then, the readers can further identify which channel bandwidths the note 6 applies to from the channel bandwidths with superscript of NOTE 6.

Figure 4-4:  One possible approach

Observation 11: Granularity of NOTEs applicability in a table may be even finer than originally MCC expected in TR21.801.

Given that many notes invite more errors in the specifications and accordingly more correction CRs for them, it would be good to discuss handling of NOTEs together with MCC if RAN4 considers handling of NOTEs in a table in TR21.801 is obsolete.

Proposal 6: RAN4 should discuss necessity of more practical drafting rule for NOTEs handling in a table for the future specifications.

- Conclusion

This paper obtained following eleven observations and six proposals.

<simplification of ΔTIB,c and ΔRIB,c tables>

Observation 1: RAN4 requirements are minimum requirements to accommodate various UE implementation options and the requirements should be reasonable for the industry including aspects such that power consumption, cost etc.

Observation 2: Option 2 tightens the existing requirements which may preclude some implementation choices.

Observation 3: Option 2 has no technical justification to tighten the existing requirements as well as does not guarantee future challenges that RAN4 may face, e.g., larger number of bands in a certain frequency range may require more relaxations.

Observation 4: Adopting Option 2 means no tables for relaxation values and it leads to a situation that specifications lose some essential information like simultaneous Rx/Tx and accompanied values specifically for BCs with two bands.

Observation 5: Having reasonable values in specifications is also one of the key factors for the quality of the specifications and the values in the current specifications are the ones that we have been able to find out as the most reasonable values.

Proposal 1: RAN4 shall not adopt Option 2 or similar approaches and not tighten the existing requirements for the sake of specification quality improvement.

Proposal 2: Adopt the following approach.

      - Apply followings to band combinations (BCs) with two bands,

        - Maintain the values for ΔTIB,c and ΔRIB,c for BCs in the tables for BCs with two bands.

        - Put 0 dB being handled as zero (i.e., not listed in the tables for two bands) with respective BCs in the tables for BCs with two bands.

      - Apply followings to BCs with more than two bands, values for ΔTIB,c and ΔRIB,c for BCs with more than two bands are derived by taking maximum among all the values for the corresponding respective bands from all the fallback BCs only with two bands.

        - For example, regarding CA_n1-n3-n78, ΔTIB,c for CA_n1-n3, CA_n1-n78 and CA_n3-n78 are (0.3 dB, 0.3 dB), (0.3 dB, 0.8 dB) and (0.6 dB, 0.8 dB), respectively.

        - Take max for each band like max (0.3 dB, 0.3 dB) for n1, max (0.3 dB, 0.6 dB) for n3 and max (0.8 dB, 0.8 dB) for n78. Then, the values for the respective bands for CA_n1-n3-n78 become (0.3 dB, 0.6 dB, 0.8 dB) = (n1, n3, n78).

        - It’s noted that the values currently specified are (0.6 dB, 0.6 dB, 0.8 dB) = (n1, n3, n78). Thus, this does not always give the same values in the current specifications.

        - Thus, some exceptions are needed for specifically BCs whose degree of implementation challenges drastically changes compared to fallback BCs with two bands

      - If a BC with more than two bands has more than two bands at least in any one of the frequency ranges, below 1 GHz, 1.4 - 2.7 GHz and above 2.7GHz, RAN4 shall discuss values on case-by-case basis instead of applying the 2nd main bullet in the proposal 2 to it and list the values in the corresponding tables as exception

        - For a BC with more than three bands, values for ΔTIB,c and ΔRIB,c for bands group(s) consisting of more than two bands which falls into one of the frequency ranges, the values are derived based on the 3rd main bullet (i.e., case by case analysis is needed) in the proposal 2, while values for ΔTIB,c and ΔRIB,c for the remaining band(s) are derived based on the 2st main bullet in the proposal 2 .

        - E.g., A+B+C+D+E where A+B+C falls into one of the frequency ranges, then, values derived based on the 3rd main bullet in the proposal 2 applies, while the 2nd main bullet applies to D and E to derive values.

        - For another example, if a completely new band combination with four bands whose three of the four bands are below 1GHz, then, the proponent of this band combination shall provide technical analysis on values in their TP.

      - Those above shall be allowed to be revisited whenever necessary.

        - This is because there may be aspects that we have not been aware of at the writing of this contribution

<Wording ambiguities>

Observation 6: The discussion on CA requirement applicability requires technical discussion.

Proposal 3: CA requirement applicability should be handled in HPUE CA power class agenda item.

Observation 7: From following current situations, if we use one of dual Tx and 2Tx, using 2Tx is rational.

- If we selected dual Tx, i.e., replaced 2Tx with dual Tx, could we change e.g., 2Tx-2Tx into dual Tx - dual Tx for uplinkTxSwitching? If we select 2Tx, 2Tx for uplinkTxSwitching can stay as they do.

- For 4Tx, we haven’t used quad Tx thus far. If we used dual Tx instead of 2Tx, we would need to use quad Tx instead of 4Tx for consistency.

Proposal 4: Replace “dual Tx” with “2Tx”.

<modifiedMPR-Beehaviour>

Observation 8: “If the bit is not set” has no ambiguity if a band has multiple definitions in modifiedMPR-Behaviour

Observation 9: “If the bit is not set” is still understandable even if a band has single definition in modifiedMPR-Behaviour if readers are aware of Observation 8.

Proposal 5: If RAN4 pursues more clarification, modify the text in a way of “if the bit is not set or a parameter of modifiedMPR-Behaviour in BandNR is not present,”

<Handling of NOTEs in a table>

Observation 10: handling NOTEs in a table is not consistent across a specification or over the RAN4 specifications

Observation 11: Granularity of NOTEs applicability in a table may be even finer than originally MCC expected in TR21.801.

Proposal 6: RAN4 should discuss necessity of more practical drafting rule for NOTEs handling in a table for the future specifications.

- Reference

[1] R4-2410714, WF on UERF_Spec_Improvement, Qualcomm

[2] R4-2407225, Simplifying RX/TX RF Refsens/Pout tables, Apple

[3] R4-2407625, Views on technical wording ambiguities and table modifications for spec improvement, Samsung

[4] R4-2411240, (NR_newRAT-Core) Clarification on modifiedMPR-Behaviour, Huawei, HiSilicon

[5] R4-2411241, (NR_newRAT-Core) More on clarification on modifiedMPR-Behaviour, Huawei, HiSilicon