3GPP TSG-RAN4 Meeting #112	R4-2411254

Maastricht, NL, 19th – 23rd August, 2024

Agenda item:	8.17.2

Source:	Keysight Technologies

Title:	OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases

Document for:	Approval

# Introduction

This contribution addresses test system and testability considerations for AI/ML beam management use cases.

# Discussion

Beam Management is one objective of the AI/ML [1]

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

While not specifically defined anywhere, we believe that the beam management topic is primarily applicable to FR2 and eventually possibly to the 7-24 GHz range but likely not to FR1. As RAN1 has only focused on FR2 simulations, we propose to limit the RAN4 work on FR2 only.

Observation 1: The AI/ML beam management objective seems primarily targeted for FR2.

Proposal 1: It is proposed to limit the testability discussions of AI/ML Beam Management test cases to FR2 only.

The following agreements were made in the last meeting regarding testability and test system topics [2]

Issue 2-5: Test setup needs Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

and

Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

This contribution intends to provide additional views from our side how these agreements can be addressed in OTA test systems using channel emulation.

# Applicability of OTA Test Systems to AI/ML Beam Management Test Conditions

The review of FR2 OTA test systems is not repeated here but can be found in [3].

In the following, we review certain applicabilities of FR2 OTA test systems to AI/ML beam management test conditions.

As highlighted in [3], if the intention/requirement is to implement CDL channel models, only the FR2 MIMO OTA test system is applicable. The IFF test system with just a single probe is certainly not able to implement a CDL channel model. While the Enhanced IFF test system has multiple probes, the relatively large probe separations and lack of absolute probe locations defined prevent it from being applicable to CDL channel models.

Observation 2: Only the FR2 MIMO OTA test system can properly and accurately implement CDL channel model.

If the intention/requirement is to implement TDL channel models, any of the FR2 test systems could be used as a single, dual-polarized probe with high isolation between ports/polarizations is sufficient to emulate a TDL channel model.

Observation 3: All existing FR2 test systems could properly and accurately implement a TDL channel model.

If multiple TRPs must be realized simultaneously (with TDL channel models at most), only the Enhanced IFF and FR2 MIMO OTA test systems are applicable given the multiple probes used in the respective systems. This scenario is visualized schematically in Figure 1 for either test system.

Figure 1: Test systems supporting two simultaneous TRPs (with TDL channel models at most), left: Enhanced IFF, right: FR2 MIMO OTA test system.

Observation 4: The Enhanced IFF and FR2 MIMO OTA test systems can realize multiple TRPs (with TDL channel models at most) simultaneously.

If the test system for AI/ML Beam Management needs to present true FF conditions, only the IFF and Enhanced IFF methodologies can be used.

Observation 5: Only the IFF and Enhanced IFF truly present FF conditions.

If the test system for AI/ML Beam Management needs to present two spatially separated TRPs with CDL channel models simultaneously, an expanded FR2 MIMO OTA test system with more than 6 probes would be required as schematically visualized in Figure 2 with three different potential implementations. Existing FR2 MIMO OTA test system would not easily be upgradeable to support two vastly different TRP directions as the chamber size might not be sufficient; additionally, the cost and complexity of such test system would likely be significantly higher due to increased number of probes/RRHs/upgrade of chamber, etc. However, if the difference in TRP directions could be limited, less complex and less costly system upgrades could be envisioned as shown in the centre and right of Figure 2.

Figure 2: Expanded FR2 MIMO OTA test systems supporting two TRPs with CDL channel models simultaneously.

Observation 6: The default FR2 MIMO OTA test system cannot present two vastly dislocated TRPs with CDL channel models simultaneously.

Observation 7: Depending on the angular separation of the TRPs, system upgrades of existing FR2 MIMO OTA systems could increase cost&complexity and potentially the chamber size.

Some discussions [11] were held whether two dislocated TRPs could be presented to the UE in a sequential fashion, i.e., one TRP at t=t0 and another TRP from a different direction at t=t0+DT.

When presenting the UE with two different and dislocated TRPs involve UE positioner changes, the following applies:

- For the FR2 MIMO OTA system, the two different TRPs being presented to the DUT (including channel propagation conditions modelled as CDL ) are visualized in Figure 3 with a 30º offset between the two TRPs.

- Similarly, Figure 4 illustrates this approach with two different TRPs being presented for the 1 AoA IFF test system (with at most a TDL channel model applied).

Figure 3: FR2 MIMO OTA test system supporting two TRPs with CDL channel models sequentially, left at t=t0 and right at t=t0+DT with a 30° angular offset.

Figure 4: IFF test system supporting two TRPs with TDL channel models sequentially, left at t=t0 and right at t=t0+DT with a 30° angular offset.

Here, it should be noted that the minimum achievable DT is a function of the positioning time, i.e., the time it takes for the positioner to move the required angle by acceleration, moving at max speed, deceleration, and a dwell time to guarantee the UE being at a steady state. Even for small angular movements, e.g., 15°, the positioning time is in the order of ~1s which could be too excessive to properly test  the AI/ML beam management requirements.

Observation 8: Multiple TRPs can be presented to the UE sequentially by rotating the UE but the positioning time in excess of 1s might be too slow.

When presenting two different TRPs to the UE without UE positioner changes simultaneously, see Figure 1, or sequentially, a multi-probe system is required, e.g., in the sequential case:

For the FR2 MIMO OTA system, the two different TRPs presented (including at most TDL channel models applied) to the DUT at t=t0 and t=t0+DT are visualized in Figure 5with a 30º offset between the two TRPs.

- Similarly, Figure 6 illustrates this approach with two different TRPs being presented to the DUT at t=t0 and t=t0+DT for the Enhanced IFF test system (with at most a TDL channel model applied) with a 60º offset between the two TRPs.

In both cases, DT is insignificant and is limited by the switching network.

Figure 5: FR2 MIMO OTA test system supporting two TRPs with TDL channel models (at most) sequentially, left at t=t0 and right at t=t0+DT with a 30° angular offset.

Figure 6: Enhanced IFF test system supporting two TRPs with TDL channel models (at most) sequentially, left at t=t0 and right at t=t0+DT with a 60° angular offset.

Observation 9: Multiple TRPs can be presented to the UE sequentially without UE positioner movements with either the Enhanced IFF or FR2 MIMO OTA test system with insignificant delays.

Table 1: Applicability of existing OTA test systems to testability criteria for AI/ML beam management

| Applicability Criteria | IFF | Enhanced IFF | FR2 MIMO OTA |
| --- | --- | --- | --- |
| CDL Channel Models | No | No | Yes |
| TDL Channel Models | Yes | Yes | Yes |
| Multiple TRPs simultaneously | No | Yes | Yes |
| True FF Conditions | Yes | Yes | Not necessarily |
| Multiple TRPs with CDL channel models presented simultaneously | No | No | Requires upgrade whose cost & complexity might be not negligible |
| Multiple TRPs with TDL channel models presented simultaneously or sequentially at t0 and t0 + DT (without positioner movement) | No | Yes | Yes |
| Multiple TRPs with CDL channel models presented sequentially at t0 and t0 + DT (using positioner movement) | No | No | Yes, but DT≥1s might be excessive |
| Multiple TRPs with at most TDL channel models presented sequentially at t0 and t0 + DT (using positioner movement) | Yes, but DT≥1s might be too slow | Yes, but DT≥1s might be too slow | Yes, but DT≥1s might be too slow |

Proposal 2: Take the FR2 OTA test systems and applicabilities in Table 1 into account for the testability discussions of AI/ML Beam Management.

Given the complexity of introducing multiple, simultaneous TRPs (with large angular separation between TRPs) beyond 2, e.g., lack of absolute probe locations for Enhanced IFF and lack of wide angular separation for FR2 MIMO OTA, it is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required.

Observation 10: It is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required

# Justification for CDL Models, Reasonable Beamforming Scenarios, and Test System Impact/Implementation

The performance of beam management operations depends strongly on the spatial characteristics of the multipath propagation environment. The UE receives multiple copies of the signal as clusters of different AoAs and delays due to multipath propagation.

A simplified illustration of multipath propagation environment and two alternative gNB and UE beam selections is shown in Figure 7.

Figure 7: Illustration of multipath propagation environment and two alternative gNB and UE beam selections.

Each of the multipath propagation paths can be modelled as multiple close-by rays which compose a cluster with propagation model dependent departure and arrival angle spreads. These real-world propagation effects can be approximated by the CDL channel models with inclusion of 3D antenna/beam radiation patterns. It is essential to include realism into testing of beam management operations based on AI/ML algorithms. Realistic spatial channel models, such as CDL models should be used for the testing instead of overly simplified TDL models which cannot reflect critical beamforming effects at all.

Observation 11: CDL models are realistic channel models for beam management test cases while TDL models which cannot reflect any realism for the beamforming effects at all.

The impact of beamforming to resulting time-varying multipath propagation channel characteristics has been analysed thoroughly in [12][13]. Applying beamforming at the gNB will effectively filter out many of the multipath propagation paths and simplifying the resulting channel model seen by the UE to a reasonable number of strong clusters for multiprobe OTA environment modelling. However, depending on the applied gNB beam, the resulting dominant cluster arrival angles can change as illustrated in Figure 8 for CDL-B model for 4 strong gNB beam candidates.

Figure 8: BS beam gain patterns and cluster powers and AoAs of CDL-B model for 4 strongest beams.

This clearly illustrates that there are multiple strong gNB beam candidates for beam selection in multipath propagation environments and that the resulting power angular spectrum received by the UE can be different for each gNB beam option.

Observation 12: Multiple strong gNB beam candidates for beam selection in multipath propagation environments exist.

Observation 13: Each gNB beam candidates can result in different arrival PAS for the UE reception, potentially impacting also UE beam selection.

Observation 14: Beamforming performed at both link ends (gNB and UE) filters out weak multipath clusters of the channel model, hence applying gNB beamforming simplifies the resulting channel model seen by the UE to a reasonable number of strong clusters.

The number of gNB beam candidates has an impact on test system complexity, specifically related to the system simulator and channel emulator. It is therefore important to get feedback on from infra vendors to clarify the details around gNB beam candidates that should be presented to the UE, including the minimum number of beams, beam steering capabilities/codebook, latency.

Proposal 3: Infra vendors to clarify the details around gNB beam candidates, including the minimum number of beams, beam steering capabilities/codebook, latency, that should be presented to the UE.

The AoA profile (PAS) illustrations presented in Figure 8 are simplified in that sense that it does not include the cluster-wise angle spreads with multiple rays. Only cluster centroids are visualized in the illustrations for simplicity. In reality, the cluster-wise ray PAS is also impacted by the gNB (and UE) beam.

A sample ray distribution of the UMi CDL-C channel model is visualized in Figure 9 [14]. Here, it should be highlighted that in FR2, small per cluster angular spread (AS) can be observed, when gNB beamforming is considered with the channel model.

Figure 9: Distribution of rays with indication of powers for UMi CDL-C [14]

The illustrations presented above show only the average powers of each cluster. Each of the clusters has independent fading profiles over time, which makes the optimal beam selection at the gNB and at the UE more challenging. The decision of beam selection cannot be made based on a single power measurement due to time variant fading.

The cluster fading characteristics depend on each cluster angle spread and AoA direction with respect to direction of travel. Furthermore, the beamforming also impacts the cluster-wise ray powers as illustrated in Figure 8. Therefore, the resulting fading channel characteristics for different beam selection options are different, which will further impact the link performance for the different beam selection options.

The changes of beam states due fading and/or beam management operation will potentially cause multiple changes in channel model characteristics as explained in [13]. The emerging path components, due to dynamic allocation of beams, cause sudden changes in radio channel characteristics. Not only the PAS change, but consequently also the dominant cluster Doppler and delay changes, which may affect the synchronization and channel estimation accuracy of the receiver. Furthermore, also the Doppler power spectrum changes, causing variation in temporal auto-correlation function and channel coherence time. Also, the power delay profile changes, causing sudden changes to frequency correlation function and the coherence bandwidth. Dynamic variation of PAS may result variation of Tx and Rx correlation coefficients of different sub-arrays. These aspects make the optimal gNB and UE beam pair selection challenging, and the AI/ML algorithms are expected to cope with this complexity.

Observation 15: gNB and UE beam selection impacts resulting cluster-wise fading characteristics.

Observation 16: To properly assess AI/ML algorithms, the complexities of beam dependent fading characteristics and realistic channel models need to be taken into account.

These real-world propagation effects are captured in CDL channel models with the FR2 MIMO OTA system illustrated schematically in Figure 10 with inclusion of 3D antenna patterns, whereas the TDL models cannot include the impact of 3D antenna beam patterns.

The principle of the FR2 MPAC OTA test methodology for geometric channel models, illustrated in Figure 10, is the following [16]: In the bottom right is an assumed two-cluster channel model in the angular domain and in the bottom left in the delay domain. Both the blue and green clusters are mapped onto one or more probes within the anechoic chamber (top left). The actual fading, with all time/Doppler and frequency/delay domain effects, is generated inside the fading emulator. The function of fading emulator is to convolve input signal from UE/BS emulator with the fading radio channel impulse responses and map the faded signals to appropriate probes within the anechoic chamber. Power levels, polarizations, and directions of probes generate the power angular spectrum (PAS) within a test zone around the DUT. The target PAS to be reconstructed is specified by the target channel model.

Observation 17: The performance of beam management operations depends strongly on the spatial characteristics of the multipath propagation environment which can be accurately assessed in the FR2 MPAC OTA system.

Figure 10. The principle of FR2 MPAC OTA and emulation of geometric channel models [16].

It is therefore proposed to consider the FR2 MIMO MPAC OTA test system as the baseline for AI/ML Beam management - DL Tx beam prediction use case.

Proposal 4: Consider the FR2 MIMO MPAC OTA test system as the baseline for AI/ML Beam management - DL Tx beam prediction use case.

Considering the BM-Case1, i.e., spatial-domain downlink beam prediction from [17], the requirement is to emulate the corresponding L1 RSRPs for Set B of beams in a lab environment as illustrated schematically in Figure 11.

Figure 11: Example Lab Environment for BM-Case1

In order to define requirements, the beam radiation patterns of the beams of Set A or B need to be defined for example by defining the antenna array model and beam directions of each beam index of specific beam set. The resulting average channel gain with the Tx- and Rx- antenna models embedded can be easily calculated for a known channel model using the principles defined in Clause 7.3 of [18]. Subsequently, the target L1 RSRP can be calculated by defining the Tx-power and NR numerology related frequency response sampling etc. parameters.

The beam specific reference channel gains and respective L1-RSRPs for each beam of different beam sets can be calculated only by using a spatial channel model, e.g. CDL model. Beam specific channel gains or L1-RSRPs cannot be determined based on TDL models. Therefore, a spatial channel model, such as CDL model, is required as a basis of test condition definition to determine RSRP relations between the beams of different sets.

Note that the calculation of resulting average channel gain with the Tx- and Rx antenna models embedded and target L1 RSRP for each beam of Set A (from L1 RSRP measurement results for each beam of Set B) will be done by the device when exercising AI/ML beam management functionality in actual deployment. Therefore, such calculation needs to be considered when quantifying the achievable performance of the functionality which makes spatial channel model a requirement for the test setup.

Observation 18: A spatial channel model is required as a basis of test condition definition to determine L1-RSRP relations between the beams of different sets.

Observation 19: The reference L1-RSRP relations between the beams can be determined using the channel modelling methodology defined in TR 38.901 with parameters and details based on TR 38.827/TS 38.151 after the Tx-beams and Rx-beams are defined

As the channel modelling and 3D beam embedding aspects are inherently included in the CDL channel modelling methodology and the details are defined, the implementation of CDL models is straightforward and not more complex than TDL model implementation.

Observation 20: From a purely channel model perspective and the realization in state-of-the-art channel emulators, implementation of CDL models is not more complex than TDL models

If the UE Rx-beamforming is not disabled and if its antenna pattern is not isotropic, it will have impact also on the DL-Tx-beam power relations that the UE is observing in real-life or in the test in the OTA chamber. As the UE is connected OTA in the test and it is operating using its actual antenna in the chamber, the UE antenna pattern should not be embedded in the channel model. Therefore, only the BS antenna beam patterns should be embedded into the channel model when determining the reference L1-RSRP relations between different Tx beams. The UE antenna pattern can be considered isotropic for this definition. This approach/definition is already utilized in existing FR1 and FR2 channel model definitions for OTA testing.

Considering that DL Tx beam specific channel model multipath profile is filtered by the DL Rx beams, the UE antenna characteristics and beam sweeping procedures will have a crucial impact on the test results. As the UE beamforming cannot be fixed for this test and it is filtering the received signals using its own 3D antenna characteristics, the purpose of the OTA test environment is to model the multipath characteristics of the reference spatial channel model such that the Rx-beam direction and shape/width dependent received L1-RSRP follows the reference channel model. As the UE is operating using the physical antenna, it is not possible to embed the Rx-antenna pattern into the channel model. Obviously, it is also not possible to determine the received L1-RSRP with different DL Rx-beams using a TDL model even if the Rx-beam pattern would be known. If non-spatial channel model (TDL) or non-spatial OTA chamber configuration (single probe) is used in the test, it is not possible to reflect the RSRP differences between the different Rx-beam options.

Each Tx-beam will have a specific AoA profile as explained in Figure 8. If a multi-probe test system is used, the probe constellation should be defined such that the multipath characteristics of the reference spatial channel model characteristics are realized for the DUT such that the Rx-beam direction and shape/width dependent received L1-RSRP follows the reference channel model. Otherwise, the Rx-beam characteristics dependent L1-RSRP relations between different beams will not be consistent with realistic data used to train AI/ML model enabling correct predictions.

In summary, the test conditions should be based on logical and realistic behaviour of observed L1-RSRP relations of different beams in spatial and temporal domain. Both spatial and temporal domain beam specific channel gain (L1-RSRP) behaviour depend on 3D antenna characteristics with respect to the spatial channel model as explained above. Also, as explained above, the impact of the UE 3D antenna beam characteristics (direction and shape) to the resulting L1 RSRP depend on the Rx-PAS of the reference spatial channel model vs. the 3D UE beam characteristics, and as the test condition for the UE is OTA, the beam impact can be emulated only if the OTA test system emulates the (Tx-beam embedded) multipath Rx-PAS profile of the reference channel model for the DUT. This can be implemented by using multiple probes following the methodology of standard MIMO OTA test defined in [9][10].

The Tx-beam specific power offsets and beam filtered channel characteristics (incl. relevant Rx-PAS) can be modeled by defining the Tx-beam patterns and channel models. A beam grid with multiple beams needs to be defined for the beam management testing. A good reference for basis for beam management testing can be taken from Clause 7.3 of [9], where a beam grid of 128 beams is defined as illustrated in Figure 12, where the gain of each beam is indicated by the color coding. This can be considered to represent a good example approximation of typical FR2 gNB beam grid.

Figure 12: FR2 MIMO OTA Tx-beam grid

The average channel gains with each Tx-beam index are simulated with the channel model in FR2 MIMO OTA channel model definition and strongest beam is chosen to be included into the channel model. Illustration of resulting channel gains with each beam index/direction for CDL-C UMi channel model is shown in Figure 13. Beam index leading to highest channel gain, i.e. beam 50 is chosen for the MIMO OTA channel model.

Figure 13: Illustration of channel gain with each Tx-beam index with CDL-C UMi channel model (FR2 MIMO OTA)

However, any other beam or multiple beams can be chosen to be included into the channel model. The number of TX-beams that can be included into emulation depends on the test system capability. If the number of probes of the FR2 MIMO OTA test system is extended from 6 to 8, multiple TX-beams can be emulated with sufficient accuracy. The accuracy of emulated RX-PAS is measured by PSP (PAS Similarity Percentage) metric in FR2 MIMO OTA [9]. A PSP simulation study with FR2 MIMO OTA channel models of [9] was performed for 4 strongest TX-beams in an 8-probe setup. The simulation results summarized in Table 2 indicate higher than 80% PSP for most of the simulated channel models and beam options and higher than 70% PSP for all simulated channel model options. The simulation was performed for all InO and UMi scenario NLOS CDL channel models (A, B and C). The LOS models D and E are less sensitive to probe constellation and yield higher PSP than the NLOS models and therefore omitted in this study. Thus, based on the simulation study, we can state that the Rx-PAS of all CDL models of UMi and InO scenarios can be emulated with sufficient accuracy for 4 strongest Tx-beams of a typical gNB beam grid.

Table 2: PSP simulation results for 4 strongest TX-beams of UMi and InO scenario NLOS CDL models

| Model | PSP (Beam-1) [%] | PSP (Beam-2) [%] | PSP (Beam-3) [%] | PSP (Beam-4) [%] |
| --- | --- | --- | --- | --- |
| NR MIMO OTA InO CDL-A | 93.6 | 93.4 | 93.9 | 96.9 |
| NR MIMO OTA InO CDL-B | 71.9 | 72.4 | 76.3 | 79.2 |
| NR MIMO OTA InO CDL-C | 77.5 | 75.3 | 85.0 | 71.8 |
| NR MIMO OTA UMI CDL-A | 78.6 | 78.7 | 95.0 | 78.2 |
| NR MIMO OTA UMI CDL-B | 89.7 | 82.6 | 89.3 | 81.9 |
| NR MIMO OTA UMI CDL-C | 95.9 | 85.1 | 95.9 | 84.5 |

A visual presentation of PSP simulations of the 4 strongest TX-beams with UMi CDL-C channel model is shown in Figure 14 for reference.

Figure 14: Illustration of channel PSPs for 4 strongest beams for UMi CDL-C

Observation 21: The RX PAS of 4 strongest TX beams can be emulated reliably and accurately with 8 probes

Based on our experience with developing beam management test cases, we believe a relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes with a sample system visualized in Figure 15, is suitable to meet the following assumptions for a single TRP or two TRPs with close angular proximity:

- Emulation of Tx-beam powers and RX-PAS for 4 strongest or most relevant Tx-beams

- 4 Tx-beam RX-PAS profiles can be emulated for CDL-A, B, C, D and E UMi/UMa/InO scenarios utilizing 8 probes.

- Additionally, to emulate set B Tx beams beyond 4 [up to 16], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics.

Figure 15: Sample FR2 test system supporting AI/ML BM testing. Probes in blue correspond to the FR2 MIMO OTA probes.

Observation 22: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 Tx-beam RX-PAS profiles.

Observation 23: To emulate set B Tx beams beyond 4 [up to 16], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles.

Some of these testing aspects are already supported by Keysight’s S8708A Advanced Performance Test (APT) toolset which covers FR2 MIMO OTA tests according to [9], [10] and also allows validation of beam management in real world radio channel conditions by integrating channel and network emulation capabilities with a 3D OTA test environment.

With just minor adjustments, it can address all the following challenges:

- Validate dynamic beam management performance for over the air 3D spatial fading and blocking conditions

- Verifying that gNodeB and UE beam management procedures operate seamlessly in highly dynamic fading channels at FR2 in all mobility scenarios

- Simulating dynamic channels with time-variant characteristics like Doppler shifts and temporal fading

- Testing AI models' robustness under various controlled yet realistic conditions to ensure reliability

Observation 24: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Proposal 5: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# AI/ML Training vs Conformance Testing

While the training of the AI/ML models/algorithms is not part of the 3GPP scope and conformance testing, the eventual assessment of the AI/ML models and the efficacy of these models related to beam management are scope of the WI [1].

The training exercise is left to OEMs and the systems and algorithms for training might be largely proprietary. However, it can be assumed that the AI/ML algorithms will be trained using highly realistic environmental conditions, i.e., channel models, AoAs, etc., to optimize the UE performance in the field. OTA test systems with a high degree of complexity to correspond to realistic, real-world conditions could be leveraged for such training. It is also possible that the AI models may need to be customized for each UE model/SKU and not just for each chipset. Training with anything other than highly realistic CDL channel models is likely to provide inferior performance. Training using synthetic data obtained from hyper-realistic digital twins of real-world environments is also a possibility.

Conformance testing certainly does not require test systems with the same degree of complexity and capabilities as the training and should rather be a subset of systems used for training. Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results and hence is not recommended.

Observation 25: Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The AI/ML beam management objective seems primarily targeted for FR2.

Observation 2: Only the FR2 MIMO OTA test system can properly and accurately implement CDL channel model.

Observation 3: All existing FR2 test systems could properly and accurately implement a TDL channel model.

Observation 4: The Enhanced IFF and FR2 MIMO OTA test systems can realize multiple TRPs (with TDL channel models at most) simultaneously.

Observation 5: Only the IFF and Enhanced IFF truly present FF conditions.

Observation 6: The default FR2 MIMO OTA test system cannot present two vastly dislocated TRPs with CDL channel models simultaneously.

Observation 7: Depending on the angular separation of the TRPs, system upgrades of existing FR2 MIMO OTA systems could increase cost&complexity and potentially the chamber size.

Observation 8: Multiple TRPs can be presented to the UE sequentially by rotating the UE but the positioning time in excess of 1s might be too slow.

Observation 9: Multiple TRPs can be presented to the UE sequentially without UE positioner movements with either the Enhanced IFF or FR2 MIMO OTA test system with insignificant delays.

Observation 10: It is advisable to limit the maximum number of spatially separated TRPs to 2 if more than 1 TRP is required

Observation 11: CDL models are realistic channel models for beam management test cases while TDL models which cannot reflect any realism for the beamforming effects at all.

Observation 12: Multiple strong gNB beam candidates for beam selection in multipath propagation environments exist.

Observation 13: Each gNB beam candidates can result in different arrival PAS for the UE reception, potentially impacting also UE beam selection.

Observation 14: Beamforming performed at both link ends (gNB and UE) filters out weak multipath clusters of the channel model, hence applying gNB beamforming simplifies the resulting channel model seen by the UE to a reasonable number of strong clusters.

Observation 15: gNB and UE beam selection impacts resulting cluster-wise fading characteristics.

Observation 16: To properly assess AI/ML algorithms, the complexities of beam dependent fading characteristics and realistic channel models need to be taken into account.

Observation 17: The performance of beam management operations depends strongly on the spatial characteristics of the multipath propagation environment which can be accurately assessed in the FR2 MPAC OTA system.

Observation 18: A spatial channel model is required as a basis of test condition definition to determine L1-RSRP relations between the beams of different sets.

Observation 19: The reference L1-RSRP relations between the beams can be determined using the channel modelling methodology defined in TR 38.901 with parameters and details based on TR 38.827/TS 38.151 after the Tx-beams and Rx-beams are defined

Observation 20: From a purely channel model perspective and the realization in state-of-the-art channel emulators, implementation of CDL models is not more complex than TDL models

Observation 21: The RX PAS of 4 strongest TX beams can be emulated reliably and accurately with 8 probes

Observation 22: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 Tx-beam RX-PAS profiles.

Observation 23: To emulate set B Tx beams beyond 4 [up to 16], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles.

Observation 24: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Observation 25: Performing conformance testing with TDL models that are far less realistic and that the ML model likely has not been trained on due to the lack of realism is likely to provide inconsistencies in test results.

Proposal 1: It is proposed to limit the testability discussions of AI/ML Beam Management test cases to FR2 only.

Proposal 2: Take the FR2 OTA test systems and applicabilities in Table 1 into account for the testability discussions of AI/ML Beam Management.

Proposal 3: Infra vendors to clarify the details around gNB beam candidates, including the minimum number of beams, beam steering capabilities/codebook, latency, that should be presented to the UE.

Proposal 4: Consider the FR2 MIMO MPAC OTA test system as the baseline for AI/ML Beam management - DL Tx beam prediction use case.

Proposal 5: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# References

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #103, March 2024

- R4-2401570, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2409770, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM)

- TR 38.871, Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements

- TR38.810, Study on test methods

- R4-2309469, On Absolute Probe Locations for Multi-Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #107, May 2023

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- R4-2404215, On AI/ML RAN4 Use Cases: Beam Management, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 110bis, April 2024

- R4-1711840, Channel model spatial filtering using 38.803 gNB Tx antenna assumptions, Keysight, 3GPP TSG-RAN WG4 Meeting #84bis, October 2017

- R4-1713774, Impact of fast fading on strongest beam index for static geometry channels, Keysight, 3GPP TSG-RAN WG4 Meeting #85, December 2017

- R4-2002478, System Implementation of FR2 3D MPAC Systems, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #94-e, March 2020

- R4-1705838, Simplified sectorized MPAC for RRM/Demodulation Setup, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #83, May 2017

- R4-1813602, Channel model and test method considerations for NR radiated requirements, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #88-bis, October 2018

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz