3GPP TSG-RAN WG4 Meeting #112	                                 R4-2411256

Maastricht, Nethelands, August 19 – August 23, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on simultaneous Rx-Tx of CA_n40-n41

Agenda item:	7. 11. 2

Document for:	Discussion

Introduction

In the last RAN4#111 meeting, the remaining issues on the simultaneous Rx-Tx in Rel-18 was discussed [1] and the WF [2] was approved. In this contribution we discuss below topic for Rel-19 simultaneous Rx-Tx work item.

How to implement simultaneous Rx-Tx requirements to CA_n40-n41 need further discussion.

Discussion

Remaining issue regarding simultaneous Rx-Tx of CA_n40-n41

One of the remaining issues is about simultaneous Rx-Tx of CA_n40-n41 since these two bands are close to each other. This leads to high MSD for PC2 (31.1dB for n40 DL and 34.4dB for n41 DL) and PC3 (28.1dB for n40 DL and 31.4dB for n41 DL) due to lack of cross band isolation between them. The filter isolation performance can be improved but it causes the higher insertion loss of the filter. To avoid high MSD, separation of antennas for two bands might help increase the isolation for the Tx leakage but at least 6 antennas would be required for n41 4x4 DL MIMO operation, which is huge burden for compact size handheld UE.

Therefore, there have been concerns on relaxing the 4 Rx requirement of n41 or how to clarify on SRS-AS scheduling restriction or even disabling SRS-AS or disabling UL MIMO. However no concrete consensus has been made during Rel-18 discussion.

One example RF front end architecture for CA_n40-n41

Figure 1 shows one example simplified RF front end architecture for CA_n40-n41 using 4 antennas with simultaneous Rx-Tx operation (antenna switches are omitted for simplicity). It can be shown that n40 Tx leakage is going into n41 LNA2 which degrades sensitivity of n41 Rx. Similarly, n41 Tx SRS is also leaked to n40 LNA1 and n40 LNA2 (SRS antenna switch block is not shown here) which also impacts the sensitivity performance of n40 Rx.

Figure 1. One example RF front end architecture for CA_n40-n41 (SRS antenna switching block is not shown here)

Observation 1: Large MSD is expected for n40 Rx and n41 Rx with n41 4x4 DL MIMO using 4 antennas.

Implementation of simultaneous Rx-Tx of CA_n40-n41 using 4 antennas

Relaxing the 4 Rx requirement to 2 Rx requirement of n41 in case of simultaneous Rx-Tx of CA_n40-n41 is one of the simplest ways for the implementation. This might be done by sending signalling to BS which needs further discussion within RAN groups.

However, it should be noted that the Tx leakage of ACLR2 might not be a big concern except cell edge area. In other words, the noise interferer entering the other band’s receiver would be much lower in case where the UE is not too far from BS. The high order modulation such as 256QAM requires high SNR (~35dB) for demodulation which means that high T-put using 4x4 DL MIMO is not a feasible option for cell edge area, i.e. sensitivity area. Also, there is no sensitivity benefit from 4 Rx diversity gain since the MSD is already very excessive.

Observation 2: Tx leakage of ACLR2 might not be a big concern except cell edge area.

Therefore, one option for the implementation of simultaneous Rx-Tx of CA_n40-n41 using 4 antennas is to relax 4 Rx requirement to 2 Rx when UE is near cell edge and keep 4 Rx requirement when UE requires high data T-put using 4x4 DL MIMO. Figure 2 shows that n41 LNA2 and n41 LNA4 are off for 2 Rx relaxation when UE is near cell edge.

Figure 2. Relaxation to 2 Rx when UE is near cell edge.

Figure 3 shows another RF front end architecture for CA_n40-n41 supporting simultaneous Rx-Tx and n41 UL MIMO. Similarly, we can keep 4 Rx requirement in the case of high data T-put operation and relax to 2 Rx when UE is near cell edge with n41 UL MIMO.

Figure 3. Another RF front end architecture for CA_n40-n41 (SRS antenna switching block is not shown here)

Proposal 1: Relax 4 Rx requirement to 2 Rx when UE is near cell edge and keep 4 Rx requirement when UE requires high data T-put using 4x4 DL MIMO.

It should be considered that how and when 4 Rx is switched to 2 Rx vice versa. It might follow modulation order signalling such as MCS or RSRP information from BS or FFS.

Proposal 2: How to switch 4 Rx and 2 Rx is FFS. Interesting companies are invited to provide their views at the next meeting.

Conclusion

In this contribution, we discussed the remaining issue regarding the simultaneous Rx-Tx of CA_n40-n41 and proposed one implementation method as below.

Observation 1: Large MSD is expected for n40 Rx and n41 Rx with n41 4x4 DL MIMO using 4 antennas.

Observation 2: Tx leakage of ACLR2 might not be a big concern except cell edge area.

Proposal 1: Relax 4 Rx requirement to 2 Rx when UE is near cell edge and keep 4 Rx requirement when UE requires high data T-put using 4x4 DL MIMO.

Proposal 2: How to switch 4 Rx and 2 Rx is FFS. Interesting companies are invited to provide their views at the next meeting.

References

[1]	R4-2407610, “Discussion on the remaining issues of simultaneous Rx-Tx in Rel-18”, Huawei, Hisilicon, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 14th, 2024

[2]	R4-2410563, “WF on simultaneous Rx-Tx issues”, Huawei, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 14th, 2024