3GPP TSG-RAN WG4 Meeting #112	R4-2411258

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	8.17.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we are going to discuss following issue from last meeting.

- Issue 1-1: Post deployment testing

2	Discussion

2.1 Issue 1-1: Post deployment testing options

Issue 1-1: Post deployment testing - Proposals - Option 1: Option 1 is feasible only if model updates are infrequent - Option 2: Option 2 is feasible only if the validation can be performed by UE vendors individually (no need to test in a certified lab) - FFS whether some validation information needs to be shared with gNB - Option 3: Add an option in which the conformance test inputs are stored and used for post-deployment testing/validation - Option 4: Drop Option 1, only discuss post deployment handling as part of LCM - Option 5: RAN4 should discuss a framework to ensure that at least a validated model exists at the UE - Option 6: RAN4 should discuss a framework to ensure that non-validated/non-tested models are not used by the UE - Option 7: Others - Recommended WF To be discussed, multiple options ca be adopted Interested companies are invited to bring analysis on the feasibility of the options proposed so far.

In our understanding,

- Doing conformance test for all potential models/updates before deployment would lead to frequent operations which require significant efforts. The feasibility and necessity both are questionable.

- Major model update or new model release need to pass conformance test before loading to devices to ensure the performance meeting requirements. But for minor update or fine-tuning, the performance could be guaranteed by device vendors via sufficient offline tests before loading to devices. The loading procedure may relate to model transfer/update, which is still under discussion in other WGs. Whether and how to define test on this aspect can wait for more explicit progress.

- To ensure the reliability of offline tests, building a testing dataset could be a potential solution. It is suggested to consider the possibility of building a public testing database for device vendors, which can be used for model performance testing. The source and the composition of the testing database could be further discussed.

- Either conformance test or offline test, it still belongs to pre-deployment. Anyway, we need a mechanism to check how the functionality/model actually operates in the field after deployment. This is our understanding of post-deployment handling. Monitoring procedure is an indispensable feature of AI/ML functionality although it is not fully specified at this moment. Effective monitoring can provide timely reaction when deployed condition varies in the field to avoid severe performance degradation.

Proposal 1: Consider effective performance monitoring for post-deployment handling of minor model update or fine-tuning.

Proposal 2: Whether and how to define test on model transfer/update shall wait for more explicit progress from other WGs.

Proposal 3: Suggest to discuss potential solutions for offline performance tests conducted inside vendor.

Conclusion

In this contribution, the issue of post deployment testing is discussed with following proposals:

Proposal 1: Consider effective performance monitoring for post-deployment handling of minor model update or fine-tuning.

Proposal 2: Whether and how to define test on model transfer/update shall wait for more explicit progress from other WGs.

Proposal 3: Suggest to discuss potential solutions for offline performance tests conducted inside vendor.