3GPP TSG-RAN WG4 Meeting #112	R4-2411259

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	8.17.2

Source:	CAICT

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion

1	Introduction

In this contribution, we are going to discuss the following issues from last meeting.

- Issue 2-2: RSRP prediction accuracy

- Issue 2-5: Test setup needs

- Issue 2-6: Datasets for training/testing

- Issue 2-7: Consistency

2	Discussion

2.1 Issue 2-2: RSRP prediction accuracy

Issue 2-2: RSRP prediction accuracy - Proposals - Option 1: Difference between the predicted RSRP and the measured RSRP of the same Tx beam - Option 2: Difference between the predicted RSRP and the ideal RSRP of the same Tx beam - Option 3: Difference between the predicted RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 4: Difference between the predicted RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 5: RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K - Option 6: others - Recommended WF - To be discussed

Firstly, we think the discussion on the definition of RSRP prediction accuracy could be separated for UE-side model and NW-side model. Considering RAN4 mainly focus on UE-side test, we just provide our initial thoughts for UE-side model in following. We could come back for NW-side model if NW-side test would be considered.

For UE-side model, based on progress in RAN1, UE shall report predicted beam ID and corresponding RSRP values to gNB. Based on our understanding, gNB directly decides the best beam based on reported results from UE is more intuitive. If so, there can be four cases as following:

- Case 1: The Top-1 predicted beam is not within the Top-M actual beams

- Case 2: The Top-1 predicted beam is within the Top-M actual beam, but not the Top-1 actual beam, and the Top-1 actual beam is not within the Top-K predicted beams reported to gNB

- Case 3: The Top-1 predicted beam is within the Top-M actual beam, but not the Top-1 actual beam, and the Top-1 actual beam is within the Top-K predicted beams reported to gNB

- Case 4: The Top-1 predicted beam is the Top-1 actual beam

Case 1 is the worst, thus it is more necessary to ensure beam prediction accuracy, especially Top-1(%) and Top-1/K(%), to at least avoid the worst case. Case 4 has no impact on the overall performance as the best beam is chosen. For case 2 and 3, what affect the overall performance is the difference between the ideal RSRP of the actual best beam and that of the predicted best beam. Thus, adopting option 3 in the proposal as the definition of RSRP prediction accuracy is more sensible from system perspective. Option 3 may require DUT to do measurements for all setA beams to identify the actual best beam (ideal RSRP can be approximated as measured RSRP under sufficient high SNR level).

To ease the test, adopting the difference between the predicted RSRP and actual RSRP of Top-1 predicted beam, i.e., option 2, could also be considered. Theoretically, the more accurate the RSRP prediction, the more likely we are to pick the best beam.

For option 1 and 4, i.e., the options that compare predicted RSRP with measurement RSRP, a concern is both prediction and measurement have some level of uncertainty. The meaning of difference between two values, and how to define the accuracy requirements need clarification.

Proposal 1: For RSRP prediction accuracy, suggest to prioritize option 3 and 1, and potential test burden with each option in future discussion.

2.2 Issue 2-5: Test setup needs

Issue 2-5: Test setup needs - Proposals - Option 1: - number of Tx beams - AoA, AoD - Propagation conditions (including need for LoS/NLoS) - UE rotation yes/no - Option 2: - other parameters - Recommended WF - To be discussed Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

This issue has been extensively discussed in last meeting. There are many aspects related to the test environment, the critical part is how to emulate multiple beams and corresponding measurements with current settings.

In our understanding, different beams mean that radio signal would be shaped to different directions when transmitted. This directed signal would pass multiple paths and arrive at receiver’s panel with a set of various conditions, e.g., different arrival directions, different power levels, etc. Thus, the possibility of reflecting the effect of diverse Tx beams on receiver side through identifying a set of propagation conditions in channel model could be discussed in RAN4. The method proposed in R4-2404215 [1] could be considered as a starting point for discussion. Note that, the effect may also relate to the Rx beam selection method of receiver (e.g., fixed Rx beam, or best Rx beam, or random Rx beam, or others). RAN4 may need to discuss whether to consider a common assumption on Rx beam selection in the test.

Figure 1 Illustration of the effect on receiver side with different Tx beams

If current settings are deemed to be insufficient or infeasible for AI-based beam management, it would be necessary to pursuit the inputs from OTA testing perspective. Introducing a separate study objective into Rel-19 SI on NR FR2 OTA testing enhancement could be considered.

Proposal 2: RAN4 to study the possibility of reflecting the effect of diverse Tx beams on receiver side through identifying a set of propagation conditions in channel model.

Proposal 3: RAN4 to discuss whether to introduce the testing feasibility and possible methodology for AI/ML beam management into Rel-19 study item on NR FR2 OTA testing enhancement.

2.3 Issue 2-6: Datasets for training/testing

Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

In last meeting, several principles for training datasets had been agreed as above. As to testing datasets, it may relate to the form we adopt to test AI functionality/model. The TE can be connected to the DUT to run normal communication processes to test the AI functionality/model. Alternatively, the TE can generate test data according to test environments/conditions to serve as input for test. If later is adopted, we think some structural parameters of datasets should also be specified to unify the input format.

For the overfitting problem, we think this can be left to UE implementation in training and ensured by comprehensive generalization test.

Proposal 4: if testing datasets is adopted, suggest to specify basic structural parameters to unify input format of inference test.

Proposal 5: Suggest to discuss whether the overfitting problem can be relieved via generalization test.

2.4 Issue 2-7: Consistency

Issue 2-7: Consistency - Proposals - Option 1: consistency between set A and set B is reflected by below aspects: - Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference. - Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?) - Option 2: Consistency should be defined in a different way - Option 3: no need for any consistency definition - Recommended WF - To be discussed

As to the consistency between set A and set B, if it implies the same antenna/beam configuration and same channel modeling for both set A and B, we think this should be kept by default in the performance test. In generalization test, these conditions may or may not be the same, depending on the dimension of generalization we want to test.

More to this point is the consistency between training and inference based on the WID, as highlighted in the following. The discrepancy between the antenna/beam configuration implemented by DUT vendor and TE vendor would lead to a significant performance degradation in the test. The assumptions about network-side conditions that need to be aligned, as well as how to achieve this alignment, both require further study. The relevant discussion is also proceeding in RAN1 (summarized in R1-2405680), and we can wait for more progress on this issue. Offline collaboration would also be a way out, but this may require considerable engineering efforts.

[RP-240774] - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Proposal 6: Hold on the discussion on consistency until RAN1 has sufficient progress on this.

Conclusion

In this contribution, testability and interoperability issues for beam management are discussed with following proposals:

Proposal 1: For RSRP prediction accuracy, suggest to prioritize option 3 and 1, and potential test burden with each option in future discussion.

Proposal 2: RAN4 to study the possibility of reflecting the effect of diverse Tx beams on receiver side through identifying a set of propagation conditions in channel model.

Proposal 3: RAN4 to discuss whether to introduce the testing feasibility and possible methodology for AI/ML beam management into Rel-19 study item on NR FR2 OTA testing enhancement.

Proposal 4: if testing datasets is adopted, suggest to specify basic structural parameters to unify input format of inference test.

Proposal 5: Suggest to discuss whether the overfitting problem can be relieved via generalization test.

Proposal 6: Hold on the discussion on consistency until RAN1 has sufficient progress on this.

Reference

[1] R4-2404215, On AI/ML RAN4 Use Cases: Beam Management, Qualcomm