3GPP TSG-RAN WG4 Meeting #112	R4-2411260

Maastricht, NL, Aug. 19th – 23th, 2024

Source: 	Murata Manufacturing Co. Ltd

Title: 	Test points for DC_(n)8AA BCS0

Agenda item:	7.2.2

Document for:	Approval

- Introduction

This contribution proposes test points for DC_(n)8AA both 1UL and 2UL configurations.

# 2. Discussion

2.1 Background Information

In RAN#104, a new BCS0 was requested for DC_(n)8AA with a maximum 30MHz uplink bandwidth[1]. Table 1 shows DC_(n)8AA bandwidth combinations.

Table 1: Bandwidth combinations for DC_(n)8AA

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENDC configuration | Uplink ENDC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)8AA | DC_(n)8AA | 5, 10 | 5, 10, 15, 20 |  | 30 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |

Regarding NR band n8, asymmetric UL/DL channel bandwidths are introduced in 38.101-1. As per table 2, channel bandwidths highlighted in yellow can be optional. This contribution focuses on test points for symmetric configurations, but we are open to discuss other test point candidates including asymmetric configurations.

Observation 1: Regarding NR band n8, asymmetric UL/DL channel bandwidths are introduced in 38.101-1. we are open to discuss other test point candidates including asymmetric configurations.

Table 2: Asymmetric UL and DL channel bandwidth combinations for n8

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n8 | 20 | 35 | 0 |
|  | 10, 15, 20 | 25, 35 | 1 |
|  | 10, 15, 20 | 25, 30, 35 | 2 |

2.2 1UL test points

Although the request UL ENDC configurations indicates 2UL operation, single UL should also be allowed due to excessive interferences to DL CCs like CA_(n)5AA cases. Figure 1 shows the worst case for when LTE DL CC is configured closest to the NR UL CC and when the NR DL CC is configured closest to the LTE UL CC.

Figure 1: Interferences Landscape for DC_(n)8AA 1UL

Observation 2: Regarding 1UL configuration, when LTE DL CC is configured closest to the NR UL CC, both LTE and NR DL CCs have IM products. Regarding the NR DL CC, the existing REFSENS for a single NR band n8 20MHz is already takes IM products into consideration. On the hand, the current REFSENS for a single LTE band B8 does not account for such IM products. When NR DL CC is configured closest to the LTE UL CC, NR DL may have the higher Tx RxBN and negligible de-sense in the LTE DL.

Proposal 1: MSD test points for 1UL in Table 3 are introduced for DC_(n)8AA REFSENS exception.

Table 3: Test points for intra-band DC bandwidth class

| EN-DC configuration/Channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (MHz) |  |
| DC_(n)8AA | 8 | 890 | 10 | N/A | 935 | [TBD] | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | [0] |  |
| DC_(n)8AA | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | [0] |  |
|  | n8 | 895 | 20 | N/A | 940 | [TBD] |  |

2.3 2UL test points

If single UL is not allowed, 2UL test points should also be specified to evaluate MSD due to IMD interferences. To simplify the test points, two worst cases can be considered depending on the frequency orientation of the LTE/NR CCs such as when NR CC is low (NR DL closest to LTE UL) and when LTE CC is low (LTE DL closest to NR UL) as described in Figure 2.

Figure 2 IMD Landscape for DC_(n)8AA 2UL

When LTE+NR is 10+20MHz for LTE CC low, full IMD3 overlaps in LTE DL and full IMD5 falls into NR DL. When NR+LTE is 20+10MHz for NR CC low, NR DL have full overlapping IMD3 and LTE DL have full overlapping IMD5.

Proposals 2: Use test points for 2UL as shown in Table 4.

Table 4: Test points for intra-band DC bandwidth class

| EN-DC configuration/Channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (MHz) |  |
| DC_(n)8AA | 8 | 910 | 10 | 10(RBend = 49) | 955 | [TBD] | FDD |
|  | n8 | 895 | 20 | 10 (RBstart = 35) | 940 | [TBD] |  |
| DC_(n)8AA | 8 | 890 | 10 | 10 (RBstart = 40) | 935 | [TBD] |  |
|  | n8 | 905 | 20 | 10(RBend = 105) | 950 | [TBD] |  |

# Conclusion

Observation 1: Regarding NR band n8, asymmetric UL/DL channel bandwidths are introduced in 38.101-1. we are open to discuss other test point candidates including asymmetric configurations.

Observation 2: Regarding 1UL configuration, when LTE DL CC is configured closest to the NR UL CC, both LTE and NR DL CCs have IM products. Regarding the NR DL CC, the existing REFSENS for a single NR band n8 20MHz is already takes IM products into consideration. On the hand, the current REFSENS for a single LTE band B8 does not account for such IM products. When NR DL CC is configured closest to the LTE UL CC, NR DL may have the higher Tx RxBN and negligible de-sense in the LTE DL.

Proposal 1: MSD test points for 1UL in Table 3 are introduced for DC_(n)8AA REFSENS exception.

Table 3: Test points for intra-band DC bandwidth class

| EN-DC configuration/Channel allocations/MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band |  | FC (UL) |  | Channel bandwidth |  | UL |  | FC (DL) |  | MSD |  | Duplex mode |  |
|  |  |  | (MHz) |  | (MHz) |  | allocation (LCRB) |  | (MHz) |  | (MHz) |  |  |  |
| DC_(n)8AA | 8 |  | 890 |  | 10 |  | N/A |  | 935 |  | [TBD] |  | FDD |  |
|  | n8 |  | 905 |  | 20 |  | 20 (RBstart = 86) |  | 950 |  | [TBD] |  |  |  |
| DC_(n)8AA | 8 |  | 910 |  | 10 |  | 25 (RBstart = 25) |  | 955 |  | [0] |  |  |  |
|  | n8 |  | 895 |  | 20 |  | N/A |  | 940 |  | [TBD] |  |  |  |

Proposals 2: Use test points for 2UL as shown in Table 4.

Table 4: Test points for intra-band DC bandwidth class

| EN-DC configuration/Channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (MHz) |  |
| DC_(n)8AA | 8 | 910 | 10 | 10(RBend = 49) | 955 | [TBD] | FDD |
|  | n8 | 895 | 20 | 10 (RBstart = 35) | 940 | [TBD] |  |
| DC_(n)8AA | 8 | 890 | 10 | 10 (RBstart = 40) | 935 | [TBD] |  |
|  | n8 | 905 | 20 | 10(RBend = 105) | 950 | [TBD] |  |

# References:

- RP-241673, “New WID: Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (x<=6) and single or two NR Supplementary Uplink (SUL) bands”, Moderator (RAN4 Chair, Huawei), RP#104, Shanghai, China

# Appendix