3GPP TSG-RAN WG4 Meeting #112                               R4-2411261

Maastricht, Netherlands, August 19 – 23, 2024

Source: 	 SK Telecom, Murata Manufacturing Co. Ltd

Title: 		 TP for TR 37.898 to include DC_1A_n78A with 3Tx

Agenda Item: 	 7.7.2

Document for:  Approval

- Introduction

In the last RAN#104 meeting, we have discussed how to handle 3Tx in Rel-19 RAN4 baskets and made the following agreement [1]:

Agreement: - Capture 3Tx with high power classes and PC3 as fallback in HPUE_DC_LTE_NR_R19 and HPUE_NR_CADC_SUL_R19.

Based on the agreement, this contribution deals with a text proposal for TR 37.898 to include DC_1A_n78A with 3Tx, where 1Tx on Band 1 and 2Tx on Band n78.

It should be noted that we slightly modify the format of the draft TR skeleton to capture 3Tx in a way that maximum output powers for DC with UL MIMO and Tx Diversity are inserted.

- Text Proposal

--- Start of Changes ---

5.x	DC_1A_n78A

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n78A7, 23 | DC_1A_n78A21, 23 |
| NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 23: Minimum requirements for Power Class 2 are applicable for this EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |

We herein assume that Table 5.x.1-1 of this section corresponds to Table 5.5B.4.1-1 of TS 38.101-3 V18.6.0. Then, to specify 3Tx, we add NOTE 23 to this configuration while NOTE 7 and NOTE 21 are originally there.

5.x.2	Maximum output power for DC

Table 5.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |  |  |

Please note that the maximum output power for DC_1A_n78A was already specified in Table 6.2B.1.3-1 of TS 38.101-3 V18.6.0. To specify 3Tx for this configuration, we do not need to add any new information to Table 5.x.2-1 of this section.

5.x.3	Maximum output power for DC with UL MIMO

Table 5.x.3-1: Maximum output power for inter-band EN-DC with UL MIMO (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_1A_n78A, we add this configuration with NOTE 5 to Table 5.x.3-1 of this section, which corresponds to Table 6.2H.1.3-1 of TS 38.101-3 V18.6.0.

5.x.4	Maximum output power for DC with Tx Diversity

Table 5.x.4-1: Maximum output power for inter-band EN-DC with Tx Diversity (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n78A | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with Tx Diversity in NR band. NOTE 7: FWA form factor is targeted unless otherwise stated. |  |  |  |  |

To specify 3Tx for DC_1A_n78A, we add this configuration with NOTE 5 to Table 5.x.4-1 of this section, which corresponds to Table 6.2L.1.3-1 of TS 38.101-3 V18.6.0.

5.x.5	REFSENS requirements for DC

Based on the UE coexistence study of Band 1 and Band n78, we identify the following interference impact:

- The IMD4 caused by the 3rd order of Band 1 UL and the 1st order of Band n78 UL may impact on Band 1 DL.

  - For this interference, the existing MSD values of PC3 and PC2 DC_1A_n78A, which were already specified in Table 7.3B.2.3.5.1-1 and Table 7.3B.2.3.5.1-1a of TS 38.101-3 V18.6.0, respectively, can be applied to DC_1A_n78A with 3Tx.

5.x.6	∆TIB and ∆RIB values

The existing ∆TIB and ∆RIB values of DC_1_n78, which were already specified in Table 6.2B.4.2.3.1-1 and Table 7.3B.3.3.1-1 of TS 38.101-3 V18.6.0, respectively, can be applied to DC_1A-n78A with 3Tx.

--- End of Changes ---

- Reference

[1] RP-241675, Moderator Summary for Rel-19 RAN4 Baskets, 3GPP TSG-RAN Meeting #104

[2] 3GPP TS 38.101-3 V18.6.0 (2024-06), User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)