3GPP RAN WG4 Meeting #112																	R4-2411263

Maastricht, Netherlands, 19th – 23th August 2024

Agenda item:	7.19.2

Source:	Apple

Title:	Initial considerations on the band plan for the combined L-band and extended L-band

WI/SI:	NR_NTN_combinedLband

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

During Rel-18 several new NTN bands were standardised, such as NTN L-/S-band (n254) and the extended L-band (n253). And after the RAN#104 meeting a new Rel-19 WI was agreed [3], which aims at adding support for the so-called combined L-band in which the DL blocks of band n255 and n253 would be treated as one contiguous DL block. However, since the UL blocks are still not contiguous, we present our preliminary considerations on the potential band plan.

# 2	Band plan for the combined L-band

For the sake of further clarity, Table 2-1 below presents a summary of existing L-band (255), extended L-band (253) and so-called combined L-band (to be specified). As can be seen from the table, DL blocks of NTN bands 253 and 255 form a contiguous block of spectrum, which can be potentially treated as a single band. Indeed, from the network perspective it is more flexible and efficient to schedule resources over contiguous chunk of 41MHz instead of having two separate bands as 7 and 34MHz. However, while DL blocks of bands 253 and 255 are contiguous, the corresponding UL blocks have a 7.5 MHz gap between them.

Table 2-1: L-band, extended L-band and combined L-band blocks

| Acronym | NTN band | DL block (MHz) | UL block (MHz) | Size (MHz) |
| --- | --- | --- | --- | --- |
| Extended L-band | 253 | 1518-1525 | 1668-1675 | 7 |
| L-band | 255 | 1525-1559 | 1626.5-1660.5 | 34 |
| Combined L-band | TBD | 1518-1559 | 1626.5-1660.5 1668-1675 | 41 (DL) 7 and 34 (UL) |

According to the table of frequency allocations, the gap between two UL blocks, i.e. 1660.5-1668MHz, is occupied by various services, amongst which the most critical and sensitive one is passive radio astronomy. ITU-R documents RA.517 and RA.516 provide general guidelines and recommendations on how to minimise harmful interference to the radio astronomy stations in particular stating "that in order to reduce interference detrimental to the RAS, all practical, technical means, for example, the use of filters in transmitters to confine emissions to the allocated band, and in radio astronomy receivers to avoid sensitivity to signals outside the allocated band, should be adopted to the maximum practicable extent". One of the ways to ensure that emissions caused by UEs are not causing harmful interference to the neighbour blocks is to define a band with clear boundaries. From the 3GPP specification perspective it will impose the corresponding out-of-band emission requirements on the UE side, and corresponding UE front end architectures can be designed to comply with these requirements. It should be also noted that th 3GPP specifications do not have any precedent of specifying non-contiguous spectrum as a single band.  Thus, it is in our view that is impossible to define a new band as FDD band with DL and non-contiguous UL blocks.

The potential band plan for the combined L-band may look like the one presented in Table 2-2 below. There will be two NTN bands sharing the same DL contiguous spectrum block but having different UL blocks matching existing UL blocks from NTN bands 253 and 255. It is also worth noting that variable duplex distance support for the new bands xx1 and xx2 might be needed in order to allow scheduling flexibility to utilize the full range of RBs in the DL and UL blocks of each band.

Table 2-2: Exemplary band plan for the combined L-band

| Acronym | NTN band | DL block (MHz) | UL block (MHz) | Size (MHz) |
| --- | --- | --- | --- | --- |
| Combined L-band | xx1 | 1518-1559 | 1626.5-1660.5 | 41 (DL) / 34 (UL) |
|  | xx2 | 1518-1559 | 1668-1675 | 41 (DL) / 7 (UL) |

Proposal:	To support the combined L-band, introduce two NTN bands with the common DL block (at 1518-1559) and two UL blocks (at 1626.5-1660.5 and 1668-1675).

# 3	Conclusions

In this discussion paper we have presented our initial view on the band plan for the combined L-band. While DL blocks are contiguous forming a single chunk of spectrum of 41MHz, the UL blocks have a gap between them making it impossible to define it as a single FDD band, especially considering presence of the radio astronomy services in the gap. Thus, our view is that we can define two NTN bands sharing the common DL part but having different UL allocations.

Proposal:	To support the combined L-band, introduce two NTN bands with the common DL block (at 1518-1559) and two UL blocks (at 1626.5-1660.5 and 1668-1675).

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-241689, "Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz)"