3GPP TSG-RAN WG4 Meeting #112															R4-2411265

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.12.2.3

Source:	Apple

Title:	Discussion on FR1 MIMO OTA FoM definitions

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

As defined in [1] RAN4 agreed to allow the study of interference-limited environment condition as Figure of Merit (FoM) for FR1 MIMO OTA with Dynamic Channel Model, i.e.:

# 2  	Discussion

As detailed in [2-3], receiver sensitivity is typically assessed in terms of the minimum downlink power which will provide a defined level of performance at the output of the receiver. For example, a test system used to measure the reference sensitivity of a MIMO DUT will establish the minimum downlink power required to obtain a specified data throughput. In this test configuration, the DUT’s low-signal performance is typically limited by its internal noise floor. While such a measurement is useful for assessing receiver sensitivity, it is not useful for MIMO performance assessment. Only in cases where a MIMO-capable DUT experiences no co-channel interference will its spatial-multiplexing performance be limited by its internal noise. Areas in which no co-channel interference exists are rare in actual networks. In a typical network, co-channel interference (and therefore the SIR) will determine when spatial multiplexing can be supported. Consequently, it’s appropriate to assess the DUT’s spatial multiplexing performance in terms of the lowest SIR value at which a pre-defined data throughput can be obtained. The MIMO-capable DUT will be exposed to a relatively high downlink signal power from the emulated serving cell while the level of an AWGN power source (which is used to emulate co-channel interference in an actual network) is varied. The AWGN noise power used to create the desired SIR within the test zone shall be represented by an unfaded, omnidirectional signal.

Specifically considering MIMO antenna metrics, the Mean Effective Gain (MEG) and Branch Power Ratio (BPR) can demonstrate significant variation while illuminated by an anisotropic channel environment, mainly when the reference antenna radiation pattern is null-aligned with the main power beam defined in the anisotropic propagation environment. The significant variation in MEG and BPR must be recognized and mitigated while designing MIMO antenna systems, mainly when the primary environment has low SNR, e.g., in indoor, fringe, or rural areas. However, for MIMO radiated performance in rich scattered environments with high signal-to-noise ratio, e.g. urban environments, canyons, etc. The antenna MEG and BPR are secondary while the spatial correlation between antennas is more relevant to discriminating optimal MIMO antenna system designs.

Observation 1:		MIMO OTA based on noise-limited FoM evaluate devices in low S/R scenarios equivalent to rural, fringe, or similar areas with poor scattered conditions.

Observation 2:		MIMO OTA based on interference-limited FoM evaluate devices in high S/R scenarios equivalent to urban, urban canyons or similar areas with rich scattered conditions.

Considering the condition of MIMO OTA performance on Dynamic Channel Model the signal to interference power ratio per channel model segment s is computed as the ratio of time averaged power values

SIRs=1Msm=0Ms-1Sts,0+m Δt1Msm=0Ms-1Its,0+m Δt,

Where ts,0 is the first time instant of segment s, m=0,1,…,Ms-1 is the index of time instants, and Δt is the increment between measured time instants. Note that the time increment Δt and the number of stored time samples are different in this measurement as compared to other validation measurements. This post-processing is performed separately for the V-polarized and the H-polarized SIR measurements.

Lengths of time segment and time increment between segments in seconds are specified separately for both UMa and Umi models.

To pass the validation, the measured average SIR values, both for the V- and H-polarized cases, shall fit within specific limits at each time segment.

Observation 3:		Both MIMO OTA FoM, i.e.: Throughput vs. Noise and Throughput vs. SIR can be measured in the same Multi-Probe Anechoic Chamber (MPAC) with boundary array. To reduce test setup complexity and cost the interference level necessary to achieve a given SIR inside the test zone, the interferer is generated as an independent process at each antenna probe with equal power, regardless of the signal power transmitted through that probe.

Observation 4:	Both MIMO OTA test methodologies, i.e.: Throughput vs. Noise and Throughput vs. SIR, are already being implemented by most LTE MIMO OTA system integrators.

# 3	Conclusions Observations and Proposal

The discussion in section 2 and the relevant information from [2-3], provide data evidence that the MIMO OTA FoM definition can alter the perception of devices true MIMO OTA performance in the network. While is still debatable which test scenario, i.e.: urban or rural test scenario should be prioritized. Due to simple test setup changes required to produce both FoM(s), RAN4 shall gather data adopting both FoM and proper analyse results to define the proper MIMO OTA radiated performance that corresponds to network deployments.

Observation 1:		MIMO OTA based on noise-limited FoM evaluate devices in low S/R scenarios equivalent to rural, fringe, or similar areas with poor scattered conditions.

Observation 2:		MIMO OTA based on interference-limited FoM evaluate devices in high S/R scenarios equivalent to urban, urban canyons or similar areas with rich scattered conditions.

Observation 3:		Both MIMO OTA FoM, i.e.: Throughput vs. Noise and Throughput vs. SIR can be measured in the same Multi-Probe Anechoic Chamber (MPAC) with boundary array. To reduce test setup complexity and cost the interference level necessary to achieve a given SIR inside the test zone, the interferer is generated as an independent process at each antenna probe with equal power, regardless of the signal power transmitted through that probe.

Observation 4:	Both MIMO OTA test methodologies, i.e.: Throughput vs. Noise and Throughput vs. SIR, are already being implemented by most LTE MIMO OTA system integrators.

Proposal:	RAN4 shall gather data adopting both FoM(s), i.e.: Throughput vs. Noise-limited and Throughput vs. Interference-limited environmental conditions.  Thus, analysing measurement results to define the proper MIMO OTA radiated performance that corresponds to network deployment priority.

# 4	References

- R4-2409931, “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo, 3GPP TSG-RAN4 Meeting #111

- CTIA 01.40 Test Methodology, MIMO, Static Channel Model, Multi-Probe Anechoic Chamber v6.0, this is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

- I. Szini, B. Yanakiev and G. F. Pedersen, "MIMO Reference Antennas Performance in Anisotropic Channel Environments," in IEEE Transactions on Antennas and Propagation, vol. 62, no. 6, pp. 3270-3280, June 2014, doi: 10.1109/TAP.2014.2311468.