3GPP TSG RAN WG4 Meeting #112		R4-2411269

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	11

Source:	Apple

Title:	Motivation paper for band plan for satellite communication in terrestrial bands

Release:	Rel-19

Document for:	Discussion and Decision

# 1	Introduction

Commercial satellite communication is generally conducted under two radiocommunication services, namely, Mobile Satellite Service (MSS) and Fixed Satellite Service (FSS). Traditionally, FSS has been the dominant vehicle to carry out satellite communication services to fixed stations, whereas MSS has been used to communicate with handheld and portable devices. Major FSS frequency bands include Ku- and Ka-bands with a total bandwidth of in the order of several gigahertz. On the contrary, MSS operations have been using much lower frequencies, due to technical reasons including link budget, where available bandwidths have been limited. Handheld devices are usually equipped with low-gain, omnidirectional antennas.  Therefore, in order to provide a wide range of services for end users via satellite, it is important to address the spectrum challenge.

In March 2023 the Federal Communications Commission (FCC) issued a Notice of Proposed Rulemaking seeking comment on a proposed regulatory framework necessary to enable so-called Supplemental Coverage from Space (SCS) [1].  In this Notice the FCC surveyed spectrum blocks used by the terrestrial wireless service providers that could facilitate the development of satellite-based connectivity, especially in under-served areas. In March 2024, the Federal Communications Commission adopted a Report and Order (R&O) establishing a domestic regulatory framework to enable expanded satellite-based coverage to a terrestrial licensee’s subscribers [2].

In this discussion paper we present further details of the FCC R&O and outline next action points for 3GPP to analyse designated spectrum blocks and how they can be used in the final band plan.

# 2	FCC order for Supplemental Coverage from Space

## 2.1	Background

As mentioned in the Introduction part, the Federal Communications Commission adopted a Report and Order establishing a domestic regulatory framework to enable expanded satellite-based coverage to a terrestrial licensee’s subscribers, especially in remote, unserved, and underserved areas. The key points from the R&O can be summarised as follows:

-	Adopted a secondary, bi-directional, mobile-satellite service (MSS) allocation along with a new definition for SCS in certain frequency bands.

-	Adopted the proposal to enable SCS authorizations in the following defined GIAs: (1) the continental U.S.; (2) Alaska; (3) Hawaii; (4) American Samoa; (5) Puerto Rico/U.S. Virgin Islands; and (6) Guam/Northern Mariana Islands.

-	Established a framework in which the Commission would authorize SCS in bands where one or more terrestrial licensees lease access to the terrestrial spectrum rights to a satellite operator.

-	Required modified or new equipment authorizations for terrestrial devices and granted a limited waiver of certain equipment authorization rules.

-	Imposed technical rules and other recommendations to mitigate potential harmful interference to existing services, including radio astronomy.

-	Clarified international coordination obligations, including outlining steps to ensure that SCS operations will be consistent with relevant ITU Radio Regulations

On the list of spectrum blocks for the SCS operation, the FCC added a secondary MSS allocation in the following bands allocated to terrestrial services, along with a new non-federal footnote NG33A indicating that MSS operations in the bands are subject to the Commissions SCS rules:

-	600 MHz: 614-652 MHz and 663-698 MHz;

-	700 MHz: 698-769 MHz, 775 MHz-799 MHz, and 805-806 MHz;

-	800 MHz: 824-849 MHz and 869-894 MHz;

-	Broadband PCS: 1850-1915 MHz and 1930-1995 MHz;

-	AWS-H Block: 1915-1920 MHz and 1995-2000 MHz.

## 2.2	Action points for 3GPP and potential objectives for studies

While FCC established a framework for supplemental coverage from space and designated a set of spectrum blocks, in which this operation would be possible, there is no specific band plan. In fact, in line with technology neutrality principle, even the duplexing mode is not mentioned leaving it up to other bodies and organisations to find the best possible way on how this spectrum can be utilised. Thus, potential objectives for this work can be formulated as presented below:

-	Analyse frequency blocks designated by the FCC for satellite-based communication.

-	Analyse existing terrestrial 3GPP LTE and NR bands in the frequency range of spectrum blocks designated by the FCC for satellite-based communication.

-	Devise the potential 3GPP band plan for satellite-based communication in the given spectrum blocks.

# 3	Conclusions

In this discussion paper we have presented an overview of the FCC final Report and Order (R&O) on Supplemental Coverage from Space (SCS), which enables expanded satellite-based coverage to a terrestrial licensee’s subscribers. FCC has also designated a set of spectrum blocks, in which SCS operation will be possible. However, since there is no specific band plan, i.e. there are no details on which duplexing mode should be used and/or which frequency blocks should be grouped in case of paired spectrum, we ask 3GPP to initiate a new study item to study the potential band plan for satellite communication in terrestrial bands. The corresponding draft SID can be found in [3].

Proposal:	Instantiate a new study item to study band plan for satellite communication in terrestrial bands.

# 4	References.

FCC, "Single Network Future: Supplemental Coverage From Space; Space Innovation", Notice of Proposed Rulemaking, IB Docket No. 22-271, March 2023

FCC, "Single Network Future: Supplemental Coverage from Space ", REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING, IB Docket No. 22-271, March 2024, URL: https://docs.fcc.gov/public/attachments/FCC-24-28A1.pdf

RP-241501, "New SID on band plan for satellite communication in terrestrial bands", Apple Inc.