3GPP TSG-RAN4 #112	R4-2411278

Maastricht, Netherlands, 19 Aug. – 23 Aug. 2024

Title:	(NR_newRAT-Perf) Views on QPSK PDSCH demodulation test with PTRS configuration

Source:			Anritsu Corporation

Agenda Item:			4.6

Document for:			Others

# 1.	Introduction

At the last RAN4 #111 meeting in Fukuoka, one issue was raised regarding the relationship between the PTRS configuration for QPSK PDSCH demodulation test and its associated reference measurement channels [1]. The CR [2] to solve the issue was postponed and an action item was imposed on test equipment vendors to confirm an actual implementation in a test equipment.

In this contribution we discuss our views on the identified issue.

# 2.	Discussion

  - Interpretation of PTRS configuration in TS 38.214

Regarding the interpretation of RAN1 spec TS 38.214 [3] cl. 5.1.6.3, our understanding is aligned with the view in [1].

Since the MCS index of RMC for Table A.3.2.2.5-1 and Table A.3.2.2.5-4 in TS 38.101-4 [4] is 4, we agree that UE shall assume PTRS is not present.

Extract from TS 38.214 clause 5.1.6.3

- otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or - the scheduled MCS from Table 5.1.3.1-4 is smaller than 3, or - the number of scheduled RBs is smaller than 3, or

Extract from TS 38.214 clause 5.1.3.1

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |

Extract from TS 38.101-4.

Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and FR2.120-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 127 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |

Observation 1: UE shall assume PTRS is not present in a case of FR2 QPSK PDSCH demodulation test.

Based on the interpretation above, our signalling tester does not configure PTRS at the corresponding REs.

Next with regards to the transport block size (TBS) determination, in our understanding, there is no factor of PTRS within the equation to derive TBS at clause 5.1.3.2 in TS 38.214. Therefore, information bit payload size per slot in Table A.3.2.2.5-1 or A.3.2.2.5-4 in TS 38.101-4 [4] does not differ regardless of the existence of PTRS.

Observation 2: Information bit payload size per slot in Table A.3.2.2.5-1 or A.3.2.2.5-4 in TS 38.101-4 does not differ regardless of the existence of PTRS.

Based on the observations above, we agree with the contents in the previously postponed CR to TS 38.101-4 [2].

Proposal 1: RAN4 agrees with the contents of the previously postponed CR to TS 38.101-4 [2].

- Conclusion

In this contribution we shared our views on the relationship between the PTRS configuration for QPSK PDSCH demodulation test and its associated reference measurement channels.

Observation 1: UE shall assume PTRS is not present in a case of FR2 QPSK PDSCH demodulation test.

Observation 2: Information bit payload size per slot in Table A.3.2.2.5-1 or A.3.2.2.5-4 in TS 38.101-4 does not differ regardless of the existence of PTRS.

Proposal 1: RAN4 agrees with the contents of the previously postponed CR to TS 38.101-4 [2].

# 4.	References

[1] R4-2408987, “[NR_newRAT-Perf] Discussion on PTRS configuration for UE demodulation requirements”, Huawei, HiSilicon, RAN4 #111, Fukuoka, Japan

[2] R4-2409937, “[NR_newRAT-Perf] CR on PTRS configuration for UE demodulation requirements”, Huawei, HiSilicon, RAN4 #111, Fukuoka, Japan

[3] TS 38.214, “Physical layer procedures for data”, v18.3.0, 2024-06

[4] TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v18.4.0, 2024-06