3GPP TSG-RAN4 #112	R4-2411279

Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:	Study on CDL channel model for AI/ML beam management in FR2

Source:			Anritsu Corporation

Agenda Item:			8.17.2

Document for:			Approval

# 1.	Introduction

At the RAN4 #111 meeting in Fukuoka, one issue was captured in the topic summary [1] on a channel model for the verification test of the beam management performance, extracted below for reference. Though there was no outcome captured in the way forward [2], since the choice of channel model has a huge impact on a dedicated test system configuration, we started a study on the CDL channel model as one of the candidates.

Sub-topic 2-4 Channel models in tests Issue 2-4: Channel models - Proposals - Option 1: CDL - Option 2: TDL - Option 3: AWGN - Option 4: others - Recommended WF - Option 1

In this contribution we share our findings and would like to discuss on the necessary conditions to apply the CDL channel model to the beam management verification.

# 2.	Discussion

  - Study on a grouping and reduction of clusters from the existing CDL models

When we consider a channel model candidate for the AI/ML beam management verification in FR2 where one of the essential characteristics needed is that angle of arrivals to deliver DL signals from a system simulator should have multiple different directions, it appears to be natural that we choose the CDL channel model to carry the test.

However, since the existing CDL channel models defined in TR 38.901 [4] and TR 38.827 [5] are incorporating a lot of clusters as many as 24 clusters, it is not practical to apply those models without any change. Thus, we first studied a grouping and reduction of clusters from the existing CDL models.

Observation 1: Due to the total number of clusters, it is not practical to apply the existing CDL channel model directly for the beam management verification test.

We currently have multiple NLOS and LOS CDL models described in TR 38.901 and TR 38.827. So, we tried to see arrangements of clusters (in other words angle of arrivals) in each CDL model after applying the previously proposed conditions to group/ reduce clusters by the UE vendor [3]. The proposed conditions are extracted for reference.

- AOA offset threshold is set as +/-15 degrees (i.e. Clusters within +/- 15 degrees of angle of arrival are grouped.)

- power threshold is -15dB (i.e. Clusters whose power is lower than 15 dB from a cluster with the highest power are reduced.)

Please note that the number of clusters is actually not only reduced (when complying with condition 2) but also “grouped”, “grouped” should be understood as signals from multiple adjacent clusters (complying with condition 1) are grouped and emitted from one same TE probe.

All the clusters in each CDL model are plotted in the appendix A at the bottom of this contribution. Also, an example of CDL-C channel model after grouping and reduction is shown in Appendix B. As it can be seen, there are still many clusters (angle of arrivals) ranging from 5 to 10 in a case of NLOS CDL channel model, summarized in the table 2.1-1 below. In a case of LOS, clusters can be reduced down to two cluster groups.

Table 2.1-1: Number of angle of arrivals per CDL channel model after grouping/ reduction

| Spec. | Model | Num of angles of arrival | Channel profile |
| --- | --- | --- | --- |
| 38.901 | CDL-A | 8 | NLOS |
|  | CDL-B | 6 | NLOS |
|  | CDL-C | 10 | NLOS |
|  | CDL-D | 1 | LOS |
|  | CDL-E | 1 | LOS |
| 38.827 | UMi CDL-A | 5 | NLOS |
|  | UMi CDL-B | 5 | NLOS |
|  | UMi CDL-C | 7 | NLOS |
|  | UMi CDL-D | 1 | LOS |
|  | UMi CDL-E | 1 | LOS |
|  | InO CDL-A | 5 | NLOS |
|  | InO CDL-B | 7 | NLOS |
|  | InO CDL-C | 8 | NLOS |
|  | InO CDL-D | 1 | LOS |
|  | InO CDL-E | 2 | LOS |
|  | Extrapolated/imaginary 3D MPAC model | 6 | Unknown |

Observation 2: In a case of NLOS CDL channel model, there are many clusters (angle of arrivals) from 5 to 10 cluster groups (requiring from 5 to 10 TE probes) even after reducing and grouping them.

Observation 3: In a case of LOS CDL channel model, clusters can be reduced down to two cluster groups (requiring two TE probes).

When considering test system configurations which cover all those angles of arrivals, we found multiple concerns. We would like to explain them one by one from the next clause.

2.2 Test system complexity

As shown in the table 2.1-1 above, the total number of clusters (angle of arrivals) are ranging from 5 to 10. Considering our current test equipment with its limited number of antenna ports, there are the following concerns.

- multiple system simulators, fading simulators with millimetre wave converters per angle of arrival are required to support all clusters

- the system would not be practical from system complexity, costs and a power consumption point of views in a laboratory.

- it should also be remembered that each test probe per cluster (angle of arrival) needs to have two polarizations.

Observation 4: The number of clusters must be minimized as much as possible to avoid an overly complex test system with its associated drawbacks.

Observation 5: It must be noted that two polarizations are needed for each test probe per cluster.

2.3 Arrangement of probes in a chamber

Next, we discuss an arrangement of probes in a chamber. Figure 2.3-1 and Figure 2.3-2 depict examples of angle of arrivals for CDL-C in TR 38.901 and InO CDL-A in TR 38.827. As can be seen in those figures, clusters are arranged in a wide range of azimuth angles from -180 to 180 degrees, which may increase difficulty in an actual probe arrangement in an FR2 OTA chamber. Also, a footprint of the OTA chamber may become huge if we need to arrange probes over the whole x-y plane and its proximity while fulfilling the far-field criteria.

Observation 6: Clusters are arranged in a wide range of azimuth angles from -180 to 180 degrees, which may increase the difficulty for arranging the actual probes in a FR2 OTA chamber.

Observation 7: A footprint of the OTA chamber may become huge if we need to arrange probes over the whole x-y plane and its proximity while fulfilling the far-field criteria.

Figure 2.3-1: CDL-C (TR 38.901)

Figure 2.3-2: InO CDL-A (TR 38.827)

Additionally, the probe arrangement needs to avoid having probes facing each other. For example, in Figure 2.3-1 the cluster around +90, -100 degrees with zenith and azimuth angles and the one around +90, +70 degrees with zenith and azimuth angles are almost facing each other. In that case, there are concerns of cross-interference, degradation of quality of quiet zone (QoQZ).

Even when the clusters (probes) are not arranged on the horizontal plane (90 degrees of zenith angle plane), as for example the clusters shown in Figure 2.3-2 around +110, -80 degrees with zenith and azimuth angles and the one at +70, +100 degrees with zenith and azimuth angles are facing each other.

Observation 8: Care must be taken not to place probes facing each other to avoid interference and degradation of QoQZ.

There is also a concern of increase of number of probes in the chamber to support all CDL channel models. As can be seen in the plots of cluster arrangement, they are located at various places depending on channel models. A possible idea would be to add a structure to shift each probe mechanically to cover some close locations and reduce maximum number of probes in the chamber. However, by shifting probes, there is an expectation that bending RF cables may cause cable-loss changes especially in FR2. To avoid any concern of error of output power calibration in FR2, we expect that it becomes mandatory that probes are fixed in the chamber, and we cannot allow a procedure to change probe locations to adjust for each channel model during tests.

Observation 9: The total number of probes may increase to support all the CDL channel models.

Observation 10: It is expected that a procedure to change probe locations cannot be allowed due to concern of output power calibration errors in FR2.

Observation 11: To minimize the number of probes, further study and clarification are necessary with the following main questions to be answered.

- How many and which CDL channel models need to be supported for the beam management test?

- How many clusters are necessary in each channel model?

- How much antenna offset is necessary to avoid the interference and degradation of QoQZ?

- What conditions can we apply to achieve minimum number of clusters per channel model?

Observation 12: Grouping by assigning antenna probes to different clusters having AOA offset below the threshold set as +/-15 degrees, even though preservation of original delays and Doppler frequencies, AoAs reproduction in conformance testing will be impacted particularly if many CDL models need to be supported each having many clusters and different AoAs. A restricted number of CDL models developed for conformance testing while realistic would be ideal.

Proposal 1: RAN4 should decide what are the CDL channel models necessary for testing and limit its number as much as possible. This will help determining what is possible from a test point of view.

Proposal 2: RAN4 should model new CDL channel model parameter sets taking into account of the conformance testing, and group/ reduce number of clusters and angle of arrivals.

- Conclusion

In this contribution we shared our findings and discussed on the necessary conditions to apply the CDL channel models to the beam management verification.

Observation 1: Due to the total number of clusters, it is not practical to apply the existing CDL channel model directly for the beam management verification test.

Observation 2: In a case of NLOS CDL channel model, there are many clusters (angle of arrivals) from 5 to 10 cluster groups (requiring from 5 to 10 TE probes) even after reducing and grouping them.

Observation 3: In a case of LOS CDL channel model, clusters can be reduced down to two cluster groups (requiring two TE probes).

Observation 4: The number of clusters must be minimized as much as possible to avoid an overly complex test system with its associated drawbacks.

Observation 5: It must be noted that two polarizations are needed for each test probe per cluster.

Observation 6: Clusters are arranged in a wide range of azimuth angles from -180 to 180 degrees, which may increase the difficulty for arranging the actual probes in an FR2 OTA chamber.

Observation 7: A footprint of the OTA chamber may become huge if we need to arrange probes over the whole x-y plane and its proximity while fulfilling the far-field criteria.

Observation 8: Care must be taken not to place probes facing each other to avoid interference and degradation of QoQZ.

Observation 9: The total number of probes may increase to support all the CDL channel models.

Observation 10: It is expected that a procedure to change probe locations cannot be allowed due to concern of output power calibration errors in FR2.

Observation 11: To minimize the number of probes, further study and clarification are necessary with the following main questions to be answered.

- How many and which CDL channel models need to be supported for the beam management test?

- How many clusters are necessary in each channel model?

- How much antenna offset is necessary to avoid the interference and degradation of QoQZ?

- What conditions can we apply to achieve minimum number of clusters per channel model?

Observation 12: Grouping by assigning antenna probes to different clusters having AOA offset below the threshold set as +/-15 degrees, even though preservation of original delays and Doppler frequencies, AoAs reproduction in conformance testing will be impacted particularly if many CDL models need to be supported each having many clusters and different AoAs. A restricted number of CDL models developed for conformance testing while realistic would be ideal.

Proposal 1: RAN4 should decide what are the CDL channel models necessary for testing and limit its number as much as possible. This will help determining what is possible from a test point of view.

Proposal 2: RAN4 should model new CDL channel model parameter sets taking into account of the conformance testing, and group/ reduce number of clusters and angle of arrivals.

# 4.	References

[1] R4-2408944, “Topic summary for [111][133] NR_AIML_air”, Moderator (Qualcomm), RAN4 #111, Fukuoka, Japan

[2] R4-2410570, “WF on AI/ML”, Moderator (Qualcomm), RAN4 #111, Fukuoka, Japan

[3] R4-2407846, “Discussion on testability and interoperability issues for beam management”, Xiaomi, RAN4 #111, Fukuoka, Japan

[4] TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz", v18.0.0, 2024-03

[5] TR 38.827, "Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)", v16.8.0, 2022-09

# Appendix A:  Cluster arrangements after applying grouping/ reduction conditions

Clusters are plotted based on definitions in TR 38.901 and TR 38.827 with following conditions which were previously proposed in [3].

- AOA offset threshold is set as +/-15 degrees (i.e. Clusters within +/- 15 degrees of angle of arrival are grouped.)

- power threshold is -15dB (i.e. Clusters whose power is lower than 15 dB from a cluster with the highest power are reduced.)

Blue dots represent clusters (angle of arrivals), and red circles are expressing areas within +/- 15 degrees. Thus, the total number of red circles are the number of necessary angle of arrivals in an OTA test chamber. Note that zenith angle from 0 to 180 degrees at the vertical axis is shown upside down compared to the actual definition of the coordinate system. (i.e. 0 degrees should be the top in the actual coordinate system in the chamber.)

Figure A-1: CDL-A (TR 38.901)						Figure A-2: CDL-B (TR 38.901)

Figure A-3: CDL-C (TR 38.901)						Figure A-4: CDL-D (TR 38.901)

Figure A-5: CDL-E (TR 38.901)						Figure A-6: UMi CDL-A (TR 38.827)

Figure A-7: UMi CDL-B (TR 38.827)						Figure A-8: UMi CDL-C (TR 38.827)

Figure A-9: UMi CDL-D (TR 38.827)						Figure A-10: UMi CDL-E (TR 38.827)

Figure A-11: InO CDL-A (TR 38.827)						Figure A-12: InO CDL-B (TR 38.827)

Figure A-13: InO CDL-C (TR 38.827)						Figure A-14: InO CDL-D (TR 38.827)

Figure A-15: InO CDL-E (TR 38.827)						Figure A-16: 3D MPAC (TR 38.827)

# Appendix B:  Channel model example after grouping/ reduction of clusters

Table B-1 is shown for the example of CDL-C channel model after grouping/ reduction of clusters.

The application of the proposed conditions 1 and 2 from [3] are highlighted in the Table  B-1 respectively in yellow and green.

- AOA offset threshold is set as +/-15 degrees (i.e. Clusters within +/- 15 degrees of angle of arrival are grouped.).

- power threshold is -15dB (i.e. Clusters whose power is lower than 15 dB from a cluster with the highest power are reduced.)

| Table B-1: CDL-C channel model after grouping / reduction of clusters from the table in TR 38.901 [4]Cluster # | Normalized delay | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] | Valid AoA | Valid ZoA | Probe # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4 | -46.6 | -101 | 97.2 | 87.6 | -101 | 87.6 | 1 |
| 2 | 0.2099 | -1.2 | -22.8 | 120 | 98.6 | 72.1 | 120 | 72.1 | 2 |
| 3 | 0.2219 | -3.5 | -22.8 | 120 | 98.6 | 72.1 | 120 | 72.1 |  |
| 4 | 0.2329 | -5.2 | -22.8 | 120 | 98.6 | 72.1 | 120 | 72.1 |  |
| 5 | 0.2176 | -2.5 | -40.7 | -127.5 | 100.6 | 70.1 | -127.5 | 70.1 | 3 |
| 6 | 0.6366 | 0 | 0.3 | 170.4 | 99.2 | 75.3 | 170.4 | 75.3 | 4 |
| 7 | 0.6448 | -2.2 | 0.3 | 170.4 | 99.2 | 75.3 | 170.4 | 75.3 |  |
| 8 | 0.656 | -3.9 | 0.3 | 170.4 | 99.2 | 75.3 | 170.4 | 75.3 |  |
| 9 | 0.6584 | -7.4 | 73.1 | 55.4 | 105.2 | 67.4 | 55.4 | 67.4 | 5 |
| 10 | 0.7935 | -7.1 | -64.5 | 66.5 | 95.3 | 63.8 | 66.5 | 63.8 |  |
| 11 | 0.8213 | -10.7 | 80.2 | -48.1 | 106.1 | 71.4 | -48.1 | 71.4 | 6 |
| 12 | 0.9336 | -11.1 | -97.1 | 46.9 | 93.5 | 60.5 | 46.9 | 60.5 | 5 |
| 13 | 1.2285 | -5.1 | -55.3 | 68.1 | 103.7 | 90.6 | 68.1 | 90.6 | 7 |
| 14 | 1.3083 | -6.8 | -64.3 | -68.7 | 104.2 | 60.1 | -68.7 | 60.1 | 6 |
| 15 | 2.1704 | -8.7 | -78.5 | 81.5 | 93 | 61 | 81.5 | 61 | 8 |
| 16 | 2.7105 | -13.2 | 102.7 | 30.7 | 104.2 | 100.7 | 30.7 | 100.7 | 9 |
| 17 | 4.2589 | -13.9 | 99.2 | -16.4 | 94.9 | 62.3 | -16.4 | 62.3 | 10 |
| 18 | 4.6003 | -13.9 | 88.8 | 3.8 | 93.1 | 66.7 | 3.8 | 66.7 |  |
| 19 | 5.4902 | -15.8 | -101.9 | -13.7 | 92.2 | 52.9 | #N/A | #N/A | #N/A |
| 20 | 5.6077 | -17.1 | 92.2 | 9.7 | 106.7 | 61.8 | #N/A | #N/A | #N/A |
| 21 | 6.3065 | -16 | 93.3 | 5.6 | 93 | 51.9 | #N/A | #N/A | #N/A |
| 22 | 6.6374 | -15.7 | 106.6 | 0.7 | 92.9 | 61.7 | #N/A | #N/A | #N/A |
| 23 | 7.0427 | -21.6 | 119.5 | -21.9 | 105.2 | 58 | #N/A | #N/A | #N/A |
| 24 | 8.6523 | -22.8 | -123.8 | 33.6 | 107.8 | 57 | #N/A | #N/A | #N/A |