3GPP TSG-RAN WG4 Meeting #112	R4-2411289

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	KDDI, LG Uplus, NTT DOCOMO INC., SoftBank

Title: 	Discussion on the number of CCs for non-collocated EN-DC,NR-CA Type 4a4b

Agenda Item:	8.5.2.3

Document for:	Discussion

- 1. Introduction

In RAN#103 meeting, a new WI for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE. Some discussions were organized in the last RAN4#111 meeting and the agreements were captured in [2]. We try to solve remaining issues on the number of CCs for Type 4a and 4b in this contribution.

- 2. Discussion

2.1 Background

Firstly, we share the operators’ motivation of non-collocated scenarios here again based on [3].
Regarding co-existence with other systems, different co-existence conditions apply for C-band. As a result, Tx antenna collocation network side is not always available. Regarding timing of spectrum allocation, the three blocks in C-band were allocated at different time in Japan, and also, the two blocks were allocated at different time in Korea (Figure-1). This makes Tx antenna collocation cost-inefficient or sometimes infeasible.

Figure-1. Spectrum point of view in Japan and Korea

2.1 Working assumption

According to WID [1], RAN4 need to specify Type-4a/4b capability for FWA. Based on the working assumption previously agreed as WF in RAN4#106 [4], in RAN#103 we propose the following working assumption in Table-1 same as [3].

Table-1. Working assumption for UE architecture

Observation 1: The working assumption of RP-240101 can be reused in RAN4.

2.2 The number of CCs for Type 4a/4b

Regarding the number of CCs for Type 4a/4b, RAN4 agreed the following WF[2] in the last meeting.

RAN4#111) < Issue 2-3-1: The number of B42 CCs for EN-DC > Way Forward: - Check the number with operators until next meeting again. < Issue 2-3-2: The number of NR CCs for EN-DC and NR-CA > Way Forward: - Check the number with operators until next meeting again.

On the other hand, RAN4 also agreed the following WF[7] in the RAN4#110-bis meeting.

RAN4#110-bis) < Issue 2-2-5: The number of CCs > Way Forward: - Consider the conclusion of Type 2 discussion in RAN4#110bis. - Continue further discussion with the following baselines in the next meeting. - Non-collocated EN-DC - B42: multiple contiguous CCs up to four, collocated - n77/n78: one CC - Non-collocated NR-CA - n77/n78: Non-contiguous two CCs, non-collocated - If needed, check operators’ preference before the next May meeting.

And then, regarding the number of CCs for Type 2 EN-DC, RAN4 had already agreed the following WF[8] in the RAN4#110-bis meeting. Thus, the following conclusion for Type 2 can be also adopted to Type 4a/4b.

- RAN4#110-bis) Way Forward Capture the following note to TS 38.101-3 Table 7.10B.3-1 to make the case of multiple contiguous LTE CCs possible: “NOTE 6: For Inter-band EN-DC configurations with multiple contiguous E-UTRA CCs in one band, REFSENS in this table equals to 5MHz REFSENS+10*log(aggregated BW(MHz)/5) of all the contiguous E-UTRA CCs of the wanted band. BWwanted and BWanother represent the aggregated BWs of all the CCs of the wanted and another wanted band, respectively. The maximum power spectral density imbalance between the contiguous E-UTRA CCs in one band, is within 6 dB.” Remark: Only co-located contiguous LTE CCs is considered in Note 6.

Observation 2: WF R4-2406628 for Type 2 can be reused for Type 4a/4b.

Finally, we propose to reuse and merge above WFs with simplifications as follows.

Proposal 1: Prioritize the following scenario of the number of CCs for Type 4a/4b.

- Non-collocated EN-DC

- B42: multiple contiguous CCs, all collocated

  - Based on the requested band combinations (see R4-2406628)

- n77/n78: one CC

- Non-collocated NR-CA

- n77/n78: Non-contiguous two CCs, non-collocated

Proposal 2: If TU still remains in Rel-19, discuss other scenario(s) of the number of CCs and contiguous cases later.

- 3. Conclusion

Observation 1: The working assumption of RP-240101 can be reused in RAN4.

Observation 2: WF R4-2406628 for Type 2 can be reused for Type 4a/4b.

Proposal 1: The number of CCs for Type 4a/4b.

- Non-collocated EN-DC

- B42: multiple contiguous CCs, all collocated

  - Based on the requested band combinations (see R4-2406628)

- n77/n78: one CC

- Non-collocated NR-CA

- n77/n78: Non-contiguous two CCs, non-collocated

Proposal 2: If TU still remains in Rel-19, discuss other scenario(s) of the number of CCs and contiguous cases later.

- 4. Reference

[1] RP-230838 New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple, RAN#103

[2] R4-2410686 WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN#111

[3] RP-240101 Motivation on Non-collocated ENDC,NRCA in Release 19, KDDI, Docomo, KT, LG Uplus, SoftBank, Telecom Italia, Samsung, RAN#103

[4] R4-2303696 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106

[5] R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung, RAN4#104-bis-e

[6] R4-2306605 WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106-bis-e

[7] R4-2406627 WF on NonCol_intraB_ENDC_NR_CA, KDDI, RAN4#110-bis

[8] R4-2406628 WF on clarification of RF requirements in case of contiguous LTE CCs in non-collocated scenario, Huawei, RAN4#110-bis