# 3GPP TSG-RAN WG4 Meeting #112	R4-2411292

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.17.2

Source: 	OPPO

Title: 	Testability and interoperability issues  for beam management

Document for:	Approval

- Introduction

In 3GPP RAN #102[1], for A/ML based beam management, it is agreed to provide specification support for the core requirements, especially for AI/ML LCM procedures e.g., for performance monitoring. Besides, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models are also agreed in the AI/ML WID, including: specify necessary performance requirements and tests (including metrics), and specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. ### 4.2 Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution, we will continue to discuss the testability and interoperability issue on AI/ML based beam management with the focus on KPI and performance requirement, as well as the potential test issues on BM.

- Discussion

  - KPI and performance requirement

Beam management(BM)-Case1 is agreed in RAN1, i.e. Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams. As example is shown in figure 1, the best beam(s) can be estimated through AI/ML based BM methods with L1 measurements on a beam subset.

Figure 1.  Spatial-domain DL beam prediction

Another sub use case is beam management(BM)-Case2, i.e. Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams. As example is shown in figure 2, The best beam in the future can be predicted through AI/ML based BM methods based on the beam quality at the current and historical time.

Figure 2. Temporal DL beam prediction

Regarding the model/functionality input, measurements on a sub set of beams or historical beams are utilized.

Regarding the model/functionality output and performance monitoring, intermediate KPIs (e.g. beam prediction accuracy, L1-RSRP difference of the predicted beam) are used in RAN1 for performance evaluation, and system performance related KPIs(e.g. throughput) can also be considered and agreed in RAN1.

In RAN4 R19 tests, for both BM-Case1 and BM-Case2,

- Regarding the input related test, performance requirements on BM model/functionality input (e.g. beam measurement accuracy for set B) should be considered

- Regarding the output related test, three options have been agreed in RAN4 R18, i.e.,

Option 1: RSRP accuracy

Option 2: Beam prediction accuracy

Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among 		top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

Here, for option1, different companies still have different understandings regarding the definition of RSRP accuracy . In RAN4 #110bis meeting, it is agreed to further provide inputs/proposals to refine the definition of RSRP accuracy.

According to R18 TR 38.843, “Average L1-RSRP difference of Top-1 predicted beam” is utilized as the beam prediction accuracy related KPI in RAN1, and defined as “the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam”.

Also in R18 TR 38.843, we could find another related definition that “for AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam”.

In the last meeting, RAN4 agreed “the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam”, and “the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam”.

According to above information, in RAN4 for the definition of RSRP accuracy, we may still have 3 different options for further clarification, i.e.,

|  | The RSRP difference between: |  |  |
| --- | --- | --- | --- |
| Option 1 | the ideal(or measured) L1-RSRP of Top-1 predicted beam | and | the ideal(or measured) L1-RSRP of the Top-1 ideal beam |
| Option 2 | the predicted L1-RSRP of Top-1 predicted beam |  | the ideal(or measured) L1-RSRP of the Top-1 ideal beam |
| Option 3 | the predicted L1-RSRP of Top-1 predicted beam |  | the ideal(or measured) L1-RSRP of the Top-1 predicted beam |

Proposal 1: To refine the definition of RSRP accuracy, following options should be further clarified,

- Option 1: the RSRP difference between the ideal(or measured) L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 2: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 3: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam

Besides, in RAN1#116 meeting, 4 options regarding the content in the BM report of inference results have been agreed as well, as listed below,

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Based on these agreements, metrics for BM model output tests also depends on the RAN1 discussion, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy may be the only available KPI accordingly (note: also depends on the refined definition of RSRP accuracy)

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs

- Regarding the performance monitoring

- UE can monitor and estimate the performance of AI/ML based BM model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the discussion and conclusions in RAN1 as well.

- Besides, stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

Proposal 2: Regarding the model output tests, the selection of metrics depends on the progress in RAN1,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, beam prediction accuracy may be the only available KPI accordingly (also depends on the refined definition of RSRP accuracy in RAN4),

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, both beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs.

Proposal 3: UE can monitor the performance of AI/ML based CSI model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the progress in RAN1.

Proposal 4: Stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

  - Testability issues on BM

Regarding the testability issues on BM, from our understanding, following aspects need to be considered.

- The number of FR2 beams that RAN4 tests can support remains unclear. It is necessary to clarify the limitations regarding the FR2 beams or probes that TE vendors could support. Based on this limitation(if any), RAN4 can further consider how to reach a consensus on designing/conducting tests for BM use cases and how to set/form beam set A and B of different sizes for different BM cases.

- A potential test setup could be (as shown in Figure 3)

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

- In the last meeting, RAN4#111 agreed “maximum number of set B Tx beams that test system should be able to emulate: [8-16]” and “maximum number of set A Tx beams that test system should be able to emulate:  [64-128]”

- In R18 TR 38.843, BS Antenna Configuration and UE Antenna Configuration are defined as below,

| Parameter | Value |
| --- | --- |
| BS Antenna Configuration | Number of BS beams: 32 or 64 downlink Tx beams (max number of available beams) at NW side. Other values, e.g., 256 not precluded. |
| UE Antenna Configuration | Number of UE beams: 4 or 8 downlink Rx beams (max number of available beams) per UE panel at UE side. Other values, e.g., 16 not precluded. |

- According to above agreements in both RAN4 and R18 TR, we can choose 8 beam for set B and 64 						beam for set A in RAN4 tests.

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Figure 3 A potential test setup for BM tests

- Besides, it's also crucial to further consider that, as a data/scenario-driven solution, the chamber and beam patterns provided by different TEs may impact the test results of BM (e.g., due to variations in beam settings, different considerations on multipath, and other implementations). Besides, even for a same TE, different DUT hardware implementations may also lead to variations in receiving behavior and beam measurement results. Attention must be paid to ensure that the BM model constructed on the DUT side(e.g. by corresponding training dataset) can match(or approximate match) and be utilized in the chamber environment on the TE side. Otherwise, it becomes challenging for a DUT to pass the test under unknown chamber conditions/scenarios. This may require a close collaboration between DUT and TE to ensure that the model under test is compatible with the testing environment.

Proposal 5: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 6:

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

Proposal 7: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based BM and got following proposals:

Proposal 1: To refine the definition of RSRP accuracy, following options should be further clarified,

- Option 1: the RSRP difference between the ideal(or measured) L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 2: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 3: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam

Proposal 2: Regarding the model output tests, the selection of metrics depends on the progress in RAN1,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, beam prediction accuracy may be the only available KPI accordingly (also depends on the refined definition of RSRP accuracy in RAN4),

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, both beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs.

Proposal 3: UE can monitor the performance of AI/ML based CSI model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the progress in RAN1.

Proposal 4: Stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

Proposal 5: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 6:

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

Proposal 7: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface