3GPP TSG RAN WG4 Meeting #112	R4-2411295

Maastricht, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	8.4.3

Source: 	Ericsson

Title:	Discussion on RRM requirements for NR CA/DC in less than 5 MHz

Document for:	Discussion

# Introduction

3GPP agreed to specify NR CA/DC in less than 5 MHz channel bandwidth. WID objectives can be found in [1], copied below for reference.

### 4.1 Objective of SI or Core part WI or Testing part WI - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified. ### 4.2 Objective of Performance part WI - Specify necessary performance requirements (RAN4)

Since this is the first meeting for this WI in RAN4, we look at the RRM requirements that may be impacted due to introduction of NR CA/DC in less than 5 MHz channel bandwidth.

# Discussion

In Rel-18 NR, RAN4 UE minimum RRM requirements are defined to support channel bandwidths (CBWs) less than 5 MHz. From the WID [1], we can observe that the RRM task is to specify the RRM core and performance requirements to introduce NR CA/DC support in less than 5 MHz CBWs.

The WID clearly states that the UE should support the co-located and non-co-located inter-band of CA/DC. Intra-band is out of scope of the WI. As contiguous and non-contiguous NR CA/DC are supported in NR, RAN4 needs to discuss whether we support them for less than 5 MHz.

RAN4 to define whether the UE supports inter-band contiguous and non-contiguous NR CA/DC for less than 5MHz.

The WID does not clearly state whether EN-DC, NE-DC, or NR-DC will be supported. As less than 5 MHz is supported only for NR, we proposed to limit the work scope to NR-DC. No new UE type was introduced in Rel-18 to support operations in less than 5MHz. That means the CA/DC can coexist with other band combinations.

RAN4 to support NR-DC for less than 5MHz in Rel-19.

Furthermore, according to 38.133, the RRM requirements for NR CA/DC are generally organized into the following high-level categories.

- SCell activation and deactivation requirements

- PSCell/ Conditional PSCell addition/change

- SCG activation/deactivation

In the sections below, we analyze the impact of less than 5MHz CBW on each RRM requirement.

## Impact on CA/DC related procedures

NR CA/DC are specified in chapter 8 of TS 38.133, and we look at the requirements needed to support UE with less than 5MHz capability.

### SCell activation and deactivation requirements

In Rel-18, for CBWs less than 5 MHz, it is specified that due to the puncturing, the UE takes more time to read the SSB. To understand the impact of reduced bandwidth on the SCell activation requirement in a similar context, let's examine the SCell activation time requirements specified in clause 8.3.2 of TS 38.133.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

In the above requirements, the SCell activation delay may depend on the CBW for the above time requirement parameters marked in yellow. Hence, details of the SCell activation delay should be further studied in future meetings.

SCell activation requirements shall be further investigated for potential impact.

### PSCell addition/change

In TS 38.133, the requirements for NR dual connectivity (DC) specify the delay within which the UE should be able to configure a PSCell. To understand the impact of reduced bandwidth on the addition or change of the PSCell, let's examine the PSCell addition delay requirements as outlined in clause 8.9.2.

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms when target cell is in FR1 and Tprocessing = 40 ms when target cell is in FR2. Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3*N* Trs ms. N = 1 when target cell is in FR1, N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2 T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In Rel-18, for less than 5 MHz, it is specified that due to shorted BW UE takes more time for fine time tracking and acquiring full timing information of the target cell. We introduced new requirements for T∆ in HO requirement. In the above requirements, PSCell activation for NR-DC may impacted by the shortened BW. Details can be further studied in future meetings.

PSCell activation requirements shall be investigated for potential impact.

Moreover, conditional PSCell addition delay may be impacted for the same reason. Hence, conditional PSCell activation requirements shall be further investigated for potential impact.

Conditional PSCell activation requirements shall be investigated for potential impact.

### SCG activation/deactivation

Similar to the PSCell activation delay requirement specification, shorted CBWs may also impact SCG activation time. Below are the requirements copied from clause 8.17 of TS 38.133.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length , where: Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms. Tsearch is the time for AGC settling and PSS/SSS detection. For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms. For RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known, Tsearch = 0 ms. There are no requirements if TCI state is unknown. T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms. TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

The shortened BW may impact the marked parameter T∆. Hence, a detailed study should be conducted for future meetings.

SCG activation requirements shall be investigated for potential impact.

### Handover Requirements

The purpose of the NR handover is to change the NR PCell to another NR cell. In Rel-18 NR handover for less than 5MHz is specified. Hence, introducing CA/DC for less than 5MHz would not impact the handover requirements.

NR CA/DC for less than 5MHz would not impact the handover requirements.

### Handover with PSCell from NR-DC to NR-DC

The text copied from TS 38.133 shows that the Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. The HO with PSCell-PCell and PSCell-PSCell can be reused without any major changes in the spec.

##### 6.1.5.4.1 HO with PSCell – PCell Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. - Tprocessing is the SW processing time needed by UE, including RF warm up period. For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms. For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

NR CA/DC for less than 5MHz would not impact the HO requirements with PSCell-PCell and PSCell-PSCell.

### NR Conditional Handover Requirements

Like NR handover, NR conditional handover (CHO) also aims to change the NR PCell to another NR cell. In Rel-18, we considered Rel-15 as a baseline. Therefore, the CA/DC requirements introduced in Rel-16 are not considered in the less than 5MHz WI for Rel-18. CHO was also not considered in Rel-18. In Rel-19, RAN 4 should consider and define CHO requirements.

RAN4 should define conditional handover requirements for less than 5 MHz

### Timing Requirements

In Rel-18, no timing issue was identified for less than 5MHz CBWs. Due to the same reason, we don’t foresee any impact on timing requirements for UE supports CA/DC for less than 5MHz CBWs. Legacy requirements can be reused with no apparent impact.

No seen impact to timing requirements with CA/DC support for less than 5MHz CBWs.

### Impact on Interruptions

In TS 38.133, interruptions are specified to accommodate RF retuning and carrier switching in multiple procedures. For CBWs less than 5 MHz, the requirement was relaxed due to the SSB index reading. Hence, unless an SSB index reading needs to be done in the measurement gap, it typically does not cause interruptions.

Regarding the CGI reading requirement, interruptions are due to the MIB decoding and SIB decoding. Therefore, if the CGI reading is specified for less than 5 MHz, RAN4 may need to consider the interruptions caused by the CGI reading.

RAN4 to discuss and decide whether CGI reading is considered for less than 5MHz.

### Impact on early measurement reporting (EMR)

In Rel-18, time index detection requirements are specified for CBWs less than 5MHz. Additional SSB index reading time may impact cell detection requirements. Table 4.4.2.2-1 should be checked for any impact on EMR due to shortened BW.

RAN4 to Study the Cell detection requirements in EMR measurements for less than 5MHz.

### Idle Mode CA/DC Measurements

Idle mode RSRP is measured on SSS. Puncturing in less than 5MHz does not impact SSS. Hence, there should be no impact on measurement delay or accuracy. Legacy requirements can be reused with no apparent impact.

Legacy requirements can be reused for Idle mode CA/DC measurements with no apparent impact.

## Other issues

### Potential impact on other RAN WG (RAN1/RAN2).

According to R1-2405566, the defined Feature Groups (FG) in Rel-18 are only applicable for single-carrier operation to support less than 3MHz channel bandwidth in DL and UL. Thus, depending on the agreements reached on the support CA/DC in less than 5MHz CBW, other working groups (WGs) will need to be informed for them to identify any specification impact or update on the UE Feature list to reflect the support of this feature.

Other working groups (WGs) will need to be informed so they can identify any specification impact or update on the UE Feature list to reflect the support of NR CA/DC in less than 5MHz CBW.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	NR CA/DC for less than 5MHz would not impact the handover requirements.

Observation 2	NR CA/DC for less than 5MHz would not impact the HO requirements with PSCell-PCell and PSCell-PSCell.

Observation 3	No seen impact to timing requirements with CA/DC support for less than 5MHz CBWs.

Observation 4	Other working groups (WGs) will need to be informed so they can identify any specification impact or update on the UE Feature list to reflect the support of NR CA/DC in less than 5MHz CBW.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 to define whether the UE supports inter-band contiguous and non-contiguous NR CA/DC for less than 5MHz.

Proposal 2	RAN4 to support NR-DC for less than 5MHz in Rel-19.

Proposal 3	SCell activation requirements shall be further investigated for potential impact.

Proposal 4	PSCell activation requirements shall be investigated for potential impact.

Proposal 5	Conditional PSCell activation requirements shall be investigated for potential impact.

Proposal 6	SCG activation requirements shall be investigated for potential impact.

Proposal 7	RAN4 should define conditional handover requirements for less than 5 MHz

Proposal 8	RAN4 to discuss and decide whether CGI reading is considered for less than 5MHz.

Proposal 9	RAN4 to Study the Cell detection requirements in EMR measurements for less than 5MHz.

Proposal 10	Legacy requirements can be reused for Idle mode CA/DC measurements with no apparent impact.

# References

- RP- 241622 “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel

- R1-2405566, LS on Rel-18 RAN1 UE features list for NR after RAN1#117