3GPP TSG RAN WG4 #112															     R4-2411296

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:		8.24.2

Source:				InterDigital Finland Oy

Title:		On UE assistance information for gap skipping occasions - NR XR

Document for:		Discussion and decision

# Background

During RAN1 Meeting #117, an LS to RAN4 [1] related to the UE assistance information on skipping RRM measurement occasions for NR XR has been approved.

As there was no consensus in RAN1 to support this feature, due to possible impact on RRM performance from skipping measurement occasions, the decision to support it or not was delegated to RAN4.

# Discussion

In this contribution, we share our preliminary impact analysis on UE assistance information related to measurements occasions for NR XR case and make several recommendations. For convenience we are listing below the LS text [1]:

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information. --------------------------------------------------------------------------------------------------------------------------------------- Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. To RAN4 ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

# NR gap types and their applicability

In TS 38.133 [2] the gap types are listed in the sub-clause 9.1.2 in Table 9.1.2-1 as follows:

Table 1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

The applicability of the gap patterns is defined based on deployment cases (e.g. SA, EN-DC, CA), UE type and UE capabilities. From both the NW and UE perspectives, it can be quite a complex operation for determining whether and which of the occasions in the configured gap patterns can be skipped. For skipping measurements occasions, careful determination/selection of specific supported cases (gap pattern id, deployment case) would be required as a preliminary step. For example, positioning related measurements gaps may not be skipped due to law enforcement requirements. Another case may be related to certain gap repetition periods that may be too large for skipping occasions. These would clearly affect the accuracy of the measurements.

Observation 1: Careful determination/selection of specific supported cases (gap pattern id, deployment case) would be required as a preliminary step.

Proposal 1: RAN4 to perform a clear selection of the applicable deployment cases when considering skipping of measurement occasions.

From the perspective of NR XR case, we performed preliminary evaluations to identify the impact of configuring different types of gaps patterns (for measurements) when performing DL transmissions of XR traffic. Such impact is evaluated in terms of the amount of overlap in time domain when performing DL transmissions in a single carrier of different types of XR traffic, including augmented reality (AR), virtual reality (VR) and cloud gaming (CG) [3]. The evaluated impact of the gap patterns is provided below:

Table 2: Impact of the gap patterns on XR traffic

| Gap Pattern Configurations |  | AR traffic | VR traffic | CG traffic |
| --- | --- | --- | --- | --- |
|  |  | Rate: 45Mbps, PDB: 10ms, Periodicity: 16.66ms, Jitter: [-4,4]ms | Rate: 30Mbps, PDB: 10ms, Periodicity: 16.66ms, Jitter: [-4,4]ms | Rate: 8Mbps, PDB: 15ms, Periodicity: 16.66ms, Jitter: [-4,4]ms |
| MG-0 | MGL: 6ms, MGRP: 40ms | 9.45% - 10.45% | 9.25% - 9.85% | 7.15% - 7.35% |
| MG-1 | MGL: 6ms, MGRP: 80ms | 4.6% - 5.1% | 4.6% - 4.9% | 3.6% - 3.65% |
| MG-2 | MGL: 3ms, MGRP: 40ms | 5.85% - 6.55% | 5.85% - 6.4% | 3.75% - 3.8% |
| MG-3 | MGL: 3ms, MGRP: 80ms | 2.85% - 3.3% | 2.85% - 3.15% | 1.85% - 1.9% |
| MG-4 | MGL: 6ms, MGRP: 20ms | 18.8% - 20.2% | 18.5% - 19.5% | 14.35% - 14.5% |
| MG-5 | MGL: 6ms, MGRP: 160ms | 2.35% - 2.5% | 2.25% - 2.4% | 1.75% - 1.85% |
| MG-6 | MGL: 4ms, MGRP: 20ms | 14% - 15.45% | 14.35% - 15.35% | 10.15% - 10.5% |
| MG-7 | MGL: 4ms, MGRP: 40ms | 7% - 7.7% | 7.15% - 7.75% | 5.1% - 5.2% |
| MG-8 | MGL: 4ms, MGRP: 80ms | 3.45% - 3.85% | 3.5% - 3.85% | 2.5% - 2.6% |
| MG-9 | MGL: 4ms, MGRP: 160ms | 1.7% - 1.85% | 1.65% - 1.85% | 1.15% - 1.2% |
| MG-10 | MGL: 3ms, MGRP: 20ms | 11.65% - 13.05% | 11.6% - 12.6% | 7.4% - 7.5% |
| MG-11 | MGL: 3ms, MGRP: 160ms | 1.3% - 1.55% | 1.35% - 1.5% | 0.85% - 0.95% |
| MG-12 | MGL: 5.5ms, MGRP: 20ms | 17% - 18.45% | 17.4% - 18.4% | 13.25% - 13.75% |
| MG-13 | MGL: 5.5ms, MGRP: 40ms | 8.55% - 9.3% | 8.75% - 9.35% | 6.65% - 7.25% |
| MG-14 | MGL: 5.5ms, MGRP: 80ms | 4.2% - 4.65% | 4.35% - 4.65% | 3.35% - 3.55% |
| MG-15 | MGL: 5.5ms, MGRP: 160ms | 2.05% - 2.2% | 2.1% - 2.25% | 1.6% - 1.675% |
| MG-16 | MGL: 3.5ms, MGRP: 20ms | 13.1% - 14.5% | 13.25% - 14.25% | 9.1% - 9.4% |
| MG-17 | MGL: 3.5ms, MGRP: 40ms | 6.55% - 7.3% | 6.65% - 7.25% | 4.6% - 4.85% |
| MG-18 | MGL: 3.5ms, MGRP: 80ms | 3.2% - 3.65% | 3.25% - 3.6% | 2.25% - 2.3% |
| MG-19 | MGL: 3.5ms, MGRP: 160ms | 1.65% - 1.75% | 1.6% - 1.75% | 1.05% - 1.15% |
| MG-20 | MGL: 1.5ms, MGRP: 20ms | 7.2% - 8.65% | 7.75% - 8.75% | 3.6% - 3.7% |
| MG-21 | MGL: 1.5ms, MGRP: 40ms | 3.55% - 4.3% | 3.85% - 4.4% | 1.75% - 1.85% |
| MG-22 | MGL: 1.5ms, MGRP: 80ms | 1.7% - 2.15% | 1.8% - 2.15% | 0.85% - 0.9% |
| MG-23 | MGL: 1.5ms, MGRP: 160ms | 0.85% - 1% | 0.85% - 1% | 0.35% - 0.355% |
| MG-24 | MGL: 10ms, MGRP: 80ms | 7.35% - 7.8% | 7.4% - 7.75% | 6.4% - 6.5% |
| MG-25 | MGL: 20ms, MGRP: 160ms | 6.7% - 6.85% | 6.65% - 6.85% | 6.15% - 6.2% |

From the evaluation results above, several gap patterns (e.g. gap pattern IDs 0, 4, 6, 10, 12, 16) result in considerable impact on XR traffic. When configured with such gap patterns for measurements, scheduling and data transmissions are unavailable up to 20% of the time. The gap patterns that have high impact on XR traffic are typically the ones with relatively long MGL duration (e.g. > 6ms) and/or short MGRP (e.g. 20ms).

On the other hand, even when skipping measurement occasions in certain gap patterns (e.g. pattern 24, 25), the improvement achievable for XR is not significant (e.g. 6.2 – 7.8%) but comes at the expense of loss in measurement amount/accuracy. It is up to the network to decide on suitable gap patterns to configure at UE, based on the knowledge of traffic and deployment scenarios.

# discussion on Measurement occasions skipping

The proposed measurement skipping behavior in favor of maintaining XR QoS by providing UE assistance information on a gap/measurement that can be skipped deserves a discussion on the motivation for enabling of such a feature.

Usually, the UE is configured with measurement gaps for mobility purposes. These configured gaps may be triggered by:

- Current cell/beam has a poor quality, or its quality is degrading under a certain threshold.

- The network intends to move the UE for congestion or offloading reasons.

In the first case, when the cell quality is degrading, skipping gaps would not be helpful as possible HARQ retransmissions may be envisioned in a poor-quality cell, or lower MCS would be granted, leading to a degraded XR QoS anyways.

In the second case, when the cell is good, but the UE is asked to perform measurements, we believe that is better to follow the network orders, as the UE has no idea about the cell load or any other network load balancing actions to follow. In such case, it is unclear what additional info regarding skipping gap/measurement that the UE can provide that the network does not already know.

Another factor, in our opinion would be UE speed, that may impact the handovers and measurement rate to perform in specific deployment cases. In such cases, it is imperative for the UE to not skip any gaps/measurements to avoid potential impacts on XR QoS.

Observation 2: Under poor cell conditions the XR QoS may degrade anyway through HARQ retransmissions or lower MCS grants. Also, the UE has no knowledge of the network load balancing or offloading intensions/strategy.

Proposal 2: Any further RAN4 impact analysis shall account for XR QoS under measurement gap skipping occasions versus measurement accuracy degradation.

From a different angle, the UE assistance information can be sent to the network only after the measurements are configured or triggered. This can be a dynamic process, due to real time factors such as XR traffic, buffer status, type of measurements to be performed, UE speed, HARQ success rate etc. In our opinion this process involves a latency because the UE must inform the network about the skipping occasions, so the scheduler has enough knowledge to turn this into an advantage. This signaling requires an acknowledgement from the network. This latency may delay triggering events, for example, mobility events.

Another important issue to consider is the fact that sending this UE assisting information cannot guarantee network scheduling. The UE has no knowledge about the cell load or any other scheduler pre-condition or strategy. In this case, regardless of the UE assistance info on skipping gaps/measurements, the network scheduler may factor in other inputs unknown to the UE when making scheduling decisions. As such, the UE assistance info on skipping gaps/measurements may not translate to XR QoS improvement.

Observation 3: UE assistance information signaling to the network may add delays to event triggering conditions, thus impacting mobility. Also, UE assistance information on skipping gaps/measurements may not translate to XR QoS improvement, given other impacting info/factors known only to network

Proposal 3: Any further RAN4 impact analysis shall account for UE assistance information signaling latency impacts to mobility/triggered events.

# Conclusions

This contribution discussed our preliminary impact analysis on UE assistance information related to measurements occasions for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: Careful determination/selection of specific supported cases (gap pattern id, deployment case) would be required as a preliminary step.

Proposal 1: RAN4 to perform a clear selection of the applicable deployment cases when considering skipping of measurement occasions.

Observation 2: Under poor cell conditions the XR QoS may degrade anyway through HARQ retransmissions or lower MCS grants. Also, the UE has no knowledge of the network load balancing or offloading intensions/strategy.

Proposal 2: Any further RAN4 impact analysis shall account for XR QoS under measurement gap skipping occasions versus measurement accuracy degradation.

Observation 3: UE assistance information signaling to the network may add delays to event triggering conditions, thus impacting mobility. Also, UE assistance information on skipping gaps/measurements may not translate to XR QoS improvement, given other impacting info/factors known only to network

Proposal 3: Any further RAN4 impact analysis shall account for UE assistance information signaling latency impacts to mobility/triggered events.

# References

- R1-2405736, “LS on UE assistance information”, RAN1 Meeting #117, Fukuoka, Japan, May 2024

- TS 38.133, Requirements for support of radio resource management, Rel-18

- TR 38.838, Study on XR (Extended Reality) evaluations for NR, v17.0.0, Apr 2022