112973GPP TSG-RAN WG4 Meeting #112	R4-2411297

Maastricht, Netherlands, 19 Aug 2024 - 23 Aug 2024

Source:	Charter Communications, Cable Labs

Title:	SBFD as a band specific feature and SBFD restrictions for bands with low channel bandwidth configurations

Agenda item:	8.19.2.1

Document for:	approval

Introduction

In TSG-RAN WG4 Meeting #111, a way forward [1] was approved with the following agreement:

- SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations:

  - Option 1: Declaration based method.

    - The band supported for SBFD shall be manufacturer declaration based.

    - The channel bandwidth supported for SBFD shall be manufacturer declaration based.

      - The supported channel bandwidth can be impacted by the sub band/guard band size discussion.

  - Option 2: General principle but with consideration(s) for a specific band

    - General principle: The channel bandwidth shall be larger than X MHz

      - FFS the value of X.

        - X can be different for high and middle TDD bands

      - For a specific band, the following further restriction(s) can be considered:

        - Certain restriction can be provided by operator(s) for certain band.

        - Other restrictions are not precluded.

In this contribution, we will discuss the reasons why we believe SBFD should be a band specific feature and why SBFD should have restriction for bands with low channel bandwidths.

Discussion

2.1 SBFD as a band specific feature

The following diagram shows that in North America the CBRS band is in between its adjacent bands (AMBIT and C-Band)

Figure 1

Observation 1: In Figure 1, we show that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

Observation 2: In Figure 1, we show both the AMBIT band and the CBRS band are configured as 10 MHz channel bandwidth bands.

In the approved way forward [1], the agreement showed two options (declaration-based method and general principle but with consideration(s) for a specific band.  We believe that by making SBFD feature a band specific feature, 3GPP RAN4 would be able to better define specific restrictions in a per band basis.

Proposal 1:   Define the SBFD feature as a band specific feature and have Operators drive the requirement of SBFD in a per band basis.

2.2 SBFD restrictions for bands with low channel bandwidth configurations

In the approved way forward [1], we also had an agreement showing in option 2 that, in general principle, the channel bandwidth should be larger than X, where the value of X is FFS.

We have also discussed, but not reached an agreement, that SBFD for bands with channel bandwidth of 10 MHz and below is effective given that you also need some guard bands for SBFD reducing the channel bandwidth to less than 10 MHz.

Proposal 2:   We propose the value of X for channel bandwidth shall be greater than10 MHz channel bandwidth.

Proposal 3: Restrict SBFD to operate in bands with channel bandwidths of X ≤10 MHz.

Conclusion

The following Observations and Proposals were made:

Observation 1:   there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

Observation 2:   Both the AMBIT band and the CBRS band are configured as 10 MHz channel bandwidth bands.

Proposal 1:   Define the SBFD feature as a band specific feature and have Operators drive the requirement of SBFD in a per band basis.

Proposal 2:   We propose the value of X for channel bandwidth shall be greater than10 MHz channel bandwidth.

Proposal 3: Restrict SBFD to operate in bands with channel bandwidths of X≤10 MHz.

- References

- R4-2409958 Way Forward for [110bis][313] NR duplex evo, Samsung