3GPP TSG-RAN WG4 Meeting#112							         			R4-2411302

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On equal PSD vs. equal power spectral regrowth

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.1.1	Intra-band contiguous and non-contiguous UL CA with PC1.5	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#111, a way forward [2] captured agreements related to the Release 19 power domain enhancement work item. Notably, it covered the PC1.5 intra-band ULCA feature for both contiguous and non-contiguous cases and specifically the aspects related to PCmax limitations for different implementations and power sharing between CCs. In this contribution, further developing from our contribution [1] in RAN4#111, we address the power limitations and the MPR validity under equal PSD and equal power sharing.

# Discussion

## 2.1 Way forward inputs on power sharing

In [3], the plenary input sets the objectives that are related to PC1.5 intra-band ULCA which are still aligned with way forward [2]:

Way forward on PCmax: FFS in future meetings

Proposals:

  - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE)

- For dualPA-architecture

  - PCMAX,C limitation  for each component carrier is 26dBm

- For TxD (dualTx)

  - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3

  - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3

  - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721)

In this contribution, we further develop our input on power limitation which was not captured as an input to the way forward.

## 2.2 Simulation of spectrum regrowth for equal PSD vs equal Power

In order to show the issue related to power limitations in the Dual PA case, but also to check if MPR as currently defined in 38.101-1 can handle a large deviation from equal PSD, a number of equal PSD and equal power cases were simulated using an AM/AM and AM/PM model of a PA. Equal PSD was used for the evaluation of the required backoff for all intra-band ULCA cases (PC3 in R16, PC2 in R17), except for ENDC in R15, where power sharing was assumed and then SCC may be dropped.

## 2.2.1 Cases without RB BW allocation imbalance

When the allocated RB bandwidth is the same in each CC, there is no difference between equal PSD and equal power cases. The following cases were simulated:

- DFT-s-OFDM 216RB+216RB at minimum in Figure 1 (left)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation

- DFT-s-OFDM 108RB+108RB at ~79MHz gap in Figure 1 (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation (largest separation)

=> or 40MHz+40MHz non-contiguous intra-band ULCA (as one fully allocated case example)

- DFT-s-OFDM 15RB+15RB at ~146MHz gap in Figure 2 (left)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation (largest separation)

=> or 40MHz+40MHz non-contiguous intra-band ULCA (as one partially allocated case example)

- DFT-s-OFDM 1RB+1RB at ~156MHz gap in Figure 2 (left)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation (largest separation)

=> or 40MHz+40MHz non-contiguous intra-band ULCA (as one partially allocated case example)

However, it should be noted that, in the field, there may be some PSD imbalance due to separate power control commands for each CC and it may be needed to further check these cases with some reasonable PSD imbalance.

Figure 1: 216RB+216RB_miGap (left) 108RB+108RB_79MHzGap (right)

Observation:

- 216RB+216RB_miGap (left): ACLR close to 30dB based on IM3 and IM5 regrowth, IM5 not meeting -30dBm/MHz at 165MHz offset.

- 108RB+108RB_79MHzGap (right):

  - If contiguous CA: ACLR marginal, IM3 meeting -13dBm/MHz, IM5 failing -30dBm/MHz (8dB)

  - If non-contiguous CA: ACLR OK, IM3/IM5 failing -30dBm/MHz (18dB/8dB).

Figure 2: 15RB+15RB_146MHzGap (left) 1RB+1RB_156MHzGap (right)

Observation:

- 15RB+15RB_146MHzGap (left):

  - If contiguous CA: ACLR failed, IM3 failing -13dBm/MHz (10dB), IM5 failing -30dBm/MHz (15dB)

  - If non-contiguous CA: ACLR OK, IM3/IM5 failing -30dBm/MHz (27dB/15dB)

  - -30dBm/MHz failed up to IM9 included. Note that there is IMD wrap around above IM5.

- 1RB+1RB_146MHzGap (right):

  - If contiguous CA: ACLR failed, IM3 failing -13dBm/MHz (19dB), IM5 failing -30dBm/MHz (25dB)

  - If non-contiguous CA: ACLR OK, IM3/IM5 failing -30dBm/MHz (36dB/25dB)

  - -30dBm/MHz failed up to IM13 included. Note that there is IMD wrap around above IM5.

Overall observation: in most cases, the emission limitation is related to the SEM.

## 2.2.1 Contiguous CC and allocations cases with RB BW allocation imbalance

When the allocated RB bandwidth is different in each CC, there can be significant differences between equal PSD and equal power cases. The following contiguous CC and allocation cases were simulated with equal PSD and equal power:

- DFT-s-OFDM 216RB+108RB at minimum gap in Figure 3 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation full+partial

=> 80MHz+40MHz contiguous intra-band ULCA with contiguous allocation full+full

- DFT-s-OFDM 216RB+15RB at minimum gap in Figure 4 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation full+partial

=> 80MHz+10MHz contiguous intra-band ULCA with contiguous allocation full+partial

- DFT-s-OFDM 216RB+1RB at minimum gap in Figure 5 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation full+partial

=> 80MHz+5MHz contiguous intra-band ULCA with contiguous allocation full+partial

- DFT-s-OFDM 108RB+15RB at minimum gap in Figure 6 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation partial+partial

=> 40MHz+10MHz contiguous intra-band ULCA with contiguous allocation full+partial

- DFT-s-OFDM 108RB+1RB at minimum gap in Figure 57 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with contiguous allocation partial+partial

=> 40MHz+5MHz contiguous intra-band ULCA with contiguous allocation full+partial

Figure 3: 216RB+108RB_minGap with equal PSD (left) equal power (right)

Observation 216RB+108RB_minGap, if 80MHz+40MHz CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 12dBm/MHz

  - IM3 > -13dBm/MHz up to ~50MHz offset

  - IM5 > -30dBm/MHz up to ~180MHz offset.

- Equal power (right):

  - CC1 PSD ~ 10dBm/MHz (1.7dB SNR loss), CC2 PSD ~ 13dBm/MHz (1.2dB SNR gain)

  - IM3 > -13dBm/MHz up to ~50MHz offset

  - IM5 > -30dBm/MHz up to ~200MHz offset.

Figure 4: 216RB+15RB_minGap with equal PSD (left) equal power (right)

Observation 216RB+15RB_minGap, if 80MHz+10MHz CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 13dBm/MHz

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~30MHz offset

  - IM5L > -30dBm/MHz up to ~150MHz offset, IM5H > -30dBm/MHz up to ~140MHz offset.

- Equal power (right):

  - CC1 PSD ~ 10dBm/MHz (2.7dB SNR loss), CC2 PSD ~ 22dBm/MHz (8.8dB SNR gain)

  - IM3L > -13dBm/MHz up to ~0MHz offset, IM3H > -13dBm/MHz up to ~70MHz offset

  - IM5L > -30dBm/MHz up to ~150MHz offset, IM5H > -30dBm/MHz up to ~180MHz offset.

Figure 5: 216RB+1RB_minGap with equal PSD (left) equal power (right)

Observation 216RB+1RB_minGap, if 80MHz+5MHz CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 13dBm/MHz

  - IM3L > -13dBm/MHz up to ~20MHz offset, IM3H > -13dBm/MHz up to ~20MHz offset

  - IM5L > -30dBm/MHz up to ~140MHz offset, IM5H > -30dBm/MHz up to ~125MHz offset.

- Equal power (right):

  - CC1 PSD ~ 10dBm/MHz (3dB SNR loss), CC2 PSD ~ 28dBm/MHz (20.3dB SNR gain)

  - IM3L > -13dBm/MHz up to ~25MHz offset, IM3H > -13dBm/MHz up to ~75MHz offset

  - IM5L > -30dBm/MHz up to ~140MHz offset, IM5H > -30dBm/MHz up to ~175MHz offset.

Figure 6: 108RB+15RB_minGap with equal PSD (left) equal power (right)

Observation 108RB+15RB_minGap, if 40MHz+10MHz CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 16dBm/MHz

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~20MHz offset

  - IM5L > -30dBm/MHz up to ~80MHz offset, IM5H > -30dBm/MHz up to ~80MHz offset.

- Equal power (right):

  - CC1 PSD ~ 13dBm/MHz (2.5dB SNR loss), CC2 PSD ~ 22dBm/MHz (6.1dB SNR gain)

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~35MHz offset

  - IM5L > -30dBm/MHz up to ~90MHz offset, IM5H > -30dBm/MHz up to ~100MHz offset.

Figure 7: 108RB+1RB_minGap with equal PSD (left) equal power (right)

Observation 108RB+1RB_minGap, if 40MHz+5MHz CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 17dBm/MHz

  - IM3L > -13dBm/MHz up to ~20MHz offset, IM3H > -13dBm/MHz up to ~15MHz offset

  - IM5L > -30dBm/MHz up to ~80MHz offset, IM5H > -30dBm/MHz up to ~75MHz offset.

- Equal power (right):

  - CC1 PSD ~ 14dBm/MHz (3dB SNR loss), CC2 PSD ~ 29dBm/MHz (17.4dB SNR gain)

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~35MHz offset

  - IM5L > -30dBm/MHz up to ~80MHz offset, IM5H > -30dBm/MHz up to ~95MHz offset.

Overall observation: in all cases:

- The more loaded CC loses SNR (up to 3dB) in equal power mode.

- The IM3 and IM5 ranges are extended in the equal power case, requiring larger MPR than the equal PSD case.

- To recover the larger spectral regrowth of the equal power, additional back-off is needed resulting in further reduced SNR for the more loaded CC.

2.2.1 Non-contiguous allocations cases with RB BW allocation imbalance

When the allocated RB bandwidth is different in each CC, there can be significant differences between equal PSD and equal power cases. The following non-contiguous allocation cases that are valid for both contiguous and non-contiguous CC were simulated with equal PSD and equal power:

- DFT-s-OFDM 216RB+108RB at ~40MHz gap in Figure 8 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation full+partial

=> 80MHz+40MHz+40MHz gap non-contiguous intra-band ULCA full+full

- DFT-s-OFDM 216RB+15RB at ~74MHz gap in Figure 9 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation full+partial

=> 80MHz+10MHz+70MHz gap non-contiguous intra-band ULCA full+partial

- DFT-s-OFDM 216RB+1RB at ~79MHz gap in Figure 10 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation full+partial

=> 80MHz+5MHz+75MHz gap non-contiguous intra-band ULCA full+partial

- DFT-s-OFDM 108RB+15RB at ~113MHz gap in Figure 11 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation partial+partial

=> 40+10MHz+110MHz gap non-contiguous intra-band ULCA full+partial

- DFT-s-OFDM 108RB+1RB at ~118MHz gap in Figure 12 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation partial+partial

=> 40MHz+5MHz+115MHz gap non-contiguous intra-band ULCA full+partial

- DFT-s-OFDM 15RB+1RB at ~151MHz gap in Figure 13 equal PSD (left), equal power (right)

=> 80MHz+80MHz contiguous intra-band ULCA with non-contiguous allocation partial+partial

=> 10MHz+5MHz+135MHz gap non-contiguous intra-band ULCA partial+partial

Figure 8: 216RB+108RB_40MHzGap with equal PSD (left) equal power (right)

Observation 216RB+108RB_40MHz Gap, if 80MHz+40MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 12dBm/MHz

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~20MHz offset

  - IM5L > -30dBm/MHz up to ~280MHz offset, IM5H > -30dBm/MHz up to ~250MHz offset.

- Equal power (right):

  - CC1 PSD ~ 14dBm/MHz (1.2dB SNR loss), CC2 PSD ~ 29dBm/MHz (1.8dB SNR gain)

  - IM3L > -13dBm/MHz up to ~0MHz offset, IM3H > -13dBm/MHz up to ~25MHz offset

  - IM5L > -30dBm/MHz up to ~280MHz offset, IM5H > -30dBm/MHz up to ~290MHz offset.

Figure 9: 216RB+15RB_74MHzGap with equal PSD (left) equal power (right)

Observation 216RB+15RB_74MHz Gap, if 80MHz+10MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 13dBm/MHz

  - IM3L > -13dBm/MHz up to ~30MHz offset, IM3H > -13dBm/MHz up to ~0MHz offset

  - IM5L > -30dBm/MHz up to ~220MHz offset, IM5H > -30dBm/MHz up to ~150MHz offset.

- Equal power (right):

  - CC1 PSD ~ 10dBm/MHz (2.7dB SNR loss), CC2 PSD ~ 22dBm/MHz (8.9dB SNR gain)

  - IM3L > -13dBm/MHz up to ~0MHz offset, IM3H > -13dBm/MHz up to ~40MHz offset (marginal up to 150MHz offset)

  - IM5L > -30dBm/MHz up to ~300MHz offset, IM5H > -30dBm/MHz up to ~320MHz offset.

Figure 10: 216RB+1RB_79MHzGap with equal PSD (left) equal power (right)

Observation 216RB+1RB_79MHz Gap, if 80MHz+5MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 13dBm/MHz

  - IM3L > -13dBm/MHz up to ~25MHz offset, IM3H > -13dBm/MHz up to ~0MHz offset

  - IM5L > -30dBm/MHz up to ~150MHz offset, IM5H > -30dBm/MHz up to ~70MHz offset.

- Equal power (right):

  - CC1 PSD ~ 10dBm/MHz (3dB SNR loss), CC2 PSD ~ 28dBm/MHz (20.4dB SNR gain)

  - IM3L > -13dBm/MHz up to ~0MHz offset, IM3H > -13dBm/MHz up to ~35MHz offset (marginal up to 150MHz offset)

  - IM5L > -30dBm/MHz up to ~300MHz offset, IM5H > -30dBm/MHz up to ~360MHz offset.

Figure 11: 108RB+15RB_113MHzGap with equal PSD (left) equal power (right)

Observation 108RB+15RB_113MHz Gap, if 40MHz+10MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 16dBm/MHz

  - IM3L > -13dBm/MHz up to ~15MHz offset, IM3H > -13dBm/MHz up to ~0MHz offset

  - IM5L > -30dBm/MHz up to ~300MHz offset, IM3H > -30dBm/MHz up to ~190MHz offset.

- Equal power (right):

  - CC1 PSD ~ 13dBm/MHz (2.4dB SNR loss), CC2 PSD ~ 22dBm/MHz (6.1dB SNR gain)

  - IM3L > -13dBm/MHz up to ~140MHz offset, IM3H > -13dBm/MHz up to ~150MHz offset (marginal up to 150MHz offset)

  - IM7L > -30dBm/MHz up to ~460MHz offset, IM7H > -30dBm/MHz up to ~460MHz offset.

Figure 12: 108RB+1RB_118MHzGap with equal PSD (left) equal power (right)

Observation 108RB+1RB_118MHz Gap, if 40MHz+5MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 17dBm/MHz

  - IM3L > -13dBm/MHz up to ~150MHz offset, IM3H > -13dBm/MHz up to ~0MHz offset (marginal up to 150MHz offset)

  - IM5L > -30dBm/MHz up to ~310MHz offset, IM5H > -30dBm/MHz up to ~310MHz offset.

- Equal power (right):

  - CC1 PSD ~ 13dBm/MHz (3dB SNR loss), CC2 PSD ~ 28dBm/MHz (17.4dB SNR gain)

  - IM3L > -13dBm/MHz up to ~150MHz offset, IM3H > -13dBm/MHz up to ~150MHz offset (IM5L and IM5H marginal up to 300MHz offset)

  - IM7L > -30dBm/MHz up to ~440MHz offset, IM7H > -30dBm/MHz up to ~450MHz offset.

Figure 13: 15RB+1RB_151MHzGap with equal PSD (left) equal power (right)

Observation 15RB+1RB_151MHz Gap, if 40MHz+5MHz NC CA:

- Equal PSD (left):

  - CC1 PSD = CC2 PSD ~ 25dBm/MHz

  - IM3L > -13dBm/MHz up to ~150MHz offset, IM3H > -13dBm/MHz up to ~0MHz offset (marginal up to 150MHz)

  - IM5L > -30dBm/MHz up to ~310MHz offset, IM5H > -30dBm/MHz up to ~310MHz offset.

- Equal power (right):

  - CC1 PSD ~ 22dBm/MHz (3dB SNR loss), CC2 PSD ~ 28dBm/MHz (9dB SNR gain)

  - IM3L > -13dBm/MHz up to ~140MHz offset, IM3H > -13dBm/MHz up to ~150MHz offset (IM5L and IM5H marginal up to 310MHz offset)

  - IM7L > -30dBm/MHz up to ~490MHz offset, IM7H > -30dBm/MHz up to ~480MHz offset.

Overall observation: in all cases:

- The more loaded CC loses SNR (up to 3dB) in equal power mode.

- The IM3 and IM5 ranges are significantly extended in the equal power case; sometimes IM5 is close to -13dBm/MHz and -30dBm/MHz is not reached before IM7 in some cases, requiring larger MPR than the equal PSD case.

- To recover the larger spectral regrowth of the equal power, additional back-off is needed resulting in further reduced SNR for the more loaded CC.

## 2.3 MPR validity under PSD imbalance and power limitation for DualPA

As shown in the multiple examples above, when departing from the equal PSD condition that is used for the MPR evaluation, there is both an increase and an extension of the spectral regrowth. In some extreme cases, meeting the -30dBm/MHz spurious emission is extended to IMD orders beyond IMD5.

The issue acuteness increases in the following order:

- Moderate issue for contiguous intra-band CA with contiguous allocation

- Larger issue for contiguous intra-band CA with non-contiguous allocation (increased IM3 and IM5 spectral regrowth)

- Worse for non-contiguous ULCA where spectral regrowth becomes a concern up to IM7.

- DualPA should behave only slightly better than 2Tx due to slightly lower reverse IMD (further improved for FWA) versus forward IMD. Note that for ET implementations, reverse IMD is often worse than forward IMD as it is not part of the PA control. Also, for PC1.5, since each PA is limited to 26dBm there is no intrinsic linearity gain compared to the PC2 case where each CC can reach the max power.

We understand that equal PSD is not guaranteed in operation due to independent schedulers (different MCS) and small difference in fading (especially for non-contiguous ULCA with bandwidth separation class up to 600MHz). However, we do not see fundamental reasons to allow large PSD imbalances (>20dB) in UL; while for collocated scenarios, the DL PSD imbalance is limited to 6dB.

Furthermore, when PC1.5 is implemented with Dual PA (1PA per CC), each CC is limited to 26dB and letting each CC reach 26dBm when there is large RB BW imbalance generating large PSD imbalances, which in turn increases spectral regrowth further, thus requires higher backoff that further reduces the SNR at the BS for the more loaded CC. All this is counter productive and thus large PSD imbalances should be avoided and PCmax per CC or total or global MPR account for the power limitations of the dualPA architecture.

Proposal for MPR studies:

- MPR and NS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on 2Tx architecture and should account for reasonable PSD imbalance (<6dB?)

- PC1.5 contiguous intra-band ULCA based on dualPA architecture is not specified in R19 as it does not allow UL MIMO and has power limitations such that it rarely delivers better performance than PC2 based on the same two 26dBm PAs.

- MPR studies for PC1.5 non-contiguous intra-band ULCA focuses on dualPA architecture as it avoids limitations in total BW and gap size, should account for reasonable PSD imbalance (<6dB?), and the power limitation to 26dBm per CC should be captured in the PCmax or MPR equations and account for RB BW imbalance.

- PC1.5 contiguous intra-band ULCA based on 2Tx architecture is not specified in R19 as it does not allow support for the example n77(2A) configurations.

- FFS what behavior is allowed for the UE when the PSD imbalance get too high to guarantee emissions (both related to MPR and A-MPR).

# Conclusion

In this contribution, we examined the intra-band contiguous and non-contiguous ULCA interference levels depending on power sharing in the two CCs (equal power or equal PSD) and formulate the following proposal.

Proposal for MPR studies:

- MPR and NS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on 2Tx architecture and should account for reasonable PSD imbalance (<6dB?)

- PC1.5 contiguous intra-band ULCA based on dualPA architecture is not specified in R19 as it does not allow UL MIMO and has power limitations such that it rarely delivers better performance than PC2 based on the same two 26dBm PAs.

- MPR studies for PC1.5 non-contiguous intra-band ULCA focuses on dualPA architecture as it avoids limitations in total BW and gap size, should account for reasonable PSD imbalance (<6dB?), and the power limitation to 26dBm per CC should be captured in the PCmax or MPR equations and account for RB BW imbalance.

- PC1.5 contiguous intra-band ULCA based on 2Tx architecture is not specified in R19 as it does not allow support for the example n77(2A) configurations.

- FFS what behavior is allowed for the UE when the PSD imbalance get too high to guarantee emissions (both related to MPR and A-MPR).

# References

[1] R4-2407371 PC1p5 intra-band ULCA MOP for 2Tx and Dual-PA cases, Skyworks Solutions Inc., RAN4#111

[2] R4-2410750 WF on high power UE, Samsung, RAN4#111

[3] RP‑241656 UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon, RAN#104