3GPP TSG RAN WG4 Meeting #112		R4-2411303

# Maastricht, Netherlands, 19th – 23th August 2024

Source:	EchoStar, DISH Network, TerreStar, Thales, Gatehouse, Novamint

Title:	NTN MSS S-band band plan, regulations, and deployment scenarios

Agenda item:			7.17.1

Document for:	Discussion and decision

# 1. Introduction

In the RAN#104, the WIs to define NTN S-band (2000-2020MHz UL, 2180 – 2200 MHz DL) for NR-NTN and IoT-NTN were approved. [1, 2]

This paper first clarifies the band plan, applicable the regulations to derive RF requirements and the typical deployment scenario between TN and NTN to derive UE Coexistence requirements. The papers further propose to relax UE to UE Coexistence requirement according to the TN – NTN typical deployment scenario.

# 2. Band plan and main issues

Based on the WIDs, the following band plan (range and number) in Table 1 for NR-NTN and in Table 2 for IoT-NTN can be introduced. Taking into account the extended L-band work for NR-NTN, the band number for this NTN S-band could be specified as [n252] for NR-NTN and [B252] for IoT-NTN.
A tentative band number assignment is beneficial to avoid misunderstanding since a lot of time this range is refer to as B23 (an LTE TN band which is not applicable any longer in the current specification).

Table 1: NR-NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high |  | Duplex mode |
| --- | --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz |  | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |  | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz |  | FDD |
| n253 | Reserved for extended L band in NR |  |  |  |
| n252 | 2000 – 2020 MHz |  | 2180 – 2200 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |  |

Table 2: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

Proposal 1: Band number [n252] for NR-NTN and [B252] for IoT-NTN are defined for the new NTN S-band.

As part of S-band, the newly proposed [n252/B252] has similar/same characteristics with n256/B256. Furthermore, based on WIDs, only power class PC3 for NR-NTN and PC3 and PC5 for IoT-NTN are proposed. Therefore, it is expected that the general requirements can be re-used and most of the band specific requirements for n256/B256 can also be re-used. Further consideration for band specific requirements will be discussed as necessary.

We understand that the following are main aspects that needs to be discussed. During RAN#104, there was a discussion on TN DL band protection (UE to UE coexistence) requirements that needs to be addressed taking into account the typical deployment scenario of TN-NTN. Furthermore, since the range of the proposed NTN S-band is the same as that of LTE B23 range, applicable regulatory requirements need to be clarified. Detailed observations and understanding on these aspects are discussed in following section 3 and 4.

Table 3 lists up our initial understanding of the RF requirements that needs to be defined, indicating which ones can reuse existing requirements and which ones need further discussion.

Table 3: Required changes compare to current spec TS 38.101-5 [3] 
(Note: applicable to respective sections of TS 36.102)

| Section | Requirement | Proposal for NR-NTN (Note: applicable also for IoT-NTN in respective sections of TS36.102) |
| --- | --- | --- |
| Tx requirements |  |  |
| 6.2.1 | UE maximum output power | As proposed in the WID [1,2] (PC3 for NR-NTN, PC3 and PC5 for IoT-NTN) |
| 6.2.2 | UE maximum output power reduction | No changes |
| 6.2.3 | UE additional maximum output power reduction | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.2.4 | Configured transmitted power | No changes |
| 6.3.1 | Minimum output power | No changes |
| 6.3.2 | Transmit OFF power | No changes |
| 6.3.3 | Transmit ON/OFF time mask | No changes |
| 6.3.4 | Power control | No changes |
| 6.4 | Transmit signal quality | No changes |
| 6.5.1 | Occupied bandwidth | No changes |
| 6.5.2 | Out of band emission (ACLR) | No changes |
| 6.5.3.1 | General Spurious emissions | No changes |
| 6.5.3.2 | Spurious emissions for UE co-existence | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.5.3.3 | Additional spurious emissions | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.5.4 | Transmit intermodulation | No changes |
| Rx requirements |  |  |
| 7.3 | Reference sensitivity | FFS |
| 7.4 | Maximum input level | No changes |
| 7.5 | Adjacent channel selectivity | No changes |
| 7.6.1 | General | No changes |
| 7.6.2 | In-band blocking | No changes |
| 7.6.3 | Out-of-band blocking | Reused from n256 |
| 7.6.4 | Narrow band blocking | Reused from n256 |
| 7.7 | Spurious response | No changes |
| 7.8 | Intermodulation characteristics | No changes |

Observation 1: General requirements can be re-used and most of band-specific requirements for n256/B256 can be re-used.

Observation 2: The main issues that need to be addressed are: 
1. Clarification of applicable Regulations related requirements and 
2. DL TN band protection (UE coexistence) requirements.

# 3. Regulations

## 3.1 FCC

The following are FCC regulatory requirements applicable for this band:

==========

#### 47 CFR § 25.202

According to paragraph (a)(4)(ii), the following frequencies are available for use by the 2 GHz Mobile-Satellite Service:

- 2000-2020 MHz: User-to-Satellite Link;

- 2180-2200 MHz: Satellite-to-User Link.

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

- In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)  In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

#### 47 CFR § 25.216

According to paragraph (e) on limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service, the e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

==========

Furthermore, it should be noted that for the TN Band 23, which is studied and documented in [3] more than 10 years ago, the FCC regulation part 25.252 on ATC (Ancillary Terrestrial Component) for satellite service was referred to derive the RF requirements. However, since this NTN S-band work aims at defining NTN band, it is our understanding that for this work, ATC related requirements are not applicable, hence out of scope.

Proposal 2: 	It is proposed to capture the above mentioned FCC CFR applicable for this band in the TR 38.863.

Proposal 3: ATC related requirement is not applicable, hence out of scope for this work.

# 4. Typical TN-NTN Deployment Scenario

Appropriate assumption of deployment scenario is necessary to derive and to come up with an RF requirements that make sense and usable for a real life deployment. The deployment scenario assumptions for the relation between TN and NTN should refer to what has been captured in TR 38.863. In the scope (section 1) of the TR, the following was clarified:

===== TR 38.863 Section 1 =====

NOTE:	Satellite operation is used as complementarily to Terrestrial Network operation, and in separate location when satellite operates in an adjacent channel/band of terrestrial network. For instance, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network.

============================

Furthermore, TR 38.863 section 6.2.1 clearly elaborates the assumption for NTN and TN deployment. Figure 1 below is the figure used in TR section 6.2.1. It is assumed that there is an isolation distance between the TN UE and NTN UE. Isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. In the urban scenario (case 1 in TR 38.863), the isolation distance of 1500m (2*ISD) is considered to reflect the NTN-TN selection algorithm at the border, assuming TN connection will always be prioritized over NTN.

Figure 1: Isolation distance for TN-NTN coexistence used in TR 38.863

It should be noted also that in Rel-17/18, mechanism to ensure the UE connects to NTN when TN is not available in both idle and connected state (time/distance-based cell re-selection and handover) are available.

Proposal 4: The same deployment scenario as listed in TR 38.863, and used for other NTN bands, be used for the new NTN S-band.

# 5. TN-NTN Coexistence

The following figure 1 shows the bands adjacent to and overlap with band 2000 – 2020 MHz UL, 2180 – 2200 MHz DL.

- The 2000 – 2020 MHz UL is adjacent to the n25 DL with 5 MHz separation and to n2 with 10 MHz separation.

- The 2000 – 2020 MHz UL is overlapped with n70 DL

- The 2180 – 2200 MHz DL is overlapped with n66 DL

Note that it is assumed that this band will not be deployed in the geographical area where n34 is deployed.

Figure 1: Adjacent TN bands to NTN band [n252]

- Coexistence with TN DL n25 and n2 (above case a)

One of the objectives of this WI [1] is to evaluate assumptions for co-existence and perform UE-to-UE Coexistence analysis to protect adjacent bands taking into account appropriate NTN – TN deployment assumptions.

In TR 38.863 [5] and TS 38.105 [6], the UE coexistence requirements of -50 dBm/MHz is adopted for n256, with reasoning that the requirement for TN in TS 38.101-1[7] is reused. Given the limited time at the time, RAN4 did not consider the deployment scenario and its impact on the limit as was done for TN in [4].

Focusing on UE coexistence requirements of -50 dBm/MHz, this requirement was adopted specifically for TN use case in the beginning of LTE specifications [4]. It is understood that the requirements come from the assumption of UE-to-UE separation of up to 1m which as pointed in [4] was for indoor scenarios. Although such an assumption may be reasonable in terms of TN – TN deployment, as discussed in section 4, for TN – NTN deployment scenario, the UE-to-UE distance assumption of up to 1m does not comport with the deployment scenario agreed to in [5].

Additionally, and our understanding is that services have been deployed that comply with the regulatory emission limits and no case of harmful interference has been reported to our knowledge. Therefore, we think that the TN UE - NTN UE coexistence requirement of -50 dBm/MHz and underlying assumptions should be reconsidered. RAN4 should consider value that are more appropriate for NTN satellite operation, i.e., value less stringent than -50 dBm/MHz, based on agreed upon separation distance between the TN and NTN UEs, antenna/head/body loss parameters of the NTN and TN UEs.

Observation 3:	TR 38.863 [5] refers to a separation distance of 1500 m between TN and NTN UEs. The UE-UE co-existence value of -50 dBm/MHz is based on a separation distance of 1m [4].

Proposal 5:  	RAN4 should re-evaluate the requirements of -50 dBm/MHz for TN - NTN UE co-existence using realistic deployment scenarios and UE parameters.

- Coexistence with TN DL n70 and n66 (above case b, c)

The 2000 – 2020 MHz UL is completely overlapped with n70 DL and 2180 – 2200 MHz DL is overlapped with the most 20 MHz upper part of n66 DL.

In US, n70 and the most 20 MHz upper part of n66 DL are owned/authorized to the same operator that owns/have authorizations for NTN S-band 2000 – 2020 MHz UL, 2180 – 2200 MHz DL.

In TR 38.863, it was concluded that co-channel overlapping band definition case is out of scope of RAN4 work. Therefore, coexistence for overlapping band between the proposed [n252/B252] with TN n70 and with the most 20MHz upper part of n66 DL should be out of scope of this work.

Proposal 6: 	As concluded in [5] and agreed to for n256 UL and n2/n25 DL in [6], coexistence for overlapping band between the proposed NTN S-band [n252/B252] and TN bands n70 and n66 downlinks DL be out of scope of this work.

# 6. Conclusion

The band plan and proposed tentative band number for this NTN S-band work are clarified. The main issues on applicable regulations requirements and UE Coexistence requirements where elaborated and discussed. The followings are proposed:

Proposal 1: 	Band number [n252] for NR-NTN and [B252] for IoT-NTN are defined for the new NTN S-band.

Observation 1: General requirements can be re-used and most of band-specific requirements for n256/B256 can be re-used.

Observation 2: The main issues that need to be addressed are:

1. Clarification of applicable Regulations related requirements and 
2. DL TN band protection (UE coexistence) requirements.

Proposal 2: 		It is proposed to capture the listed FCC CFR applicable for this band in the TR 38.863.

Proposal 3: 		ATC is not applicable, hence out of scope for this work.

Proposal 4: 	The same deployment scenario as listed in TR 38.863, and used for other NTN bands, be used for the new NTN S-band.

Observation 3:	TR 38.863 [5] refers to a separation distance of 1500 m between TN and NTN UEs. The UE-UE co-existence value of -50 dBm/MHz is based on a separation distance of 1m [4].

Proposal 5: 	RAN4 should re-evaluate the requirements of -50 dBm/MHz for TN - NTN UE co-existence using realistic deployment scenarios and UE parameters.

Proposal 6: 	As concluded in [5] and agreed to for n256 UL and n2/n25 DL in [6], coexistence for overlapping band between the proposed NTN S-band [n252/B252] and TN bands n70 and n66 downlinks DL be out of scope of this work.

# Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, RAN#104, June 2024, DISH Network, EchoStar, et al.

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, RAN4#104, June 2024, DISH Network, EchoStar, et al.

- TR 36.811, “Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America (Release 10)”, v10.0.0

- R4-080710, “TS36.101: TP for UE Spurious emission limits”, RAN4#46bis, April 2028, Motorola

- TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects (Release 18)”, v18.1.0

- TS 38.101-5, “User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)”, v18.6.0

- TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)”, v18.6.0