3GPP RAN4 Meeting #112		R4-2411305

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda item:	6.2.2

Source:	Apple, vivo, Huawei, HiSilicon, OPPO, Samsung, Xiaomi

Title:	OEM views toward concluding the Rel-18 TRP TRS requirements

Release:	Rel-18

Document for:	Approval

# 1	Introduction

RAN4 discussed how to conclude the work on Rel-18 TRP/TRS requirements, with the following agreement captured in the Chair notes [1]:

R4-2410765 WF for [111][336] NR_FR1_TRP_TRS_enh Type: other For: Approval Source: vivo Abstract: Agreement: - RAN4 suggests the extend the performance part of TRP/TRS WI by one quarter to complete the TRP_TRS requriements on the particular bands based on the existing data pool. Decision: Approved.

The approved WF in [2] was as follows:

### Sub-topic 2-3 Rel-18 TRP TRS requirements work Issue 2-3-1: Thresholds of UE information for measured data pool Conclusions: - Adopt the following values for the thresholds related to the devices pool for Rel-18. - Minimum number of devices for each band, each device size, each power class: 40 - Minimum number of device models: 30 - Minimum number of devices' vendors: 5 - Percentage of devices from second-half 2021 to 2024: 100% - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100% Issue 2-3-2: Requirements discussion Working procedure: - RAN4 will go to band-by-band discussion for new bands. Issue 2-3-3: Requirements decision Conclude discussions of requirements for band n1 and n28: requirements; Talk mode TRP; Talk mode TRS; Browsing mode TRP; Browsing mode TRS. n1 PC3; n28 PC3; n1 TRS; n28 TRS; n1 PC3; n28 PC3; n1 TRS; n28 TRS. requirements; [10.5]; [7.0]; [-87.3]; [-80.2]; [11.5]; [9.5]; [-88.5]; [-83.5]. Rel-18 measurement campaign is closed, there will be no data pool update.

Considering the one-quarter extension granted to this work item in RAN #105 [3], this contribution provides the OEM consolidated view toward concluding the work during the RAN4 #112 meeting.

# 2	Discussion

One of the key agreements reached during the RAN4 #111 meeting was related to the data pool itself.  The WF in [2] lists the achieved data pool quality thresholds as well as the statement that the campaign is concluded with no further data pool updates.  The requirement definition, therefore, can only follow this outcome via a direct data-driven approach.  Since RAN4 also agreed to treat the requirements band-by-band, a reliable tool to approach the definition of the requirements is the passing rate approximation from CDFs of the measured results.  Toward that end, the Rapporteur captured summaries of the relevant percentiles in the Ad-Hoc summary [4]:

With the ensuing discussion carefully recorded in the minutes, it is possible to summarize the following factors which guided the OEM view:

-	A percentile target in the range of 80 – 95% represents a meaningful starting point to discuss requirement values, considering the implementation challenges associated with the bands

-	A direct comparison to other organizations’ OTA performance values is not possible due to the use of different hand phantom sizes

-	A meaningful gap in performance is expected between browsing and talk mode requirements in a given band, given the impact of the head phantom on performance (which isn’t present in the browsing mode)

-	The performance of some bands can be impacted by front end duplexer architecture and regional optimizations

Observation 1:	Based on the analysis of the smartphone measurement results and statistics of the UE information pool, all the final requirements should be defined within 80%-95% percentile of the CDF.

Taking these factors into account, the following proposal can be made for n28 and n1 requirements:

Proposal 1:	RAN4 to confirm the Chair’s proposed values in R4-2410765

The discussion of the requirement values for bands n41 and n78 (talk mode only) ended at a less advanced state without a Chair’s recommendation.  As commented by several companies during the discussion, it is important to maintain the integrity of the OTA requirements overall in the specification.  Because the browsing mode requirements were already specified in Rel-17, there is a certain amount of consideration for legacy requirements that should be taken as part of the discussion.

Observation 2:	It is important to maintain the integrity of the OTA requirements overall in the specification; e.g., a meaningful gap in performance between browsing and talk mode requirements is expected due to impact of the head phantom.

With these considerations in mind, the OEMs proposed the following during the RAN4 #111 discussion (the initial operator proposal is also provided for reference):

| requirements | Talk mode TRP |  | Talk mode TRS |  |
| --- | --- | --- | --- | --- |
|  | n41 PC2 | n78 PC2 | n41 TRS | n78 TRS |
| initial OEMs proposal | 9 | 9 | -78.5 | -78.5 |
| initial Operators proposal | 13.46 | 13.58 | -82 | -83.42 |

In an effort to reconcile both Observation 1 and Observation 2, the following compromise proposal is submitted for RAN4 consideration:

| requirements | Talk mode TRP |  | Talk mode TRS |  |
| --- | --- | --- | --- | --- |
|  | n41 PC2 | n78 PC2 | n41 TRS | n78 TRS |
| OEMs Compromise value for Final requirements | 10 | 10.5 | -79 | -79.5 |

Proposal 2:	RAN4 to adopt the OEM Compromise values for the n41 and n78 talk mode requirements.

# 3	Conclusions

Considering the one-quarter extension granted to the Rel-18 TRP/TRS work item, this contribution provides the OEM consolidated view toward concluding the work during the RAN4 #112 meeting, with the following observations and proposals.

Observation 1:	Based on the analysis of the smartphone measurement results and statistics of the UE information pool, all the final requirements should be defined within 80%-95% percentile of the CDF.

Observation 2:	It is important to maintain the integrity of the OTA requirements overall in the specification; e.g., a meaningful gap in performance between browsing and talk mode requirements is expected due to impact of the head phantom.

Proposal 1:	RAN4 to confirm the Chair’s proposed values in R4-2410765

Proposal 2:	RAN4 to adopt the OEM Compromise values for the n41 and n78 talk mode requirements.

# 4	References

“DRAFT Meeting Report for TSG RAN WG4 meeting: 111,” 3GPP RAN4 #111, May 2024

R4-2410765, “WF for [111][336] NR_FR1_TRP_TRS_enh,” vivo, 3GPP RAN4 #111, May 2024

RP-241046, “Status report for WI Perf. part: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” Rapporteur (vivo), 3GPP RAN #104, June 2024

R4-2409920, “Ad-hoc minutes for [111][336] NR_FR1_TRP_TRS_enh,” vivo, 3GPP RAN4 #111