3GPP TSG RAN WG4 Meeting #112	R4-2411308

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	GIST, RRA, CSU

Title:	UE RF Testing Procedure under NFTF Measurement Setup

Agenda Item:		4.7

Document for:	Discussion

Introduction

The National Radio Research Agency (RRA), Chosun University (CSU), and Gwangju Institute of Science and Technology (GIST) have collaboratively developed rapid measurement systems based on the near-field to far-field transformation technique (NFTF). These systems utilize simultaneously activated probe modules for rapidly measuring the transmitter characteristics of user equipment (UEs) in the market for surveillance purposes.

During the RAN4 #110 meeting, we presented the structure of the rapid measurement system, which can measure various transmitter characteristics such as total radiated power (TRP) and peak EIRP. In the RAN4 #111 meeting, the structure of the receiver module and the calibration procedure were discussed. The receiver modules are synchronized using two signals from the GPS-disciplined rubidium oscillator: a 10 MHz reference signal and a 1 pulse-per-second signal. In this contribution, we present the baseline procedures for peak EIRP and TRP measurements. Additionally, the total number of mechanical movements for spherical near-field measurements is presented.

Discussion

Over-the-air (OTA) measurement involves the measurement of wideband UE signals radiating spherically. The signals need to be measured at multiple points over a spherical surface. NFTF processing is performed on the measured data over the spherical surface but is processed at each frequency representing a narrow bandwidth. After the near-field data is transformed to the far-field, the signal power is summed over the wideband but processed at each far-field direction.

3GPP TR 38.810, "Study on Test Methods," describes the baseline measurement procedure. It may be better understood by including the procedures involving a narrow band and the procedures involving specific directions.

Peak EIRP Measurement Procedure

A call simulator sends link signals through a link antenna. The measurement begins after the call simulator and the equipment under test (EUT) have completed the call setup procedure. The TX beam peak direction is set where the maximum total component of EIRP is found, including the respective polarization of the measurement antenna used to form the TX beam.

The measurement procedure includes the following steps:

1) 	Complete the call setup between the call simulator and the EUT and form the TX beam peak direction towards the TX beam peak direction and respective polarization.

2)	Lock the beam toward that direction for the entire duration of the test.

3)	Perform the wideband measurement (both V and H polarizations) at each grid point on the spherical surface around the EUT:

EVNFθi, ϕj,t  and EHNFθi, ϕj,t

4)	Perform the Fourier transform at each measurement points θi, ϕj to determine the amplitude and phase at narrowband frequency points.

EVNFθi, ϕj,t     FT over t     EVNFθi, ϕj,f

EHNFθi, ϕj,t     FT over t     EHNFθi, ϕj,f

5)	Perform the near-field-to-far-field transform at each frequency points to determine the far-field pattern.

EVNFθ, ϕ,fk & EHNFθ, ϕ,fk    NFTF  over θ, ϕ     EθFFθr, ϕr,fk & EϕFFθr, ϕr,fk

6)	Determine the EIRP for both polarizations towards the TX beam peak direction.

EIRP=EIRPθ+EIRPϕ

The procedure outlined above is essentially the same as the one given in 3GPP TR 38.810, except that step 4 is added.

TRP Measurement Procedure

A call simulator sends link signals through a link antenna. The measurement begins after the call simulator and the EUT have completed the call setup procedure. The TX beam peak direction is set where the maximum total component of EIRP is found, including the respective polarization of the measurement antenna used to form the TX beam.

The measurement procedure includes the following steps:

1) 	Complete the call setup between the call simulator and the EUT and form the TX beam peak direction towards the TX beam peak direction and respective polarization.

2)	Lock the beam toward that direction for the entire duration of the test.

3)	Perform the wideband measurement (both V and H polarizations) at each grid point on the spherical surface around the EUT:

EVNFθi, ϕj,t  and EHNFθi, ϕj,t

4)	Perform the Fourier transform at each measurement points θi, ϕj to determine the amplitude and phase at narrowband frequency points.

EVNFθi, ϕj,t     FT over t     EVNFθi, ϕj,f

EHNFθi, ϕj,t     FT over t     EHNFθi, ϕj,f

5)	Perform the near-field-to-far-field transform at each frequency points to determine the far-field pattern.

EVNFθ, ϕ,fk & EHNFθ, ϕ,fk    NFTF  over θ, ϕ     EθFFθr, ϕr,fk & EϕFFθr, ϕr,fk

6)	Determine the TRP value for the constant grid using the TRP integration formula in Annex G.1.3 of 3GPP TR 38.810 document.

The procedure outlined above is essentially the same as the one given in 3GPP TR 38.810, except that step 4 is added.

Number of Measurement Points

The number of measurement points is determined according to the frequency and the size of the EUT:

Δθ=Δϕ=λ2 a+λ ,

where a is the radius of the sphere circumscribing the EUT. For an EUT with a diameter of 10 cm operating at 28 GHz, Δθ=Δϕ= 5.05 degrees. If the measurement circle contains 24 probes covering 360 degrees, the numbers of positioner movements are 360 / 5.05 / 24 = 2.97 times in elevation and 180 / 5.05 = 35.62 times in azimuthal directions, respectively. The numbers are rounded up to give the total number of mechanical movements as 3 × 36 = 108.

The following table shows the number of positioner movements at 28 GHz for EUTs with different sizes. Here, 24 measurement probes are arrayed along a 360-degree circle.

| Frequency [GHz] | 28 | 28 | 28 |
| --- | --- | --- | --- |
| EUT dia. [cm] | 5 | 10 | 15 |
| Δθ, Δϕ [deg.] | 8.59 | 5.05 | 3.58 |
| Number of probes | 24 | 24 | 24 |
| EL movements | 2 | 3 | 5 |
| AZ movements | 21 | 36 | 51 |
| Total number of positioner movements | 42 | 108 | 255 |

Conclusion

The following key aspects were presented:

- Procedure for peak EIRP measurement

- Procedure for TRP measurement

- Total number of positioner movements for measurement

- Reference

- R4-2301361 CR for TR38.810: Integrating simultaneously active probe concept in the NFTF method

- R4-2400727 Discussion on NFTF System for UE Transmitter Characteristics Measurement

- R4-2407599 Discussion on the Operation of the Probe Module Array for NFTF Measurement

- 3GPP TR 38.810 Study on Test Methods

- A. D. Yanghjian, “An overview of near-field antenna measurements,” IEEE Transactions on Antennas and Propagation, vol. AP-34, No. 1, Jan. 1986.