3GPP TSG-RAN WG4 Meeting # 112	R4-2411310

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung, TELUS, Bell mobility

Title: 	Views on UE RF architecture and NW deployment assumption for fragmented carriers

Agenda Item:	8.6.2

Document for:	Discussion

- Introduction

In RAN#103, a SID was approved [1] with following objectives on the study of how to process the fragmented carriers with one Rx Chain.

We present our initial consideration in this paper.

- 2. Discussion

2.1 SID clarification

From the sentence in yellow, it can be found the SID scope is to study the bands with frequency span ≤100MHz which includes all FDD bands and some TDD bands such as n38 and n40, while the bands with large frequency span/range ＞100MHz such as n41 and n77 is out of the SI scope.

Proposal 1: Align the understanding of current SID, the scope should be those bands whose frequency span is within 100MHz, which includes all FDD bands and some TDD bands.

Further, based on operators’ interests presented in RAN4#111 [2], it is preferable to prioritize FDD bands, and some example FDD bands could be selected to facilitate the discussion as the performance could be different for different bands. In our view, the intra-band FDD CA with small duplex gap and large REFSENS degradation and with operators’ interest can be selected, such as n7, n25, n26.

Proposal 2: Prioritize FDD bands.

Proposal 3: Recommend n7, n25, n26 with 2NC CA as example combos.

2.2 UE RF architecture

According to TR 36.823 [3] Figure 6.2.2.1-1, it seems the Rx requirements in current spec for collocated DL intra-band NC CA was specified based on “Partially shared architecture”, a guess is that this architecture was selected as baseline owing to the possibility of support of DL CA along with DL MIMO/Rx diversity. While from what we know, the popular implementation in the field assumes “fully separated architecture” with existing requirements (which are defined based on “partially shared architecture”) being complied with, so in our understanding the only architecture deserves to be studied in Rel-19 is “fully shared architecture”.
For better understanding, the “fully shared architecture” and “partially shared architecture” are illustrated in Figure 1 and Figure 2 separately, while the “fully separated architecture” with totally separated chains for each CC is easy to understood so is not shown here.

Figure 1: Fully shared architecture

Figure 2: Partially shared architecture

For “partially shared architecture”, as shown above, CC1 and CC2 share the antenna and external LNA, while the Rx Chains are split afterwards, which leads to larger size of transceiver, but the size for antenna and RFFE which are outside of transceiver remain the same. For instance, if we assume a handheld UE is equipped with 2 antennas for low bands, then only 2CC NC CA can be supported with “fully separated architecture” without MIMO/Rx diversity. And with “partially shared architecture”, 2CC NC CA+MIMO/Rx diversity is feasible, but probably we should say the Rx Chain number is 4 rather than 2 in this case. While by “fully shared architecture”, the number of Rx Chains would be reduced to 2 in this case. One obvious advantage for “fully separated architecture” is its ability to process two separate carriers located arbitrarily within a frequency band owing to the presence of 2 separate LOs. In contrast, the “fully shared architecture” can only support intra-band DL CA with FS≤100MHz due to the following limitations:

- Capability of LO

- Analog filter

- FFT

The receiver would operate in low-IF mode with two CCs sharing one LO, meaning that the down-converted image signal could degrade the performance of the receiver.

As mentioned above, we think Rx performance for “partially shared architecture” and “fully separated architecture” should be quite similar/comparable. CC1 and CC2 can be picked up by different analog filters in both architectures, and the digital filter can provide further attenuation for ACS/in-band blocking/narrow band blocking interferers. Further, from what we know, “fully separated architecture” as popular implementation for pure intra-band DL CA without MIMO/Rx diversity, can satisfy the existing requirements.

Observation 1: According to TR 36.823, it seems the Rx requirements in current spec are specified based on “partially shared architecture”

Observation 2: From what we know the popular implementation for pure intra-band DL CA without MIMO/Rx diversity is “fully separated architecture”, with the existing requirements being complied with. One obvious advantage for “fully separated architecture” is that it can process two separate carriers located arbitrarily within a frequency band. One disadvantage is that it supports fewer NC CCs.

Observation 3: The performance for “partially shared architecture” and “fully separated architecture” are comparable, and the spec is implementation agnostic.

Proposal 4: For intra-band DL NC CA, RAN4 to confirm that current spec accommodate both “partially shared architecture” and “fully separated architecture”.

Proposal 5: For fragmented carriers SI, only the “fully shared architecture” needs to be studied.

2.3 PSD consideration with NW deployment assumption

It is generally assumed that the PSD difference between 2 collocated CCs of one operator is within 6 dB (equal PSD assumed with 6 dB tolerance), in which both CCs are wanted signal. However, it appears RAN4 has never had a discussion on the PSD difference between 2 collocated operators in which CC1 and CC2 from operator A are wanted signals and another CC from operator B is the interferer from operator A’s perspective. So we think it is reasonable to use this SI to determine an acceptable value for such a difference. During RAN-P#104, [4] raised a similar question which is illustrated in Figure 3.

Figure 3. The PSD between the wanted signal and the PSD between the wanted signal and interference

We learned from Canadian operators, that there is no “co-construction and co-site” deployment (meaning sharing infrastructure such as RU/BS antenna) in Canada but only “co-location” (meaning only location is shared but not infrastructure). Other operators also said that “co-location” is the most common scenario.

Given the above, we think it would be difficult to predict the power and PSD imbalances for any band as the maximum EIRP and the transmitted CBW could be different for two gNBs of two operators in the same location. In addition, 3GPP has not specified a rated power for Wide Area BS so the limitation of the power is more reliant on national regulation.

More specifically, after looking at the regulatory requirements of several bands, it appears that for many bands the maximum EIRP is specified as XX dBm/MHz (in the term of PSD) for the channel bandwidth ≥1MHz. The regulation requirements for n66 and n25 are illustrated below. The n7 band has similar technical requirements as band n25.

Illustration of Canadian regulation for 3GPP band n66, SRSP-513 [7]

Illustration of Canadian regulation for 3GPP band n25, SRSP-510 [8]

The regulation specifies the maximum allowed EIRP, so chances are the power level is different if BSs are not transmitting at maximum power, if different channel bandwidths are taken into account, the power level / PSD differences become more unpredictable.

For evaluation purposes, for a given deployment area, we feel we could go with some simple assumption as the starting point, i.e., the BSs of the two collocated inter-operators are both transmitting at maximal allowed power level /PSD level per national regulation.

But so far we have no clear view as to whether new verification cases such as new in-band blocking scenario or new PSD difference+REFSENS relaxation requirements need be introduced to reflect the PSD difference of two collocated operators.

Observation 4: It is difficult to predict the power level difference/PSD difference between two co-location operators given the EIRP and CBW of gNBs for transmission could be different.

Proposal 6: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure).

Proposal 7: For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level.

Proposal 8: Further, in the context of the Canadian regulation, for n7/n25/n66, equal PSD between two co-location operators could be assumed for evaluation purpose, as the starting point.

- Conclusion

（SID clarification）

Proposal 1: Align the understanding of current SID, the scope should be those bands whose frequency span is within 100MHz, which includes all FDD bands and some TDD bands.

Proposal 2: Prioritize FDD bands.

Proposal 3: Recommend n7, n25, n26 with 2NC CA as example combos.

（UE RF architecture）

Observation 1: According to TR 36.823, it seems the Rx requirements in current spec are specified based on “partially shared architecture”

Observation 2: From what we know the popular implementation for pure intra-band DL CA without MIMO/Rx diversity is “fully separated architecture”, with the existing requirements being complied with. One obvious advantage for “fully separated architecture” is that it can process two separate carriers located arbitrarily within a frequency band. One disadvantage is that it supports fewer NC CCs.

Observation 3: The performance for “partially shared architecture” and “fully separated architecture” are comparable, and the spec is implementation agnostic.

Proposal 4: For intra-band DL NC CA, RAN4 to confirm that current spec accommodate both “partially shared architecture” and “fully separated architecture”.

Proposal 5: For fragmented carriers SI, only the “fully shared architecture” needs to be studied.

(PSD difference consideration with NW deployment assumption)

Observation 4: It is difficult to predict the power level difference/PSD difference between two co-location operators given the EIRP and CBW of gNBs for transmission could be different.

Proposal 6: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure).

Proposal 7: For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level.

Proposal 8: Further, in the context of the Canadian regulation, for n7/n25/n66, equal PSD between two co-location operators could be assumed for evaluation purpose, as the starting point.

- 4. Reference

| [1] | RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MTK, RAN#104 |
| --- | --- |
| [2] | R4-2407414, Fragmented spectrum scenarios, TELUS, Bell Mobility, Telstra |
| [3] | TR 36.823 |
| [4] | RP-241269, Fragmented carrier, Apple, RAN#104 |
| [5] | R4-1347431, Non-contiguous intra-band Band 3 reference sensitivity, Qualcomm, RAN4#68bis |
| [6] | R4-134091, Non-contiguous intra-band Band 3 reference sensitivity, Qualcomm, RAN4#68 |
| [7] | SRSP-513 — Technical Requirements for Advanced Wireless Services in the Bands 1710–1780 MHz and 2110–2180 MHz [SRSP-513 — Technical Requirements for Advanced Wireless Services in the Bands 1710–1780 MHz and 2110–2180 MHz (canada.ca)] |
| [8] | SRSP-510 — Technical Requirements for Personal Communications Services (PCS) in the Bands 1850-1915 MHz and 1930-1995 MHz [SRSP-510 — Technical Requirements for Personal Communications Services (PCS) in the Bands 1850-1915 MHz and 1930-1995 MHz (canada.ca)] |