3GPP TSG-RAN WG4 Meeting # 112	R4-2411312

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung

Title: 	Views on non-collocated deployment

Agenda Item:	8.5.2.2

Document for:	Discussion

- Introduction

In RAN#103, a new WID on Non-collocated deployment is approved [1] which focus on specifying UE RF/RRM requirements for Type-4a/b UE capability and clarifying the UE behaviour for Type-4a/b supported UE.

Discussion was organized in RAN4#111 and the agreements were captured in [2]. It was agreed that Type-4 to reuse Type-2 RF requirements.

(Note that Type-1/2/3/4 in Rel-18 discussion are intended for different UE capabilities, rather than UE type.)

- 2. Background

Standardization on non-collocated deployment is motivated by the fact the spectrum on C band in Japan/Korea was allocated at different time, hence the legacy BS usually only supports part of the C-band spectrum. There are two choices when it comes to the new BS, either developed based on the whole C-band spectrum holding of an operator or based on the spectrum allocated later to the operator, the latter makes it non-collocated deployment along with a legacy BS to support non-collocated NR-CA or EN-DC.

Rel-15 Inter-band EN-DC with overlapping DL bands

The discussion on the necessity of non-collocated inter-band EN-DC with overlapping DL bands deployment can date back to Rel-15, during which operators proposed the possible non-collocated scenario, while due to time limitation RAN4 concluded the simple assumption, i.e., “only co-located deployment” is allowed. It should be noted that due to different band numbering system, DC_42_n77 is inter-band EN-DC with overlapping DL bands but essentially equivalent to intra-band EN-DC. Accordingly, the following restriction is provided in Rel-15 RAN4 requirement, which forbid non-collocated deployment for inter-band EN-DC with overlapping DL bands: (1) MRTD ＜3us; (2) Power imbalance between downlink carriers is no larger than 6dB.

Rel-15 Intra-band NR-CA

Similarly for intra-band NR-CA, only co-located deployment is assumed, and the restriction is provided in Rel-15 RAN4 requirement, which forbid non-collocated deployment for intra-band NR-CA, i.e., MRTD＜3us.

Rel-16/17 Inter-band EN-DC with overlapping DL bands for Non-collocated Deployment

In Rel-16/17, the requirement on Type-2 UE (i.e., UE supporting interBandMRDC-WithOverlapDL-Bands-r16, which indicates 2 layer/2 Rx Chain per CC supported for non-collocated EN-DC) has been specified in terms of RF and RRM requirements. Specifically, 25dB power imbalance and 1dB REFSENS relaxation for Type-2 UE in-band blocking requirements of inter-band EN-DC with overlapping DL bands have been specified in [2] in RAN4#102e meeting.

Rel-18 Inter-band EN-DC with overlapping DL bands for Non-collocated Deployment

Though the RF/RRM requirements have already been specified for Type-2 Inter-band EN-DC with overlapping DL bands from Rel-16, one issue was identified during the Rel-18 discussion for Type-2 intra-band NR-CA, which is deemed as a common issue for both Type-2 EN-DC and Type-2 NR-CA, i.e., NW may not be able to control UE to work under Type-1 mode or Type-2 mode, especially when maxMIMO-Layers is less than 2

Consequently, a new NW control signaling (nonCollocatedTypeMRDC) and a new UE indication capability (requirementTypeIndication-r18) are introduced in RAN2 [3][4][5] to allow NW configure UE work under Type-1 mode or Type-2 mode for Rel-18 and onwards(excerpted as below). Note that Type-2 is deemed as the default mode if UE indicate the support of Type-2, which is to align with previous releases.

In addition, the Rel-18 discussion focus on 2CC non-contiguous EN-DC such as DC_42A_77A.

(TS 38.331)

Note that there is an error in TS38.331 (yellow marked), should be “intra-band”.

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

(TS 38.306)

Note that this requirementTypeIndication-r18 is only applicable to EN-DC case but not NR-CA, and to our understanding “requirementTypeIndication-r18+ interBandMRDC-WithOverlapDL-Bands-r16= interBandMRDC-WithOverlapDL-Bands-r18”. Further, This IE is basically to inform NW “I am a Rel-18 UE and I can understand nonCollocatedTypeMRDC” thus NW can configure this UE.

| requirementTypeIndication-r18 Indicates whether the UE supports network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16. The UE supports this feature shall also indicate support of interBandMRDC-WithOverlapDL-Bands-r16. | UE | No | No | FR1 only |
| --- | --- | --- | --- | --- |

Rel-18 Intra-band NR-CA for Non-collocated Deployment

Similarly as the discussion of non-collocated inter-band EN-DC with overlapping DL bands, 25dB power imbalance and 1dB REFSENS relaxation is defined, while a new UE capability(intraBandNR-CA-non-collocated-r18) and a new NW control signalling(nonCollocatedTypeNR-CA) are introduced[3][4][5] for Rel-18 and onwards to enable non-collocated intra-band NR-CA assuming 2 MIMO layer/2 Rx Chain per CC. In addition, the Rel-18 discussion focus on 2CC non-contiguous NR-CA such as n77(2A)/n78(2A).

(TS 38.331)

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].

(TS 38.306)

| intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. This capability is only supported for band n77/n78. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

- Discussion

- 3.1 New UE capability

The working assumption is reproduced in Figure 1 which is excerpted from rapporteur company KDDI’s material.

Figure 1: Working assumption for Type-3a/b and Type-4a/b UE

The new UE capability(s) required for Type-4 indication was discussed and the following two options are to be further considered.

As agreed in Rel-18, EN-DC can have either Type-4a (6Rx in total) or Type-4b(8Rx in total) capability given 4Rx is optionally support for LTE band (the following table is excerpted from TS 36.101). And NR-CA only have Type-4b capability considering there is no difficulty for NR n77/n78 to support 4Rx per CC Type-4 capability for FWA.

For the per-BC UE capability design, to our understanding there are two alternatives from RAN2 perspective:

- Alt 1) One UE capability (One IE name) for EN-DC and NR-CA, contained in both NR-CA container and EN-DC container. Which means although we use one UE capability, it can be used to indicate capability for EN-DC capability container and NR-CA capability container separately.

- Alt 2) Two UE capabilities, one dedicated for EN-DC and the other for NR-CA.

Observation 1: For the per-BC UE capability design, to our understanding there are two alternatives from RAN2 perspective:

- Alt 1) One UE capability (One IE name) for EN-DC and NR-CA, contained in both NR-CA container and EN-DC container. Which means although we use one capability, it can be used to indicate capability for EN-DC capability container and NR-CA capability container separately.

- Alt 2) Two UE capabilities, one dedicated for EN-DC and the other for NR-CA.

During last meeting it was stated that there is no need to distinguish Type-4a and Type-4b, indication of Type-4b would be enough and Type-4a can be discarded for EN-DC. However, Type-4a/4b differentiation has already been agreed in Rel-18 for EN-DC, and also 4Rx is not a mandatory feature for LTE band, so we prefer to keep the previous agreements (i.e., keep Type-4a for EN-DC) to make discussion simple. Further, some FWA may be equipped with 6Rx for which Type-4b cannot be indicated.

After RAN4 make the decision, we can just simply deliver our agreements and demands to RAN2, while one UE capability or two UE capabilities could be left for RAN2 experts to decide. In case Option 1 is taken, Samsung RAN2 preference is to have two UE capabilities (i.e., Alt1) in Observation 1.

Proposal 1: RAN4 to decide between following options, and deliver the agreements/demand to RAN2. While 1 UE capability or 2 UE capabilities can be left for RAN2 to determine and design.

Option 1: UE can support either Type-4a or Type-4b for EN-DC; UE can support Type-4b for NR-CA →Samsung preference

Option 2: UE can support Type-4 (i.e., Type-4b) for EN-DC; UE can support Type-4 (i.e., Type-4b) for NR-CA

- 3.2 UE behavior

The relationship between Type-4 capability and Type-2 capability is agreed as following in last meeting, i.e., as long as UE indicates the support of Type-4 capability, Type-2 capability is assumed as support by default. This agreement has been taken into account in the following UE behavior discussion including:

- UE initial configuration/behavior on collocated or non-collocated deployment, at the initial connection mode

- UE switching behavior between collocated and non-collocated deployment

Before diving to the UE behavior discussion, we would like to revisit the above agreements made in last meeting. One point has been overlooked probably is that for legacy gNB who has no idea of Type-4 capability, UE still needs to report Type-2 UE capability if UE supports it. Thus, we have following observation and proposal:

Observation 2: To keep backward compatibility (i.e., for legacy gNB), UE still needs to report Type-2 UE capability if UE supports it.

Proposal 2: Modify last meeting’s agreements as following.

- 3.2.1 UE switching behavior between Type-4 and Type1/Type-2/single carrier

Our thinking is illustrated in the following figure assuming UE is configured on Type-4 mode at this moment. The figure shows all the possible “switching path” from Type-4 mode to either Type-2 mode or Type-1 mode or single carrier operation. In our understanding, probably there is no need to argue which path is the most desirable one with maximum benefit, but rather focus on making everything possible and under NW full control.

(Please note that in Rel-18 design, it points to Type-2 when “nonCollocatedTypeMRDC” or “nonCollocatedTypeNR-CA” is “absent”, rather than when they are “present”.)

More explanation of above figure,

- From Type-4 to Type-2: As both Type-4 and Type-2 are non-collocated with same RF requirements and most likely same RRM requirements as well, there is no new NW signaling needed as the deployment scenario does not change, UE can just do MIMO layer re-config to become 2 layer per CC(Type-2).

- From Type-4 to Type-1: This is from non-collocated to collocated scenario. Collocated can be either up to 4-layer/per CC or 8 layer/per CC or 2-layer per CC. We think 4 layer/CC can be the baseline for collocated Type-1 in this path, and if NW want UE to have 8-layer or 2-layer per CC, it can be achieved by further MIMO layer re-config. In our understanding new NW signaling is needed here, owing to the pre-condition/restriction of the usage of the Rel-18 NW signaling which is only present for a UE configured with maxMIMO-layers≤2. Therefore, the exiting NW IE (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) cannot be used to instruct the Type-4 to Type-1 switch because Type-4 can support up to 4 layer per CC. More details of the Rel-19 new signaling would be shared in next subclause. In addition, technically, this path maybe the least likely one, as there is no strong justification for a FWA to do switch between collocated and non-collocated.

- From Type-4 to single carrier operation: Achieving by Scell release, and Pcell remains, which is a legacy approach.

Proposal 3: It is proposed to confirm the possibility for the “switching paths” in above figure. Further, it is proposed to confirm that all paths are under NW full control. In addition.

- No new NW signaling needed from Type-4 to Type-2 switch

- No new NW signaling needed from Type-4 to single carrier fallback

- New NW signaling needed from Type-4 to Type-1 (more details of New NW signaling is present in Proposal 4)

- 3.2.2 At initial connection mode

We discuss the “switching behavior” in last clause, and the other case to be discussed is for the initial connection mode. How to configure the UE working under either collocated scenario or non-collocated scenario at the beginning.

In Rel-18, the definition of Rel-18 NW signaling is copied as below for reference. It is worth nothing that the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2 which is a pre-condition/restriction of the usage as mentioned in last subclause. And if maxMIMO-layers ＞2, it is collocated.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Observation 3: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

In Rel-19, we think same methodology can be followed in RAN2 NW signaling design. More specific,

1) New Rel-19 NW signaling is utilized to differentiate Type-4 and Type-1 with the condition/restriction it is only present when 2＜maxMIMO-layers ≤4

2) If maxMIMO-layers ＞4, it is collocated

Proposal 4: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Our initial design is something like the following, in a simple “present-absent” way.

For the initial connection,

1）maxMIMO-layers≥8, it is collocated

2）maxMIMO-layers=4, it could be either collocated (Type1) or non-collocated(Type-4), pending on whether new NW signaling is present

3）maxMIMO-layers=2, it could be either collocated (Type 1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

While during the offline discussion among companies before submission deadline, there are two good counter-examples provided by company, which demonstrates the above Rel-19 NW signaling design is not good enough. More specific, if Rel-19 NW signaling is designed in a simple “present-absent” way, we would face the following issues, and the Rel-18 argument would repeat.

- Rel-19 type 4 capable UE behavior operate under Rel-18 network

[Samsung] Assuming a Rel-19 UE indicates both Type-4 and Type-2, then Rel-18 NW only understand Type-2 and use maxMIMO-layers=4 to control UE work under collocated if following above design, but Rel-19 UE would see "new Rel-19 NW signaling =absent + maxMIMO-layers=4", then this UE misthink Non-collocated Type-4. Discrepancy between UE and gNB happens.

(Note in our understanding a UE has no idea of the NW is of which release, and also a NW has no idea of the UE is of which release)

- Only type 2 capable Rel-19 UE behavior under Rel-19 network.

[Samsung] Seems we can just follow Rel-18 design in this case? If Type-4 is not reported, it implies this UE does not understand new Rel-19 NW signaling. So for collocated, Rel-19 NW just use " maxMIMO-layers=4". If this is common understanding, seems there would be no issue. But this needs to be clarified.

To well resolve above issues, it is proposed to have the Rel-19 NW signaling design in the “absent, present with value=0, present with value=1” way:

Proposal 5: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

For the initial connection, if

1）maxMIMO-layers≥8, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

More specific, “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 4. Conclusion

Observation 1: For the per-BC UE capability design, to our understanding there are two alternatives from RAN2 perspective:

- Alt 1) One UE capability (One IE name) for EN-DC and NR-CA, contained in both NR-CA container and EN-DC container. Which means although we use one capability, it can be used to indicate capability for EN-DC capability container and NR-CA capability container separately.

- Alt 2) Two UE capabilities, one dedicated for EN-DC and the other for NR-CA.

Proposal 1: RAN4 to decide between following options, and deliver the agreements/demand to RAN2. While 1 UE capability or 2 UE capabilities can be left for RAN2 to determine and design.

- Option 1: UE can support either Type-4a or Type-4b for EN-DC; UE can support Type-4b for NR-CA →Samsung preference

- Option 2: UE can support Type-4 (i.e., Type-4b) for EN-DC; UE can support Type-4 (i.e., Type-4b) for NR-CA

Observation 2: To keep backward compatibility (i.e., for legacy gNB), UE still needs to report Type-2 UE capability if UE supports it.

Proposal 2: Modify last meeting’s agreements as following.

Proposal 3: It is proposed to confirm the possibility for the “switching paths” in above figure. Further, it is proposed to confirm that all paths are under NW full control. In addition.

- No new NW signaling needed from Type-4 to Type-2 switch

- No new NW signaling needed from Type-4 to single carrier fallback

- New NW signaling needed from Type-4 to Type-1 (more details of New NW signaling is present in Proposal 4)

Observation 3: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

Proposal 4: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Proposal 5: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

For the initial connection, if

1）maxMIMO-layers≥8, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

- Note: “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 5. Reference

[1] RP-240838, R19 WID_Intra-band non-collocated EN-DC and NR-CA, RAN#103

[2] R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111