3GPP TSG-RAN WG4 Meeting # 112	R4-2411313

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung, CHTTL

Title: 	Views on technical wording ambiguities

Agenda Item:	10.1.1.1

Document for:	Discussion

- Introduction

In last meeting which is the second RAN4 discussion for spec improvement, we share our views on the identified ambiguity of some technical wording and table simplification/modification [1]. The group reached consensus on table simplification aspect and the initial discussion on some technical wording ambiguities were also organized. The agreements are captured in [2].

In this paper, we further present our views on the following wording ambiguities.

- 2. Discussion

2.1 What is “Assigned to”?

This issue has been discussed while no formal agreement reached in last meeting, our views can be found in [1]. Our position remains unchanged at this meeting, so in the following we only list the observations and proposal for simplicy.

Observation 1: “Assigned to” has different meanings for different requirements.

Observation 2: Change closed release may have adverse impact on commercial implementation.

Observation 3: Change from open release may cause NBC issue.

Observation 4: Based on the experience of the “famous” over one year debate of NR_Power_Class, we feel pessimistic there would be any meaningful outcome.

Proposal 1: Stop the discussion for clarification of “assigned to” and do not make change to spec.

2.2 Dual Tx/2Tx/TxD/4Tx

This issue has not been discussed online in last meeting due to the limited time, the proposals from Qualcomm, Samsung and Ericsson are listed in Moderator summary as following. As can be found, the three companies’ views are quite aligned but ours include more information/observation. Thus, it is suggested to first align the understanding before further discuss how to improve the spec. We think our proposal can be taken as the common understanding.

Proposal 2: Confirm the following understanding:

In current RAN4 spec,

- “Dual Tx” is not intended for “dualPA-Architecture”

- “Dual Tx” intends for “2Tx-TxD” if the relevant description is not from UL-MIMO clauses

- “Dual Tx” intends for “2 Tx antenna connectors” if the relevant description is from UL-MIMO clauses

- “2Tx” intends for “2Tx antenna connectors”, “4Tx” intends for “4Tx antenna connectors”

- “TxD” can be “2Tx-TxD” and/or “4Tx-TxD”, it is easy to judge which it is based on the context.

Then we can discuss how to integrate above clarification into spec. As shared above, it is easy to judge TxD is “2Tx-TxD” or “4Tx-TxD” based on the context, so the items need to be clarified in spec is the meaning of “2Tx”, “4Tx” and “dual Tx”. There are two potential alternatives in our mind for further clarification if all the existing “dual Tx”, “2Tx” and “4Tx” would still be kept.

- Alt 1:  Add clarification in clause 4.2

- Alt 2:  Add clarification in clause 6.1

For Alt 1, in current clause 4.2, there is already some description for 2Tx-TxD and 4Tx-TxD as following, therefore having additional clarification here could be one option intuitively.

For Alt 2, owing to “2Tx”, “4Tx” and “dual Tx” are all Tx relevant, and these notion are not shown elsewhere of Clause 5 and Clause 7, so we think clause 6.1 is a better place for the clarification.

Given CR is still not allowed in this meeting, we provide an example as below for the group’s further consideration, based on TS38.101-1 V 18.5.0.

6.1 General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

For UEs that do not indicate IE dualPA-Architecture, transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports/4 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The applicability of transmitter requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

NOTE :	For band n100 and n101, the EIRP requirements shall be converted to conducted requirements based on the post chipset unit antenna connector gain Gn100_101post connector declared for the UE following the principle described in annex M.

Unless otherwise stated, in the following sub clauses,

- The term “Dual Tx” refers to two Tx antenna connectors if the associated description is in suffix D.

- The term “Dual Tx” refers to 2Tx Tx diversity capability if the associated description is in the suffix other than suffix D.

- The term “Dual Tx” is not intended for dualPA-Architecture.

- The terms “Dual Tx” and “2 Tx” all refer to two Tx antenna connectors, and “4Tx” refer to four Tx antenna connectors.

Proposal 3: Add clarification in Clause 6.1, the following can be the starting point.

Unless otherwise stated, in the following sub clauses,

- The term “Dual Tx” refers to two Tx antenna connectors if the associated description is in suffix D.

- The term “Dual Tx” refers to 2Tx Tx diversity capability if the associated description is in the suffix other than suffix D.

- The term “Dual Tx” is not intended for dualPA-Architecture.

- The terms “Dual Tx” and “2 Tx” all refer to two Tx antenna connectors, and “4Tx” refer to four Tx antenna connectors.

2.3 What does “this bit is not set” mean?

This issue has not been discussed online during last meeting due to the limited time.

In Table L.1.1, it mentions many times “If this bit is set to 1, XXX requirements apply, If the bit is not set, YYY requirements applies”. Take the following table as example, it is not crystal clear to us what does “If the bit is not set” mean? There could be the following three interpretations:

- Interpretation 1: modifiedMPR-Behavior shall be present and this bit is set to 0 → From the perspective of English grammar

- Interpretation 2: modifiedMPR-Behavior is absent → Given it is not necessary for UE to specifically indicate this IE if only leftmost bit is utilized which is a waste of signaling

- Interpretation 3: Including both Interpretation 1 and 2. → Most reasonable interpretation given for example a legacy Rel-16 UE supporting n39 may not indicate this IE

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | Requirements for network signalling value NS_21 as defined in Clause 6.5.2.3.y of 38.101-1 v17.6.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v17.6.0. | This bit shall be set to 1 by a UE supporting the Rel-17 version of the specification. If the bit is not set, then requirements for NS_21 as defined in Clause 6.5.2.3.3 of 38.101-1 v16.11.0 and A-MPR as defined in Clause 6.2.3.14 of 38.101-1 v16.11.0 apply. |
| n39 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

Proposal 4: RAN4 to discuss what is the correct interpretation for “If the bit is not set” in Table L.1-1 of 38.101-1.

- Interpretation 1: modifiedMPR-Behavior shall be present and this bit is set to 0 → From the perspective of English grammar

- Interpretation 2: modifiedMPR-Behavior is absent → Given it is not necessary for UE to specifically indicate this IE if only leftmost bit is utilized which is a waste of signaling

- Interpretation 3: Including both Interpretation 1 and 2. → Most reasonable interpretation given for example a legacy Rel-16 UE supporting n39 may could not indicate this IE

Proposal 5: If the above interpretation 3 is agreed for “if the bit is not set”. Change “If the bit is not set” to “If modifiedMPR-Behavior is absent, or modifiedMPR-Behavior is present with this bit set to 0” of Table L.1-1 of 38.101-1.

- 3. Conclusion

(For “Assigned to”)

Observation 1: “Assigned to” has different meanings for different requirements.

Observation 2: Change closed release may have adverse impact on commercial implementation.

Observation 3: Change from open release may cause NBC issue.

Observation 4: Based on the experience of the “famous” over one year debate of NR_Power_Class, we feel pessimistic there would be any meaningful outcome.

Proposal 1: Stop the discussion for clarification of “assigned to” and do not make change to spec.

(For “Dual Tx/2Tx/TxD/4Tx”)

Proposal 2: Confirm the following understanding:

In current RAN4 spec,

- “Dual Tx” is not intended for “dualPA-Architecture”

- “Dual Tx” intends for “2Tx-TxD” if the relevant description is not from UL-MIMO clauses

- “Dual Tx” intends for “2 Tx antenna connectors” if the relevant description is from UL-MIMO clauses

- “2Tx” intends for “2Tx antenna connectors”, “4Tx” intends for “4Tx antenna connectors”

- “TxD” can be “2Tx-TxD” and/or “4Tx-TxD”, it is easy to judge which it is based on the context.

Proposal 3: Add clarification in Clause 6.1, the following can be the starting point.

Unless otherwise stated, in the following sub clauses,

- The term “Dual Tx” refers to two Tx antenna connectors if the associated description is in suffix D.

- The term “Dual Tx” refers to 2Tx Tx diversity capability if the associated description is in the suffix other than suffix D.

- The term “Dual Tx” is not intended for dualPA-Architecture.

- The terms “Dual Tx” and “2 Tx” all refer to two Tx antenna connectors, and “4Tx” refer to four Tx antenna connectors.

(What does “if the bit is not set” mean?)

Proposal 4: RAN4 to discuss what is the correct interpretation for “If the bit is not set” in Table L.1-1 of 38.101-1.

- Interpretation 1: modifiedMPR-Behavior shall be present and this bit is set to 0 → From the perspective of English grammar

- Interpretation 2: modifiedMPR-Behavior is absent → Given it is not necessary for UE to specifically indicate this IE if only leftmost bit is utilized which is a waste of signaling

- Interpretation 3: Including both Interpretation 1 and 2. → Most reasonable interpretation given for example a legacy Rel-16 UE supporting n39 may could not indicate this IE

Proposal 5: If the above interpretation 3 is agreed for “if the bit is not set”. Change “If the bit is not set” to “If modifiedMPR-Behavior is absent, or modifiedMPR-Behavior is present with this bit set to 0” of Table L.1-1 of 38.101-1.

- 4. Reference

[1] R4-2407625, Views on technical wording ambiguities and table modifications for spec improvement, Samsung, CHTTL, RAN4#111

[2] R4-2410714, WF on [111][139] UERF_Spec_Improvement_v5, Qualcomm, RAN4#111