3GPP TSG-RAN WG4 Meeting # 112	R4-2411315

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung

Title: 	Views on HPUE intra-band CA

Agenda Item:	8.1.1.1.1

Document for:	Discussion

- Introduction

The agreements and open issues of last meeting are captured in WF [1]. We present our further consideration in this paper for HPUE intra-band CA.

- 2. Discussion

2.1 Consideration on SAR for all scenarios of this WI

Observation 1: 3GPP duty cycle solution is meaningless for UE vendors, as SAR is tested by different test house with 3GPP compliance, while the SAR test houses do not admit 3GPP’s duty cycle solution/scheme, they only admit P-MPR (sensor based) and operators’ official TDD configuration.

Observation 2: Duty-cycle solution is not specified for PC2 FDD single band.

Observation 3: There is no IE for FDD-FDD EN-DC duty-cycle solution.

Observation 4: Duty-cycle solution is not specified for increasing high power limit feature.

Proposal 1: 3GPP duty-cycle solution is not specified for any scenarios of this WI.

2.2 PCMAX,f,c

The agreement of last meeting is excerpted as following.

For intra-band PC1.5 CA with dual PA-architecture meaning one PA per CC, the PCMAX,C for each CC should be limited to 26dBm owing to the PA capability limitation. For intra-band PC1.5 CA with TxD capability, we think there is no need to make restriction on PCMAX,C as equal PSD is just the assumption for MPR evaluation, in the field UE can adopt unequal PSD for sake of full power, so this evaluation assumption(i.e. Equal PSD) should not be reflected in spec for PCMAX,C at least.

Proposal 2: For PCMAX,C

-For PC1.5 UE with Dual PA architecture, PCMAX,C for each CC is limited to 26dBm

-For PC1.5 UE with TxD, no need to make restriction on PCMAX,C to reflect equal PSD which is the assumption for MRP evaluation but not the real deployment restriction.

2.3 PCMAX

The agreement of last meeting is excerpted as following.

As mentioned above, equal PSD assumption is just for MPR evaluation purpose, not the real deployment restriction. While a large PSD difference may lead to unaffordable power back off to comply with the emission requirements. Further study is needed on whether to have some reasonable PSD difference assumption to guarantee the reasonable MPR.

Observation 5: Further study is needed on whether to have some reasonable PSD difference assumption to guarantee the reasonable MPR.

- 3. Conclusion

Observation 1: 3GPP duty cycle solution is meaningless for UE vendors, as SAR is tested by different test house with 3GPP compliance, while the SAR test houses do not admit 3GPP’s duty cycle solution/scheme, they only admit P-MPR (sensor based) and operators’ official TDD configuration.

Observation 2: Duty-cycle solution is not specified for PC2 FDD single band.

Observation 3: There is no IE for FDD-FDD EN-DC duty-cycle solution.

Observation 4: Duty-cycle solution is not specified for increasing high power limit feature.

Proposal 1: 3GPP duty-cycle solution is not specified for any scenarios of this WI.

Proposal 2: For PCMAX,C

-For PC1.5 UE with Dual PA architecture, PCMAX,C for each CC is limited to 26dBm

-For PC1.5 UE with TxD, no need to make restriction on PCMAX,C to reflect equal PSD which is the assumption for MRP evaluation but not the real deployment restriction.

Observation 5: Further study is needed on whether to have some reasonable PSD difference assumption to guarantee the reasonable MPR.

- 4. Reference

[1] R4-2410750, WF on higher power UE, Samsung, RAN4#111