3GPP TSG-RAN WG4 Meeting # 112	R4-2411316

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung

Title: 	Views on HPUE inter-band CA with 2Tx or 3Tx

Agenda Item:	8.1.1.1.2

Document for:	Discussion

- Introduction

The agreements and open issues of last meeting are captured in WF [1]. We present our further consideration in this paper for inter-band ULCA with either 2Tx or 3Tx.

- 2. Discussion

2.1 Consideration on SAR for all scenarios of this WI

Observation 1: 3GPP duty cycle solution is meaningless for UE vendors, as SAR is tested by different test house with 3GPP compliance, while the SAR test houses do not admit 3GPP’s duty cycle solution/scheme, they only admit P-MPR (sensor based) and operators’ official TDD configuration.

Observation 2: Duty-cycle solution is not specified for PC2 FDD single band.

Observation 3: There is no IE for FDD-FDD EN-DC duty-cycle solution.

Observation 4: Duty-cycle solution is not specified/adopted for increasing high power limit feature.

Proposal 1: 3GPP duty-cycle solution is not specified for any scenarios of this WI.

2.2 General MSD rules

It was stated MSD general discussion is out of scope of the WID, but actually we already reached some agreements on MSD rule in the first meeting of Rel-18 which are captured in WF [2], excerpted as following.

Further, we do not think MSD general rule discussion is against the WID, our understanding of the following sentence(yellow marked) is that leaving the specific MSD values for basket WIDs but not MSD general rules, as they are for sure not BC specific requirements. If the Note is like “NOTE: leave the band combination specific requirements, e.g., MSD values to the corresponding Rel-19 basket WIs”, that would be perfect, but even without “values” we should assume MSD general rule discussion is within scope.

In addition, we do not understand how MSD general rules can be left for basket WID, especially in Rel-19 EN-DC HUPE and NR-CA HPUE are still separate WIs then where to discuss it? In addition, general items should not be determined in basket WIDs.

Observation 5: MSD general rules are not band combination requirements, but MSD values.

Proposal 2: RAN4 to clarify that MSD general rule discussion is within the WI scope, just the specific MSD values are left for corresponding basket WIDs to evaluate.

As elaborated in our paper [3] of last meeting, the general requirements can be defined in a configuration agnostic way if duty-cycle solution for SAR compliance is dropped, so seems the only remaining general requirement to be discussed is the MSD rules.

An innovative WF [4] on MSD was agreed in last meeting, excerpted as following. We can call it MSD revolution, MSD evolution and remaining unchanged for Option 1, 2, 3. Similar forward-looking ideas are also proposed in [5][6] of last meeting.

From our observation the group is already sick of calculating MSD, checking MSD and correcting MSD, which is very time consuming but actually the values can not be fully verified by conductive tests given part of MSD values are OTA relevant. In addition, defining MSD in spec delay the NW deployment as the HPUE Note has to be added after MSD evaluation according to current procedure. In our view Rel-19 is the appropriate time to reconsider it, we are supportive to the Option 1 listed below.

Proposal 3: For all types of MSD, RAN4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- 3. Conclusion

(For SAR issue)

Observation 1: 3GPP duty cycle solution is meaningless for UE vendors, as SAR is tested by different test house with 3GPP compliance, while the SAR test houses do not admit 3GPP’s duty cycle solution/scheme, they only admit P-MPR (sensor based) and operators’ official TDD configuration.

Observation 2: Duty-cycle solution is not specified for PC2 FDD single band.

Observation 3: There is no IE for FDD-FDD EN-DC duty-cycle solution.

Observation 4: Duty-cycle solution is not specified/adopted for increasing high power limit feature.

Proposal 1: 3GPP duty-cycle solution is not specified for any scenarios of this WI.

(For MSD)

Observation 5: MSD general rules are not band combination requirements, but MSD values.

Proposal 2: RAN4 to clarify that MSD general rule discussion is within the WI scope, just the specific MSD values are left for corresponding basket WIDs to evaluate.

Proposal 3: For all types of MSD, RAN4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- 4. Reference

[1] R4-2410750, WF on higher power UE, Samsung, RAN4#111

[2] R4-2406583, WF on higher power UE for ULCA and EN-DC, Samsung, RAN4#111

[3] R4-2407631, Views on HPUE for inter-band NR-CA and EN-DC, Samsung, RAN4#111

[4] R4-2410605, WF on HPUE MSD Simplification, Skyworks, Murata, Qualcomm, Samsung, Nokia, RAN4#111

[5] R4-2408724, HPUE MSD related issues, Nokia, RAN4#111

[6] R4-2408033, HPUE_MSD, Qualcomm, RAN4#111