3GPP TSG-RAN WG4 Meeting # 112	R4-2411317

# Maastricht, Netherlands, 19th - 23th, Aug 2024

Source: 	Samsung

Title: 	Views on increasing higher power limit

Agenda Item:	8.1.1.1.3

Document for:	Discussion

- Introduction

The agreements and open issues of last meeting are captured in WF [1]. We present our further consideration in this paper for increasing higher power limit.

- Discussion

For the methodology, the agreement of last meeting is to use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point. We want to remove “as starting point” given if the existing feature works well, it is anyway not preferrable to introduce a new mechanism to further complex UE and NW design.

Proposal 1: For the methodology, use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability.

The scenarios to be considered in Rel-19 are also left open. Increasing high power limit feature was introduced in Rel-17 in order to maximize PA’s capability to achieve full power. Due to the limited time, only PC3+PC2(=27.8dBm in total) which is based on PC2 was specified in Rel-17 for both NR-CA and EN-DC with single carrier for each individual band, other scenarios were dropped. In Rel-18, the feature was further extended to PC3+PC5 (=24.6 dBm in total) which is based on PC3, one of the two bands is NR-U band with PC5 as default PC, and both single carrier for each band(2CC in total) and inter+intra band (3CC UL) are covered.

Moreover, in Rel-18 3Tx WI, it is clarified “Note 2: Increase UE power high limit feature is not included”. Accordingly though PC1.5 is introduced in this WI, PC3+PC1.5 or PC2+PC1.5 has not been discussed yet.

Observation 1: The following table summarize the specified scenarios of increasing high power limit feature in Rel-17/18.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3(FDD or TDD) | PC2(TDD) | Support from Rel-17 | One CC per band |
| PC3 | PC5(NRU band) | PC3(FDD or TDD) | Support from Rel-18 | One CC per band; One CC on band B, 2CC on band A(NRU band) |

For 2Tx inter-band CA, the following 3CC combos were introduced in TS 38.101-1/3 v18.5.0 with PC2 assuming 2PA transmitting simultaneously. Based on this, we think PC2 with one CC on one band and 2CC on the other band can be endowed with increasing higher power limit feature. In addition, in Rel-17/18, for PC2 2Tx with one CC per band, the configuration for the PC2 band is limited to TDD, we think this restriction can be removed from Rel-19.

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3C_n41A | 266 | +2/-3 | 23 | +2/-3 |
| DC_8B_n78A | 266 | +2/-3 | 23 | +2/-3 |
| DC_39C_n41A | 265 | +2/-3 | 23 | +2/-3 |
| DC_41C_n77A | [266] | [+2/-3] | 23 | +2/-3 |

In summary, for 2Tx, the following scenario can be considered for NR-CA/EN-DC in Rel-19.  While by combining the following Rel-19 scenario table and Rel-17/18 scenario table, basically all the scenarios that have already been specified would be allowed for increasing higher power limit feature.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3(FDD or TDD) | PC2 (FDD or TDD) | Support from Rel-19 | One CC on band A, 2CC on band B |
| PC2 | PC3(FDD or TDD) | PC2(FDD) | Support from Rel-19 | One CC per band |

Further, to be more future proof, for 2Tx we think all scenarios can be allowed, not just limited to the combos that have already been specified, given anyway the total power would not exceed 29dBm due to the single PA capability restriction so there would be no heat dissipation issue.

In terms of the spec change, for PC2, we think the current Notes of 38.101-1 and -3 can be modified as follows for 2Tx, while similar ones can be added into 3Tx MOP table.

(Note 7 of Table 6.2A.1.3-1, TS 38.101-1)

NOTE 7:	The UE that supports a PC2 uplink CA configuration with  a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD or FDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

(Note 8 of Table 6.2B.1.3-1, TS 38.101-3)

NOTE 8:	The UE that supports a PC2 uplink EN-DC configuration with  a composite of supporting PC3 within a TDD or FDD band and PC2 within a second TDD or FDD band may signal a higherPowerLimitMRDC-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3.

Proposal 2: For 2Tx, higher power limit feature is enabled for any specified inter-band band combination.

- “Specified” here intends for the existing combos and the ones to be added into MOP table in future

Given 3Tx and 2Tx CA are in different suffix, they can be considered separately. The following table summarize the scenarios that may could be considered/discussed in Rel-19.

Proposal 3: For 3Tx, the following scenarios may could be considered for NR-CA/EN-DC.

(Note the analysis is based on the implementation feasibility)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | -One CC per band -One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx) |
| PC1.5 | PC3 | PC1.5 | 30.0 | -One CC per band; For FWA only -One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx); For FWA only |

- 3. Conclusion

Proposal 1: For the methodology, use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability.

Proposal 2: For 2Tx, higher power limit feature is enabled for any specified inter-band band combination.

- “Specified” here intends for the existing combos and the ones to be added into MOP table in future

Proposal 3: For 3Tx, the following scenarios may could be considered for NR-CA/EN-DC.

(Note the analysis is based on the implementation feasibility)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | -One CC per band -One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx) |
| PC1.5 | PC3 | PC1.5 | 30.0 | -One CC per band; For FWA only -One CC in band A, 2CC on band B (if this scenario would be introduced for 3Tx); For FWA only |

- 4. Reference

[1] R4-2410750, WF on higher power UE, Samsung, RAN4#111