3GPP TSG-RAN4 Meeting #112	R4-2411326

# Maastricht, Netherlands, 19th Aug 2024 – 23rd Aug 2024

Source:	Samsung

Title:	Discussion on MPR reduction for FR1 single carrier

Agenda item:	8.1.1.2.1

Document for:	Discussion

Introduction

In RAN4#111, a WF [1] is approved which focus on power domain enhancements. The following WF was captured, which focus on power domain enhancements for single carrier.

## 1.1 Consideration on relaxation of requirements for identified scenarios

Agreement in main session:

- No relaxation of level of spurious emission requirements

  - FFS on whether to change the spurious emission boundary

## 1.2. Mechanisms for enabling MPR reduction and/or power boosting

Way forward:

- Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects

  - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge

    - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW

    - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

    - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

    - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

Discussion

It was agreed in RAN4#111 main session that: “No relaxation of level of spurious emission requirements” and whether to change the spurious emission boundary needs further study.

As already discussed in our paper [2] last meeting, different from SEM and ACLR, SE is deemed as regulatory requirements to our understanding. As shown in Figure 1, 3GPP has general SE requirements, which are applied from FOOB = 6 MHz for 3 MHz channel bandwidth and BWChannel + 5 for other channel bandwidths. In comparison, the regulator usually has additional requirements and could be more stringent in ITU-R radio regulations (ITU REC SM.329/328), which are applied from 250% of the “necessary bandwidth” from the center frequency, as mentioned by MTK’s paper [3] of last meeting. Therefore, the spurious emission boundary should be treated very cautiously. More specific, we can only touch the spurious emission boundary specified by 3GPP.

Figure 1: Illustration of 3GPP and ITU OOB boundary

Proposal 1: The spurious emission boundary should be treated very cautiously and we can only touch the spurious emission boundary specified by 3GPP.

When the Shifted Channel Edge approaches are applied, full RB allocation can be converted to the “inner RB allocation” and IBE requirements can be applied at this time. In this context, the spurious emission boundary specified by 3GPP can be changed. Otherwise, it is not reasonable to change spurious emission boundary since “No relaxation of level of spurious emission requirements” has been agreed in RAN4#111 main session.

Proposal 2: When the Shifted Channel Edge approaches are applied, the spurious emission boundary specified by 3GPP can be changed. Otherwise, it is not reasonable to change spurious emission boundary.

It was agreed in RAN4#111 main session that: “Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point”. Therefore, in this context, when extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW, it is not feasible to convert full RB allocation in UE CBW to “inner RB allocation”, since it is risky to apply the “inner RB allocation” IBE requirements outside the BS CBW. For example, there is another operator’s BS around as shown in Figure 2, “inner RB allocation” IBE requirements are not enough to protect another operator’s BS and there will be interference issues between operators.

Figure 2: Illustration of extended UE CBW edge exceeds the BS CBW edge

Observation 1: When extended UE CBW edge exceeds the BS CBW edge, it is risky to apply the “inner RB allocation” IBE requirements outside the BS CBW.

Proposal 3: When extended UE CBW edge exceeds the BS CBW edge, it is not feasible to apply the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW and corresponding IBE requirements.

As show in Figure 3, when extended UE CBW edge is within the BS CBW, full RB allocation can be converted to “inner RB allocation” and then IBE requirements can be applied. On the one hand, the extended UE CBW is introduced for converting full RB allocation in UE CBW to “inner RB allocation” and applying equivalent IBE requirements. If IBE requirements are applied between edges of UE CBW and BS CBW, there is no need to introduce the extended UE CBW since it can be can be replaced by BS CBW. Furthermore, for RedCap, the narrower UE CBW is usually located at the edge of wider BS CBW. In this context, if IBE requirements are applied between edges of UE CBW and BS CBW, full RB allocation will be converted to “outer RB allocation” instead of “inner RB allocation”, which departs from the starting point in approved WF. To sum up, IBE requirements should be applied between edges of UE CBW and extended UE CBW. In this case, ACLR and SEM are naturally applicable from the edge of extended UE CBW instead of the BS CBW.

Figure 3: Illustration of IBE used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

Proposal 4: IBE should be used between edges of UE CBW and extended UE CBW.

Proposal 5: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

As mentioned by MTK’s paper [3] of last meeting, if integral region and boundary of OOBE is based on extended UE CBW, spurious emission will not fulfil the 250% requirement specified by ITU-R radio regulations as shown in Figure 4. Therefore, integral region and boundary of OOBE should be based on UE CBW.

Figure 4: Illustration of integral region and boundary of OOBE based on extended UE CBW or UE CBW

Proposal 6: Integral region and boundary of OOBE should be based on UE CBW.

- 3 Conclusion

In the document, the following Observations and Proposals are made:

Proposal 1: The spurious emission boundary should be treated very cautiously and we can only touch the spurious emission boundary specified by 3GPP.

Proposal 2: When the Shifted Channel Edge approaches are applied, the spurious emission boundary specified by 3GPP can be changed. Otherwise, it is not reasonable to change spurious emission boundary.

Observation 1: When extended UE CBW edge exceeds the BS CBW edge, it is risky to apply the “inner RB allocation” IBE requirements outside the BS CBW.

Proposal 3: When extended UE CBW edge exceeds the BS CBW edge, it is not feasible to apply the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW and corresponding IBE requirements.

Proposal 4: IBE should be used between edges of UE CBW and extended UE CBW.

Proposal 5: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 6: Integral region and boundary of OOBE should be based on UE CBW.

- References

- R4-2410749, WF on power domain enhancements, Huawei.

- R4-2407590, Discussion on MPR reduction for FR1 single carrier, Samsung.

- R4-2407910, Discussion on power domain enhancement for NR single carrier, MediaTek Inc.