3GPP TSG-RAN4 Meeting #112	R4-2411327

# Maastricht, Netherlands, 19th Aug 2024 – 23rd Aug 2024

Source:	Samsung

Title:	Discussion on MPR reduction for FR1 intra-band UL CA

Agenda item:	8.1.1.2.2

Document for:	Discussion

Introduction

In RAN4#111, a WF [1] is approved which focus on power domain enhancements. The following WF was captured, which focus on power domain enhancements for FR1 intra-band UL CA.

## 2.1. MPR applicability for FR1 intra-band contiguous UL CA

Way forward:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

  - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

  - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA

## 2.2. MPR applicability for FR1 intra-band UL non-contiguous CA

Way forward:

Enhancements not aligned with the WID will not be considered.

- Further clarify the scope for NC CA in next meeting based on the WID objectives.

Discussion

## 2.1 MPR reduction for NR FR1 intra-band contiguous UL CA

As already discussed in our paper [2] last meeting, for FR1 intra-band contiguous UL CA (PC2/PC3), the RF requirements/UE behaviors can be changed at least from Rel-19, when the UL CC is deactivated from activated status or activated from deactivated status. Therefore, when only one of the 2 CC is activated, it is reasonable/justifiable to follow the MPR requirements of single CC operation, which is more stringent compared to intra-band contiguous UL CA MPR. Furthermore, in last meeting, it was agreed that the single CC MPR requirements apply for NR FR2 intra-band contiguous UL CA with single CC with activated cell. All in all, for PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. In this context, integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW since single CC MPR requirements apply.

Observation 1: It is reasonable/justifiable to follow the MPR requirements of single CC operation, which is more stringent compared to intra-band contiguous UL CA MPR.

Observation 2: It has been agreed that the single CC MPR requirements apply for NR FR2 intra-band contiguous UL CA with single CC with activated cell.

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply.

Proposal 2: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW.

As already mentioned in last meeting, power boosting feature is only specified for single CC operation not CA. Therefore, to our understanding, power boosting feature should not be applied to intra-band contiguous UL CA, even when only one of the 2 CC is activated. And we further polish our proposal of last meeting as following,

For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell,

(1) MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

(2) MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

(3) MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 3: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell,

MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

## 2.2 MPR reduction for NR FR1 intra-band non-contiguous UL CA

As already discussed in our paper [2] last meeting, for FR1 intra-band contiguous UL CA (PC2/PC3) in the WID, it is “Specify MPR applicability” instead of “Specify new MPR requirements/values” and “Specify new MPR requirements/values” brings large workload and ambiguous benefit. Therefore, to our understanding, the work scope for NC CA should be limited to only study MPR applicability.

Observation 3: In the WID, it is “Specify MPR applicability” instead of “Specify new MPR requirements/values” and “Specify new MPR requirements/values” brings large workload and ambiguous benefit.

Proposal 4: The work scope for FR1 NC CA should be limited to only study MPR applicability.

- 3 Conclusion

In the document, the following Observations and Proposals are made:

Observation 1: It is reasonable/justifiable to follow the MPR requirements of single CC operation, which is more stringent compared to intra-band contiguous UL CA MPR.

Observation 2: It has been agreed that the single CC MPR requirements apply for NR FR2 intra-band contiguous UL CA with single CC with activated cell.

Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply.

Proposal 2: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW.

Proposal 3: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell,

MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Observation 3: In the WID, it is “Specify MPR applicability” instead of “Specify new MPR requirements/values” and “Specify new MPR requirements/values” brings large workload and ambiguous benefit.

Proposal 4: The work scope for FR1 NC CA should be limited to only study MPR applicability.

- References

- R4-2410749, WF on power domain enhancements, Huawei.

- R4-2407591, Discussion on MPR reduction for FR1 intra-band UL CA, Samsung.