3GPP RAN WG4 Meeting #112	R4-2411336

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on Core requirements of PRS BW aggregation

Source: 	CATT

Agenda item:	6.1.1.4

Document for:	Discussion

Introduction

In RAN4#111 meeting, some agreements regarding the performance requirements were achieved and were captured in the WF [1]. In this paper, we will further discuss the leftovers related to the core requirements for BW aggregation.

Discussion

Interruption requirements for SRS aggregation

In RAN4#111 meeting, it was agreed to define interruption requirements for SRS aggregation:

Agreement - Define interruption requirements for SRS transmission with BW aggregation on CC without PUSCH/PUCCH. - Reuse the framework of SRS carrier and/or antenna switching based interruption requirements as the starting point. - The interruption length is FFS.

According to the above agreement, it was agreed to reuse the framework of SRS carrier and/or antenna switching based interruption requirements as the starting point, but the interruption length is FFS.

In the following section, we will discuss the interruption requirements for SRS aggregation based on the interruptions at NR SRS carrier based switching.

According to the discussions on interruptions for SRS carrier switching, two interruptions are defined which cover both switching to and switching back procedure, and each interruption is defined as SRS carrier switching time plus 6 symbols [2]. The agreements are copied below:

Agreements in RAN4#93: - SRS carrier switching time to define interruption requirements - Define requirements for 200us, 500us and 900us SRS carrier switching time - FFS for FR1/FR2 applicability of SRS carrier switching time - For the case of SRS transmission on (single or multiple) CC, define two interruption, i.e. covering switch to and switch back, each interruption is defined as - SRS carrier switching time + 6 symbols - RAN4 to define unified interruption requirements for sync and async case - Requirements are based on async case

The corresponding contents of interruptions for SRS carrier switching are copied and a figure is drawn to illustrate the referred contents.

Fig1. Interruption for SRS carrier switching

##### 8.2.2.2.9 Interruptions at NR SRS carrier based switching (Contents omitted) When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell if UE is not capable of Per-FR gap, or on active serving cell(s) in FR1 if UE is capable of Per-FR gap, during the switching to the carrier of a serving cell in FR1 not configured for PUCCH/PUSCH transmission, - with up to X1 slot as specified in Table 8.2.2.2.9-1. (Contents omitted) Table 8.2.2.2.9-1: Interruption length X1 (slot) NR Slot length; SRS carrier; Interruption length X1 (slots). (ms) of victim cell; switching time (us)Note 1; Sub carrier spacing for aggressor cell (kHz). 15; 30. 0; 1; ≤ 200; 2; 2. 300, 500; 2; 2. 900; 2; 2. 1; 0.5; ≤ 200; 3; 2. 300, 500; 3; 3. 900; 4; 4. 2; 0.25; ≤ 200; 4; 3. 300, 500; 5; 4. 900; 7; 6. 3; 0.125; ≤ 200; 7; 5. 300, 500; 9; 7. 900; 12; 10. 5; 0.03125; ≤ 200; 22; 15. 300, 500; 31; 24. 900; 44; 37. 6; 0.015625; ≤ 200; 42; 28. 300, 500; 61; 47. 900; 87; 73. Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR.. (Contents omitted) For intra-band SRS carrier switching in FR1 or FR2, interruptions in Table 8.2.2.2.9-1 and in Table 8.2.2.2.9-2 based on SRS carrier switching time ≤ 200us shall apply. For inter-band SRS carrier switching in FR1, interruptions in Table 8.2.2.2.9-1 and in Table 8.2.2.2.9-2 shall apply.

From the above highlighted contents, we notice that the interruptions are defined for active serving cell(s) except for the target carrier to which the UE switches. And the interruption requirements were defined separately for switching to the target CC(s) and switching back to the original CC as Figure 1 shows. However, the scenario is much simpler in SRS bandwidth aggregation since a serial of switching does not happen, and considering a complete interruption that covers two guard periods and SRS transmission time is enough as Figure 2 shows. Furthermore, in BW aggregation scenarios, the aggregated carriers have the same SCS and are intra-band, synchronous, which also is different from SRS carrier switching. Hence, specific interruption values need recalculating when reusing the framework for SRS carrier switching or antenna switching.

Fig2. Interruption for SRS bandwidth aggregation

Since guard period will affect the length of interruption, the corresponding agreements are copied below:

RAN4 (RF group) responded to the RAN1 LS as: - The following candidate values can be used for the guard period values as described in the second agreement. - {0us, 30us, 100us, 140us, 200us}.

And based on the above discussions, the total interruption length will be: guard period * 2 + SRS transmission. And the same assumption for SRS carrier switching is reused for SRS transmission duration, i.e., 6 OFDM symbols.

Proposal 1: Interruption length is derived by: guard period + SRS transmission + guard period.

Based on the above analysis, the interruption lengths for SRS aggregation in FR1 can be defined as follows, considering different SCS combinations for victim cells and aggressor cells.

Table 1: Interruption length X1 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 1 | 1 |
| 1 | 0.5 | 0, 30 | 1 | 1 |
|  |  | 100, 140 | 2 | 1 |
|  |  | 200 | 2 | 2 |
| 2 | 0.25 | 0 | 2 | 1 |
|  |  | 30 | 2 | 2 |
|  |  | 100, 140 | 3 | 2 |
|  |  | 200 | 4 | 3 |
| 3 | 0.125 | 0 | 4 | 2 |
|  |  | 30 | 4 | 3 |
|  |  | 100 | 5 | 4 |
|  |  | 140 | 6 | 4 |
|  |  | 200 | 7 | 5 |
| 5 | 0.03125 | 0 | 13 | 7 |
|  |  | 30 | 15 | 9 |
|  |  | 100 | 20 | 13 |
|  |  | 140 | 22 | 16 |
|  |  | 200 | 26 | 20 |
| 6 | 0.015625 | 0 | 26 | 13 |
|  |  | 30 | 30 | 17 |
|  |  | 100 | 39 | 26 |
|  |  | 140 | 44 | 31 |
|  |  | 200 | 52 | 39 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |

Similarly, the interruption lengths for SRS aggregation with aggressor cell(s) in FR2 are defined in Table 2:

Table 2: Interruption length X2 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X2 (slots) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |  |
|  |  |  | 60 | 120 | 480, 960 |
| 0 | 1 | ≤ 200 | 1 | 1 | 1 |
| 1 | 0.5 | ≤ 140 | 1 | 1 | 1 |
|  |  | 200 | 2 | 1 | 1 |
| 2 | 0.25 | 0 | 1 | 1 | 1 |
|  |  | 30 | 1 | 1 | 1 |
|  |  | 100 | 2 | 1 | 1 |
|  |  | 140 | 2 | 2 | 2 |
|  |  | 200 | 3 | 2 | 2 |
| 3 | 0.125 | 0 | 1 | 1 | 1 |
|  |  | 30 | 2 | 1 | 1 |
|  |  | 100 | 3 | 2 | 2 |
|  |  | 140 | 4 | 3 | 3 |
|  |  | 200 | 5 | 4 | 4 |
| 5 | 0.03125 | 0 | 4 | 2 | 1 |
|  |  | 30 | 6 | 4 | 3 |
|  |  | 100 | 10 | 8 | 7 |
|  |  | 140 | 13 | 11 | 10 |
|  |  | 200 | 17 | 15 | 14 |
| 6 | 0.015625 | 0 | 7 | 4 | 1 |
|  |  | 30 | 11 | 8 | 5 |
|  |  | 100 | 20 | 16 | 14 |
|  |  | 140 | 25 | 22 | 19 |
|  |  | 200 | 33 | 29 | 27 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |  |

A draft CR to incorporate this interruption requirement into spec is also submitted [3].

Proposal 2: The interruption lengths for SRS aggregation with aggressor cell(s) in FR1 are defined as:

Table 1: Interruption length X1 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 1 | 1 |
| 1 | 0.5 | 0, 30 | 1 | 1 |
|  |  | 100, 140 | 2 | 1 |
|  |  | 200 | 2 | 2 |
| 2 | 0.25 | 0 | 2 | 1 |
|  |  | 30 | 2 | 2 |
|  |  | 100, 140 | 3 | 2 |
|  |  | 200 | 4 | 3 |
| 3 | 0.125 | 0 | 4 | 2 |
|  |  | 30 | 4 | 3 |
|  |  | 100 | 5 | 4 |
|  |  | 140 | 6 | 4 |
|  |  | 200 | 7 | 5 |
| 5 | 0.03125 | 0 | 13 | 7 |
|  |  | 30 | 15 | 9 |
|  |  | 100 | 20 | 13 |
|  |  | 140 | 22 | 16 |
|  |  | 200 | 26 | 20 |
| 6 | 0.015625 | 0 | 26 | 13 |
|  |  | 30 | 30 | 17 |
|  |  | 100 | 39 | 26 |
|  |  | 140 | 44 | 31 |
|  |  | 200 | 52 | 39 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |

And for aggressor cell(s) in FR2, the interruption lengths are:

Table 2: Interruption length X2 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X2 (slots) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |  |
|  |  |  | 60 | 120 | 480, 960 |
| 0 | 1 | ≤ 200 | 1 | 1 | 1 |
| 1 | 0.5 | ≤ 140 | 1 | 1 | 1 |
|  |  | 200 | 2 | 1 | 1 |
| 2 | 0.25 | 0 | 1 | 1 | 1 |
|  |  | 30 | 1 | 1 | 1 |
|  |  | 100 | 2 | 1 | 1 |
|  |  | 140 | 2 | 2 | 2 |
|  |  | 200 | 3 | 2 | 2 |
| 3 | 0.125 | 0 | 1 | 1 | 1 |
|  |  | 30 | 2 | 1 | 1 |
|  |  | 100 | 3 | 2 | 2 |
|  |  | 140 | 4 | 3 | 3 |
|  |  | 200 | 5 | 4 | 4 |
| 5 | 0.03125 | 0 | 4 | 2 | 1 |
|  |  | 30 | 6 | 4 | 3 |
|  |  | 100 | 10 | 8 | 7 |
|  |  | 140 | 13 | 11 | 10 |
|  |  | 200 | 17 | 15 | 14 |
| 6 | 0.015625 | 0 | 7 | 4 | 1 |
|  |  | 30 | 11 | 8 | 5 |
|  |  | 100 | 20 | 16 | 14 |
|  |  | 140 | 25 | 22 | 19 |
|  |  | 200 | 33 | 29 | 27 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |  |

Conclusions

This contribution discussed the interruption requirements for SRS BW aggregation positioning and some proposals are provided:

Proposal 1: Interruption length is derived by: guard period + SRS transmission + guard period.

Proposal 2: The interruption lengths for SRS aggregation are defined as:

Table 1: Interruption length X1 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 1 | 1 |
| 1 | 0.5 | 0, 30 | 1 | 1 |
|  |  | 100, 140 | 2 | 1 |
|  |  | 200 | 2 | 2 |
| 2 | 0.25 | 0 | 2 | 1 |
|  |  | 30 | 2 | 2 |
|  |  | 100, 140 | 3 | 2 |
|  |  | 200 | 4 | 3 |
| 3 | 0.125 | 0 | 4 | 2 |
|  |  | 30 | 4 | 3 |
|  |  | 100 | 5 | 4 |
|  |  | 140 | 6 | 4 |
|  |  | 200 | 7 | 5 |
| 5 | 0.03125 | 0 | 13 | 7 |
|  |  | 30 | 15 | 9 |
|  |  | 100 | 20 | 13 |
|  |  | 140 | 22 | 16 |
|  |  | 200 | 26 | 20 |
| 6 | 0.015625 | 0 | 26 | 13 |
|  |  | 30 | 30 | 17 |
|  |  | 100 | 39 | 26 |
|  |  | 140 | 44 | 31 |
|  |  | 200 | 52 | 39 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |

And for aggressor cell(s) in FR2, the interruption lengths are:

Table 2: Interruption length X2 (slot)

|  | NR Slot length (ms) of victim cell | Guard period (us) Note 1 | Interruption length X2 (slots) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SCS for aggressor cell (kHz) |  |  |
|  |  |  | 60 | 120 | 480, 960 |
| 0 | 1 | ≤ 200 | 1 | 1 | 1 |
| 1 | 0.5 | ≤ 140 | 1 | 1 | 1 |
|  |  | 200 | 2 | 1 | 1 |
| 2 | 0.25 | 0 | 1 | 1 | 1 |
|  |  | 30 | 1 | 1 | 1 |
|  |  | 100 | 2 | 1 | 1 |
|  |  | 140 | 2 | 2 | 2 |
|  |  | 200 | 3 | 2 | 2 |
| 3 | 0.125 | 0 | 1 | 1 | 1 |
|  |  | 30 | 2 | 1 | 1 |
|  |  | 100 | 3 | 2 | 2 |
|  |  | 140 | 4 | 3 | 3 |
|  |  | 200 | 5 | 4 | 4 |
| 5 | 0.03125 | 0 | 4 | 2 | 1 |
|  |  | 30 | 6 | 4 | 3 |
|  |  | 100 | 10 | 8 | 7 |
|  |  | 140 | 13 | 11 | 10 |
|  |  | 200 | 17 | 15 | 14 |
| 6 | 0.015625 | 0 | 7 | 4 | 1 |
|  |  | 30 | 11 | 8 | 5 |
|  |  | 100 | 20 | 16 | 14 |
|  |  | 140 | 25 | 22 | 19 |
|  |  | 200 | 33 | 29 | 27 |
| Note1: Guard period is UE capability indicated by guardPeriod in NR-UL-SRS-Capability. |  |  |  |  |  |

References

[1] R4-2410353, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#111.

[2] R4-1915930, Way forward on R16 NR RRM enhancements – SRS carrier switching, ZTE, RAN4#93.

[3] R4-2311331, Draft CR on interruption requirements for SRS BW aggregation, CATT, RAN4#112.