3GPP RAN WG4 Meeting #112	R4-2411337

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on Performance requirements of Sidelink positioning

Source: 	CATT

Agenda item:	6.1.2.2

Document for:	Discussion

Introduction

In RAN4#111 meeting, fruitful progress regarding the performance requirements was achieved which are captured in the WF [1]. In this paper, we will further discuss a few open issues belonging to the performance requirements.

Discussion

In this contribution, we will discuss SL PRS-RSRP(P) measurement accuracy and delay test cases.

Whether to verify accuracy of legacy measurements in SL PRS-RSRP(P) TCs

Companies have different views on whether to verify accuracy of legacy measurements in SL PRS-RSRP(P) test cases or not. Candidate options are copied below for reference:

Proposals in RAN4#111: - Option 1: - Consider the following rules: - If RAN1/2 confirms that the SL PRS-RSRP(P) is possible to be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy is verified alone in separate section. - If RAN1/2 confirms that the SL PRS-RSRP(P) cannot be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy should be verified with RSTD/RX-TX accuracy together in one section. - Option 2: - Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP. - Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section. - Option 3: - Support option 1a to define separate sections for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

A LS was sent to RAN1 and RAN2 to confirm if SL PRS-RSRP/PRS-RSRPP can be requested/reported standalone in RAN4#110bis, and reply LSs from RAN1/2 are received [2][3]. According to the reply LSs, SL PRS-RSRP(P) cannot be requested standalone. We also checked the TS 38.355 [4] and observed the SL PRS-RSRP(P) measurements are always requested with other types of measurements (SL RSTD/Rx-Tx/AoA).

Both IEs SL-TDOA-RequestLocationInformation and SL-AoA-RequestLocationInformation are copied below:

-- ASN1START

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-START

SL-TDOA-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                     ENUMERATED { true }        OPTIONAL,

sl-LOS-NLOS-IndicatorRequest           ENUMERATED { true }        OPTIONAL,

sl-PRS-RSRP-Request                    ENUMERATED { true }        OPTIONAL,

sl-RSRPP-Request                       ENUMERATED { true }        OPTIONAL,

sl-AdditionalPathsRequest              ENUMERATED { true }        OPTIONAL,

measurementsForMultipleARP-IDs-Rx      SEQUENCE {

requestedARP-IDs-Rx                    BIT STRING (SIZE (4))  OPTIONAL

}                                                                 OPTIONAL,

...

}

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

-- ASN1START

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-START

SL-AoA-RequestLocationInformation ::= SEQUENCE {

sl-AoA-Request                        ENUMERATED { aoa, zoa, both},

measurementReportingType              ENUMERATED { gcs, lcsWithTranslation, lcsWithoutTranslation }    OPTIONAL,

measurementsForMultipleARP-IDs-Rx     SEQUENCE {

requestedARP-IDs-Rx                   BIT STRING (SIZE (4))                                        OPTIONAL

}                                                                                                      OPTIONAL,

sl-ARP-InfoRequest                    ENUMERATED { true }                                              OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }                                              OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }                                             OPTIONAL,

sl-RSRPP-Request                      ENUMERATED { true }                                             OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }                                              OPTIONAL,

...

}

-- TAG-SL-AOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

Based on the IE structures and the reply, it is quite clear that SL PRS-RSRP(P) measurements are always requested with other types of measurements (SL RSTD/Rx-Tx/AoA). Therefore it is reasonable to verify legacy measurement accuracy in SL PRS-RSRP(P) test cases to reflect UE’s positioning performance in real deployment.

Observation 1: SL PRS-RSRP(P) measurements are always requested with other types of measurements (i.e., SL RSTD/Rx-Tx/AoA).

But there is an issue if RAN4 to verify both legacy measurements and SL PRS-RSRPP in one TC which is the channel model used to derive the measurement accuracy requirements for legacy measurement and SL PRS-RSRPP is different. So a compromise is needed. Therefore we propose to verify the accuracy of legacy measurement and SL PRS-RSRP(P) in one TC, but the accuracy of the legacy measurements in SL PRS-RSRPP TC measured in two-tap channel is expected to fulfil the measurement accuracy requirement for fading channel.

Proposal 1: RAN4 to verify measurement accuracy of the legacy measurements and SL PRS-RSRP(P) in one test case in a separate clause.

Proposal 2: Measurement accuracy of the legacy measurements in SL PRS-RSRPP TC is expected to fulfil the measurement accuracy requirements defined for fading channel.

SL PRS-RSRP(P) measurement delay TCs

Regarding the SL PRS-RSRP(P) measurement delay TC, as discussed above, since the SL PRS-RSRP(P) is requested with other measurement methods together, it is reasonable to verify the measurement delay of both SL PRS-RSRP(P) and legacy requirements in one TC in a separate clause defined for SL PRS-RSRP(P) measurement TCs.

Proposal 3: RAN4 to verify measurement delay of the legacy measurements and SL PRS-RSRP(P) in one test case in a separate clause.

A draft CR is also submitted to capture the SL PRS-RSRP(P) measurement delay and accuracy TCs [5].

Conclusions

This paper discussed the accuracy requirements for Sidelink positioning, and the following proposals are provided:

Observation 1: SL PRS-RSRP(P) measurements are always requested with other types of measurements (i.e., SL RSTD/Rx-Tx/AoA).

Proposal 1: RAN4 to verify measurement accuracy of the legacy measurements and SL PRS-RSRP(P) in one test case in a separate clause.

Proposal 2: Measurement accuracy of the legacy measurements in SL PRS-RSRPP TC is expected to fulfil the measurement accuracy requirements defined for fading channel.

Proposal 3: RAN4 to verify measurement delay of the legacy measurements and SL PRS-RSRP(P) in one test case in a separate clause.

References

[1] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, RAN4#111.

[2] R4-2411003 (R1-2405511), Reply LS on SL positioning measurement, RAN1#117.

[3] R4-2411010 (R2-2405872), Reply LS on SL positioning measurements, RAN2#126.

[4] TS 38.355, V18.2.0.

[5] R4-2411334, Draft CR on SL PRS-RSRP(P) measurement delay and accuracy test cases in FR1, CATT, RAN4#112.