3GPP RAN WG4 Meeting #112	R4-2411338

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on Performance requirements of RedCap UE positioning

Source: 	CATT

Agenda item:	6.1.2.4

Document for:	Discussion

Introduction

In RAN4#111 meeting, fruitful progress regarding the performance requirements was achieved which was captured in the WF [1]. In this paper, we will further discuss a few open issues related to performance requirements.

Discussion

Measurement accuracy requirements with Rx FH

Issue1: Bandwidths configurations for accuracy requirements with Rx FH

In RAN4#111 meeting, parameters that shall be captured in the measurement accuracy requirements are further refined which are shown below:

Bandwidth configuration for accuracy requirement for positioning measurement with RX FH for RedCap positioning Agreement: - Define accuracy requirements for FH case based on at least following BW groups: - 15kHz: per-hop BW ≥ 52 RB, min total BW = [268] RB - 30kHz: per-hop BW ≥ 52 RB, min total BW = [272] RB - 60kHz (FR1): per-hop BW ≥ 24 RB, min total BW = [132] RB - 60kHz (FR2): per-hop BW ≥ 68 RB, min total BW = [264] RB - 120kHz: per-hop BW ≥ 68 RB, min total BW = [264] RB - FFS: additional BW configurations for accuracy requirements, e.g., smaller total BW. Note: per-hop BW is to be converted in the accuracy requirements to BW in MHz and aligned with RAN2 signaling. - For accuracy requirements for FH case, derive accuracy numbers from simulation results. - The requirements apply, provided that the BWtotal defined in clause core requirements is no less than the total BW above.

An open issue is whether to consider other BW configurations in the measurement accuracy requirements besides those captured in the table above. It was proposed that some smaller total BW configurations should be considered since it is not clear that how the current requirements together with above agreement will cover all general scenarios.

For example, given that the SCS is 15 kHz and per-hop BW is 52 RBs without overlapping PRBs, a UE performing positioning measurements with three times hopping will give a 30M total BW. According to the current agreement, the new measurement accuracy requirements are not applicable for such case which also exists for other implementations.

Observation 1: The measurement accuracy requirements are not clear for the cases when the new accuracy requirements are not applicable due to total BWs smaller than the minimum total BWs.

Therefore it is necessary to clarify the measurement accuracy requirements for the cases mentioned above, and the solution will be provided after discussing the following FFS:

Accuracy requirement for RedCap positioning Agreement: - Accuracy requirements with Rx FH are different (more accurate than) from accuracy requirements without Rx FH for the scenario where the total PRS BW after all hops is larger than the PRS BW without Rx FH. - FFS: With some BW configurations, accuracy requirements can be same as legacy for the cases with and without Rx FH for the scenario where the total PRS BW after all hops is equal to the PRS BW without Rx FH, if such exist.

Apparently the first bullet is true since the minimum total BWs in the agreement above are larger than the largest BWs in the legacy measurement accuracy requirements.

For the second bullet, it has been discussed in the above example when the new measurement accuracy requirements are not applicable due to the total hopping BW is smaller than the minimum total BW. Considering the time budget and workload of new simulations, it is not realistic to do simulations for new combinations. A possible solution is to reuse existing measurement accuracy requirements or the measurement accuracy requirements without FH, i.e., when the new measurement accuracy requirements are not applicable due to small total BWs, the measurement results shall satisfy the accuracy value of the same or the closet smaller BW defined in the legacy measurement accuracy requirements without FH.

Proposal 1: The measurement results shall satisfy the accuracy value of the same or the closet smaller BW defined in the legacy measurement accuracy requirements without FH, when the new accuracy requirements are not applicable because total hopping BWs are smaller than the minimum total BWs.

Regarding the structure of the measurement accuracy requirements, one example for FR1 is given below for alignment which also follows the agreement above:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥10 | ≥ 50 |
| TBD |  | 30 | ≥20 | ≥ 100 |
| TBD |  | 60 | ≥20 | ≥ 100 |

Similarly, an example for the accuracy table in FR2 is shown below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥50 | ≥ 200 |
| TBD |  | 120 | ≥100 | ≥ 400 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

Proposal 2: The structure of accuracy tables for measurements with FH in different clauses shall be aligned, and example tables for accuracy requirements with FH for 2Rx RedCap in FR1 and FR2 are given below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥10 | ≥ 50 |
| TBD |  | 30 | ≥20 | ≥ 100 |
| TBD |  | 60 | ≥20 | ≥ 100 |

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥50 | ≥ 200 |
| TBD |  | 120 | ≥100 | ≥ 400 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

Conclusions

This paper discussed the accuracy requirements for RedCap UE positioning, and the following proposals are provided:

Observation 1: The measurement accuracy requirements are not clear for the cases when the new accuracy requirements are not applicable due to total BWs smaller than the minimum total BWs.

Proposal 1: The measurement results shall satisfy the accuracy value of the same or the closet smaller BW defined in the legacy measurement accuracy requirements without FH, when the new accuracy requirements are not applicable because total hopping BWs are smaller than the minimum total BWs.

Proposal 2: The structure of accuracy tables for measurements with FH in different clauses shall be aligned, and example tables for accuracy requirements with FH for 2Rx RedCap in FR1 and FR2 are given below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥10 | ≥ 50 |
| TBD |  | 30 | ≥20 | ≥ 100 |
| TBD |  | 60 | ≥20 | ≥ 100 |

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | MHz |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥50 | ≥ 200 |
| TBD |  | 120 | ≥100 | ≥ 400 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

References

[1] R4-2410353, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#111.