3GPP RAN WG4 Meeting #112	R4-2411340

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on general aspects for AIML for NR air

Source: 	CATT

Agenda item:	8.17.1

Document for:	Discussion

Introduction

In RAN4#111 meeting, RAN4 discussed the Rel-19 AI/ML for NR air interface WI and agreements were captured in [1]. And in RAN#104, the AI/ML logistics related issues were also discussed and some issues need RAN4 discussions. In this paper, we will present our views on some open issues.

Discussion

AI/ML logistics

In RAN#104 meeting, AI/ML logistics related issues were discussed and conclusions were captured in [2]. According to the WF, there are some issues for RAN4 discussions and the conclusions will be reported to Sept. RAN meeting. Though RAN4 already had email discussions and a meeting with MCC, they were unofficial and only to share and align information. Therefore we would like to give more analysis on AI/ML logistics and hope some agreements can be made to further facilitate the RAN4 discussions and preparation of AI/ML logistics by MCC.

RAN#104: Way forward: - RAN4 AI/ML experts to identify requirements and capabilities for sharing data like, - an initial list of data to be shared (e.g. data type, possible formats, size, etc). - needed capabilities for the tool ( e.g., versioning of the datasets/models, possibility to tag, etc). - List to be shared with MCC to discuss if any of the existing tools (e.g. FORGE) can be used or there is a need for another tool (provided/administered by MCC). - Follow-up depending on MCC feedback and RAN4 needs. - Progress to be reported to RAN plenary.

Regarding the size of datasets, we take the CSI compression feedback use case as an example. The information of one dataset for CSI compression feedback is shown below:

Dataset format: - (Eigenvector/raw channel, raw output of encoder) Assumptions for data: - Number of data entries: 114000 - Data format: [channel, subband, port] = [2, 13, 32] - Quantization method: 2-bit scalar quantization - Total feedback bits: 120 bits - Data type: float32 (4 Bytes)

According to the assumptions, the estimated size of input data (eigenvector) is:

Size of total input data=114000*2*13*32*4=361.82 MB

Size of the raw output data (latent messages) is:

Size of total latent messages=114000*60 / 8=0.82 MB

Therefore for a given dataset, a storage size of hundreds of MBs is required, which is similar to the views brought up in the reflectors. Considering all sub-use cases, scenarios/configurations and the number of interested companies, the required storage size for datasets is dozens to hundreds of GBs if these companies update the datasets each time.

Observation 1: For a given dataset, at least hundreds of MBs is required for storage.

Observation 2: Dozens to hundreds of GBs are required for dataset storage every time when all datasets are updated.

But in our opinion, the datasets for calibration and discussion purpose do not need to be stored after the WI is closed, while only the datasets that will be put into the spec need. Therefore the demand on the storage size may not be a quite challenging issue, even the versioning is considered. One thing worthy of consideration is the bandwidth and concurrent download and upload, which requires more study from MCC.

Proposal 1: Only the datasets that are agreed to be put into the specifications are stored for a long term. Those for calibrations and discussions are stored temporarily.

Proposal 2: Large bandwidth may be needed for efficient download/upload, and concurrent download/upload may appear, which requires more study from MCC.

Furthermore, information about the stored datasets should be provided to make them usable, i.e., readme file. From our understanding, at least the following information is necessary:

- Scenarios

- Configurations

- Dataset format

- Data type

Proposal 3: Information in a readme file about the stored datasets at least includes:

- Scenarios

- Configurations

- Dataset format

- Data type

Storing AI/ML model is a less challenging issue, since the AI/ML models for BM/Positioning/CSI use cases usually are not very large, around 10 to 100 MB. And we also notice that the size of a Pytorch file (.pt) which includes model structure and parameters is around 1 MB. A similar file size applies for tensorflow file. Besides, a brief description of the AI/ML model functions should be provided and stored together with the AI/ML models.

Observation 3: Less storage size is required if platform files are shared, e.g., Pytorch/tensorflow files that include model structure and parameters.

Proposal 4: A brief description of the AI/ML model functions should be stored together with the AI/ML models.

As for the needed capabilities for the tool, we think the quality of datasets/models should be validated automatically. For example, it is necessary to check if the dimensions of shared datasets match the specified dimensions, if specified, and if the datasets contain any invalid value, e.g., NAN, Inf. Regarding the versioning of datasets/models, we do not have a strong view, and whether this function is needed or not depends on the demands. RAN4 can further check the demands and not to preclude it at current stage.

Proposal 5: The tools for storage should be capable of checking the validation of datasets and models, e.g., the following issues:

- Dimensions mismatch.

- Invalid values, e.g., NAN, Inf.

- Performance of AI/ML models, e.g., SGCS, is lower than a given threshold.

- Others.

Proposal 6: RAN4 further check the demands on versioning of datasets/models and not to preclude this function at current stage.

Another issue we want to discuss is about the AI/ML model sharing. Currently AI/ML models can be developed using different platforms, but an AI/ML model file generated by one platform can only be loaded by this specific platform. An intermediate file is required to achieve such purpose, e.g., ONNX. Figure 1 shows the mechanism of inter-platform sharing of AI/ML models via ONNX.

Figure1. Inter-platform AI/ML model sharing via ONNX

But there are some issues that may affect the performance of AI/ML models, such as

- Resolution loss.

- Some operations are not supported by ONNX.

- Compatibility/version mismatch.

Therefore RAN4 may need study the impacts of the above issues to check if ONNX is usable.

Proposal 7: More efforts are required to study the schemes of AI/ML model sharing among different platform. And the impacts of the following issues may need study:

- Resolution loss.

- Some operations are not supported by intermediate tools, e.g., ONNX.

- Compatibility/version mismatch.

Conclusions

This paper discussed the general issues related to AI/ML for NR air interface, and following observations and proposals are provided:

Observation 1: For a given dataset, at least hundreds of MBs is required for storage.

Observation 2: Dozens to hundreds of GBs are required for dataset storage every time when all datasets are updated.

Observation 3: Less storage size is required if platform files are shared, e.g., Pytorch/tensorflow files that include model structure and parameters.

Proposal 1: Only the datasets that are agreed to be put into the specifications are stored for a long term. Those for calibrations and discussions are stored temporarily.

Proposal 2: Large bandwidth may be needed for efficient download/upload, and concurrent download/upload may appear, which requires more study from MCC.

Proposal 3: Information in a readme file about the stored datasets at least includes:

- Scenarios

- Configurations

- Dataset format

- Data type.

Proposal 4: A brief description of the AI/ML model functions should be stored together with the AI/ML models.

Proposal 5: The tools for storage should be capable of checking the validation of datasets and models, e.g., the following issues:

- Dimensions mismatch.

- Invalid values, e.g., NAN, Inf.

- Performance of AI/ML models, e.g., SGCS, is lower than a given threshold.

- Others.

Proposal 6: RAN4 further check the demands on versioning of datasets/models and not to preclude this function at current stage.

Proposal 7: More efforts are required to study the schemes of AI/ML model sharing among different platform. And the impacts of the following issues may need study:

- Resolution loss.

- Some operations are not supported by intermediate tools, e.g., ONNX.

- Compatibility/version mismatch.

References

[1] R4-2410570, WF on AI/ML, Qualcomm Incorporated, RAN4#111.

[2] RP-241615, AI/ML Logistics, Qualcomm Incorporated, RAN#104.