3GPP RAN WG4 Meeting #112	R4-2411341

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on testability and interoperability issues for BM

Source: 	CATT

Agenda item:	8.17.2

Document for:	Discussion

Introduction

In RAN4#111 meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. There are still some issues related to testability and interoperability for beam management that needs to be further discussed. In this contribution, we will present our views on the following issues:

-	Metrics/KPIs for BM

-	RSRP prediction accuracy

-	Datasets for training/testing

-	Measurement error impact

-	UE reporting for network side model

Discussion

Metrics/KPIs for BM

Metrics/KPIs for beam management were not discussed in previous RAN4 meeting due to limited time allocation. The candidate options are summarized below for reference:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

In RAN1#117 meeting, no agreements were made on the report content of inference results for UE-sided model use case and only one working assumption was confirmed, which is not very helpful for RAN4 to make any down-selection. From our understanding, the test metrics/KPIs are highly related to the monitoring metrics which are under discussions in RAN1, and it is better to use one metric/KPI for both test and monitoring. If two different metrics/KPIs are defined for test and monitoring separately, the AI/ML models may pass the tests but perform badly in deployments. Therefore RAN4 should wait for RAN1’s progress on performance monitoring.

RAN1#117: Agreement Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Proposal 1: RAN4 not to down select any metric/KPI for BM use case until RAN1 have progress on performance monitoring metric.

RSRP prediction accuracy

Though more RAN1 progress is required, it is beneficial to refine the options to align the understandings, especially the mechanism of RSRP prediction accuracy. The proposed options are reproduced below for reference:

Issue 2-2: RSRP Prediction accuracy - Proposals - Option 1: Difference between the predicted RSRP and the measured RSRP of the same Tx beam - Option 2: Difference between the predicted RSRP and the ideal RSRP of the same Tx beam - Option 3: Difference between the predicted RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 4: Difference between the predicted RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 5: RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K - Option 6: others

Firstly, Option 2 and Option 3 mentioned the ideal RSRP but no feasible solutions were given. From our understanding, it is quite challenging to obtain the ideal RSRP during the tests. Therefore discussions on Option 2 and Option 3 can be postponed unless feasible solutions are provided.

Proposal 2: Discussions on Option 2 and Option 3 can be postponed unless feasible solutions are provided.

Secondly, Option 1 is clear and straightforward. An example is as follows:

Figure1. An example of Option 1

Figure 1 assumes that Top-4 strongest predicted beams are reported and these beams are transmitted and then measured by UE. The RSRP difference can be calculated based on the measured RSRP and predicted RSRP of each beam.

Thirdly, Option 4 and 5 have the same logic that both options target the real strongest beam. In Option 4 the RSRP difference is obtained by comparing the RSRP of the Top-1 predicted beam and the RSRP of the real strongest beam, while in Option 5 the predicted Top-K beams and the real strongest beam need to be transmitted for measurements and the RSRP difference is calculated based on the measured RSRPs. Examples of Option 4 and 5 are drawn below where K=3 for Option 5:

Figure2. Examples of Option 4 and 5

According to the Figure1 and Figure2, a difference between Option 1 and Option 4/5 is the RSRP difference may be obtained based on different beams in Option 4/5. But as we discussed in clause 2.1, it is not appropriate to preclude any KPI/metric considering RAN1 have not made any agreement on performance monitoring metric which, in our opinion, affects the test KPI/metric discussion. Therefore we propose to further discuss Option 1/4/5 down-selection can be made when RAN1 have agreements on performance monitoring metric.

Proposal 3: RAN4 continue to discuss Option 1/4/5 and down-selection can be made when RAN1 have agreements on performance monitoring metric.

Datasets for training/testing

In last meeting, datasets for training/testing were also discussed with agreements achieved. A FFS is to discuss the how to augment training data to avoid over-fitting of UE models to the test environment.

RAN4#111 meeting: Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

Based on the agreement, training data will be generated by UE vendors according to the knowledge of test environments and configurations, and UE vendors train their own AI/ML models before taking tests. Regarding the over-fitting to the test environment, whether it will appear or not depends on training procedures. From our understanding, nothing during the above training phases can be controlled by RAN4. What RAN4 could do is to provide UE vendors with enough knowledge of test environments and configurations which are made similar to field deployment conditions as much as possible as the agreement states. Therefore RAN4 does not need to discuss how to augment the training data to avoid over-fitting and it is left to UE implementations.

Proposal 4: Augmenting training data to avoid over-fitting to the test environment is left to UE implementation.

In regard to the dataset for testing, we do not see the necessity to define it, since UE measures the signals transmitted by TE and infers beam IDs and RSRP results which are reported for performance evaluation in TE during the test. Hence, RAN4 not to define dataset for testing unless there is a motivation.

Proposal 5: RAN4 not to define dataset for testing unless the motivation is clarified.

Measurement error impact

The impact of measurement error on the inference accuracy was proposed to be studied in the last meeting and options are as follows:

Issue 2-8: Measurement error impact - Proposals - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now - Option 3: No need to evaluate the measurement error impact

After reviewing TR 38.843, we found that the impact of measurement error on the inference accuracy had been studied in RAN1, and the detailed evaluation results were captured in Clause 6.3.2.3. Therefore RAN4 does not need to repeat the study again and RAN4 can refer to the results when defining performance requirements.

Observation 1: RAN1 studied the impact of measurement error on the inference accuracy in SI phase.

Proposal 6: RAN4 not to repeat the RAN1 study on the impact of measurement error.

Since the impact of measurement error on the inference accuracy is mentioned, two possible solutions for defining performance requirements are summarized for reference:

Alt 1: A margin is added to the accuracy requirements which are derived based on datasets without error or to the existing accuracy requirements if it is agreed to reuse the existing ones.

Alt 2: The accuracy requirements are newly defined based on the datasets with error.

From our standing, either solution can include the impact of the measurement error on the inference accuracy. The margin in Alt 1 can refer to the RAN1 evaluation results.

Proposal 7: Two solutions can be considered for defining performance requirements:

Alt 1: A margin is added to the accuracy requirements which are derived based on datasets without error or to the existing accuracy requirements if it is agreed to reuse the existing ones. The margin can refer to the RAN1 evaluation results.

Alt 2: The accuracy requirements are newly defined based on the datasets with error.

UE reporting for network side model

One proposal was brought up to introduce a different reporting scheme to help train the network side models. It might be helpful to enable the AI/ML features, but it is out of RAN4 scope. Therefore RAN4 shall not discuss this issue.

Proposal 8: RAN4 not to discuss to introduce a different reporting scheme.

Conclusions

This paper discussed some issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: RAN4 not to down select any metric/KPI for BM use case until RAN1 have progress on performance monitoring metric.

Proposal 2: Discussions on Option 2 and Option 3 can be postponed unless feasible solutions are provided.

Proposal 3: RAN4 continue to discuss Option 1/4/5 and down-selection can be made when RAN1 have agreements on performance monitoring metric.

Proposal 4: Augmenting training data to avoid over-fitting to the test environment is left to UE implementation.

Proposal 5: RAN4 not to define dataset for testing unless the motivation is clarified.

Observation 1: RAN1 studied the impact of measurement error on the inference accuracy in SI phase.

Proposal 6: RAN4 not to repeat the RAN1 study on the impact of measurement error.

Proposal 7: Two solutions can be considered for defining performance requirements:

Alt 1: A margin is added to the accuracy requirements which are derived based on datasets without error or to the existing accuracy requirements if it is agreed to reuse the existing ones. The margin can refer to the RAN1 evaluation results.

Alt 2: The accuracy requirements are newly defined based on the datasets with error.

Proposal 8: RAN4 not to discuss to introduce a different reporting scheme.

References

[1] R4-2410570, WF on AI/ML, Qualcomm Incorporated, RAN4#111.

[2] R4-2408944, Topic summary for [111][133] NR_AIML_air, QC, RAN4#111.