3GPP RAN WG4 Meeting #112	R4-2411343

Maastricht, NL, August 19 –August 23, 2024

Title: 	Discussion on testability and interoperability issues for CSI enh

Source: 	CATT

Agenda item:	8.17.4

Document for:	Discussion

Introduction

In RAN4#111 meeting, RAN4 discussed Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But there are still some issues related to testability and interoperability that need to be further discussed. In this contribution, we will present our views on the following issues:

-	Simulation analysis for Option 3.

-	Option 4 for 2-sided model.

-	TE decoder verification/validation.

Discussion

RAN4 have spent a lot of time discussing the derivation of test decoder for AI/ML based CSI compression feedback. There were four options and two were already down selected. Currently RAN4 focus on Option 3 and Option 4 which will respectively define a trained test decoder in spec and some key factors that help TE vendors flexibly implement a test decoder.

Simulation analysis for Option 3

The procedure that describes how to standardize a test decoder was discussed in multiple RAN4 meetings and a flow chart for guidance was provided. Steps at the beginning of the flow chart require simulations to align the performance, and the simulation assumptions were agreed in the last RAN4 meeting [1]. Besides, CNN was prioritized for evaluation according to the agreements and the model structure was also aligned [2]. Therefore we did some simulations and the analysis of the results is given below.

Firstly, the SGCS of CNN model is simulated based on the agreed simulation assumptions as Figure 1 shows.

Figure1. Performance of CNN based CSI compression

Note that the legends, i.e., 128 and 64, in Figure 1 represent the number of kernels in the decoder.

Observation 1: The SGCS of CNN based CSI compression is 75% with 64-bit feedback and 128 kernels in the decoder.

Observation 2: 4%~6% SGCS degradation is observed when the number of kernels in the decoder decreases from 128 to 64.

Furthermore, we analysed the complexity and the size of the decoder, and the results are shown in Figure 2 (a) and (b) respectively.

Figure2. (a) Complexity of the decoder                     (b) Size of the decoder

Observation 3: Significant decreases are observed in complexity and number of parameters when the number of kernels in the decoder decreases from 128 to 64.

Based on the figures, we notice that the complexity and size of decoder can be decreased significantly by reducing the number of kernels in the decoder with a limited SGCS degradation. Therefore RAN4 can consider fewer kernels when defining a test decoder, if Option 3 is agreed eventually.

Proposal 1: Fewer kernels can be considered if RAN4 agree to define test decoders.

Option 4 for 2-sided model

In last meeting, some proposals were brought up to discuss how to study the Option 4 and they were quite divergent [3]. In this clause, we present our views on this issue. To make the analysis easier to understand, a simple diagram for CSI compression feedback is illustrated in Figure 3.

Figure3. Diagram for CSI compression feedback

First, a general principle to study Option 4 is that the RAN4 agreements should not limit the implementation flexibility of encoders. Hence there is no need to specify any factor related to encoders. But this does not mean reference encoder, which might be helpful, is not allowed. If reference encoders are defined, UE vendors’ implementations do not need to be the same as the reference encoders.

Proposal 2: It is not the first priority to define reference encoders, including backbone and structure.

Next we will analyse the assumptions required for decoder implementation. Since no trained decoder will be captured in spec, TE vendors need enough information to implement a decoder that functions properly during tests. Hence finding the necessary information is the key task in study on Option 4.

Let us treat the decoder as a black box and focus on the input and output. According to Figure 3, the input of decoder is the result after bit stream is de-quantized, the length of which depends on the output of encoder. For example, if the number of the latent message generated by an encoder is 60, the length of the input to decoder is also 60. Obviously, it is necessary to define the length of latent message.

Observation 4: The length of latent message is decided by encoder output which affects the implementation of decoders.

We also notice that there are some options proposing to specify the length of bit stream. From our understanding, quantization schemes are also UE implementations which could result in different lengths of bit stream. Therefore it may not be reasonable to choose a fixed value for the length of bit stream. And a merit to specify the length of latent messages instead of bit streams is UE vendors can flexibly choose the quantization methods.

A new issue caused by flexible quantization implementations is quantization scheme sharing between UE and TE which also exists in field. To resolve this issue in tests, two solutions can be considered:

- RAN4 specify the quantization scheme in test cases.

- Follow RAN1 design on quantization scheme matching, if any.

Proposal 3: RAN4 specify the length of latent messages instead of bit stream.

Proposal 4: Quantization scheme adopted by UE needs to be shared to TE. Two solutions can be considered:

(1) Specify a quantization scheme in test cases, or

(2) Follow RAN1 design on quantization scheme matching, if defined.

After decoder infers the output data based on the reported results, TE needs to evaluate the performance of the encoder according to KPIs/metrics. In Figure 3, the performance metric is SGCS which is derived based on the input data of encoder and the output of decoder. However, the input data, which can be raw channel or eigenvectors, is obtained by UE measuring the channel and UE also needs to report this input data to TE for performance evaluation.

Proposal 5: UE under tests needs to report the input data of encoder to TE for performance evaluation.

The most important issue 4 is to align the understandings of UE and TE vendors in channel characteristics and make the encoders developed by UE vendors and decoders developed by TE vendors work together without jointly training. In order to achieve this purpose, some options were already proposed, for example, datasets are specified or a channel generation method is captured in the RAN4 spec.

From our understanding, it is premature to make a conclusion at current stage and more analysis is required. The final solution may depend on the study achievements via simulations. A high-level procedure is provided below for reference to study the feasibility of Option 4 and some conclusions obtained in Option 3 study can be reused.

Figure4. A high-level procedure of the feasibility study on Option 3 and 4

According to the Step 1 in the right side, if companies achieve a similar evaluation result based on what RAN4 specify now, i.e., model backbone/structure and simulations assumptions based on which the training/testing data are generated, it means these information are enough for UE and TE vendors to develop and train their own models respectively, and an expected test performance can be achieved. If so, RAN4 does not need to spend more efforts on defining reference models and datasets, etc. And RAN4 can further study whether more flexibility can be left to TE/UE vendors.

But if the results cannot be aligned, RAN4 can try to fix more necessary parameters to evaluate the feasibility of Option 4, including defining the channel generation method or datasets generated based on test scenarios and configurations or reference encoder/decoder.

Proposal 6: Companies to run simulations with their own trained encoder and the chosen decoder and report the results for alignment. If results are not aligned, RAN4 to find a solution to specify channel characteristics, the following methods can be considered:

- Capture the channel generation method into spec, and/or

- Define datasets based on test scenarios and configurations, and/or

- Other solutions, e.g., reference encoder/decoder.

In summary, RAN4 at least specify the following parameters for Option 4:

- Length of latent messages

- Channel generation method or datasets based on the test scenarios and configurations

Other parameters can be further discussed.

And during tests, the following steps should be paid attend to:

- Quantization scheme matching between UE and TE.

- A need for UE to report the input data of the encoder to TE for performance evaluation.

At last, the reference encoder/decoder might be helpful, but it, in our opinion, is the first priority.

TE decoder verification/validation

Some companies proposed that the AI/ML models implemented by TE vendors should be verified before UE vendors take tests in Option 4. All possible options were summarized in the last meeting, but no agreements were made.

Issue 4-6: TE decoder verification/validation - Proposals - Option 1: RAN4 will have to come up with a TE verification/validation procedure - details are FFS - Option 2: This should be left to RAN5 - Option 3: TE verification/validation is not needed - Option 4: other options

We fully understand the motivation why they wanted to discuss this issue and we agree the quality of TE is important. But, from my understanding, evaluating the quality of TE implementations is not in RAN4 scope, and I believe RAN5 have or will design mature procedures to ensure the AI/ML based test results are fair.

Proposal 7: RAN4 not to discuss the TE verification/validation.

Conclusions

This contribution discussed some issues related to AI/ML for CSI feedback enhancement, and some observations and proposals are provided:

Observation 1: The SGCS of CNN based CSI compression is 75% with 64-bit feedback and 128 kernels in the decoder.

Observation 2: 4%~6% SGCS degradation is observed when the number of kernels in the decoder decreases from 128 to 64.

Observation 3: Significant decreases are observed in complexity and number of parameters when the number of kernels in the decoder decreases from 128 to 64.

Proposal 1: Fewer kernels can be considered if RAN4 agree to define test decoders.

Proposal 2: It is not the first priority to define reference encoders, including backbone and structure.

Observation 4: The length of latent message is decided by encoder output which affects the implementation of decoders.

Proposal 3: RAN4 specify the length of latent messages instead of bit stream.

Proposal 4: Quantization scheme adopted by UE needs to be shared to TE. Two solutions can be considered:

(1) Specify a quantization scheme in test cases, or

(2) Follow RAN1 design on quantization scheme matching, if defined.

Proposal 5: UE under tests needs to report the input data of encoder to TE for performance evaluation.

Proposal 6: Companies to run simulations with their own trained encoder and the chosen decoder and report the results for alignment. If results are not aligned, RAN4 to find a solution to specify channel characteristics, the following methods can be considered:

- Capture the channel generation method into spec, and/or

- Define datasets based on test scenarios and configurations, and/or

- Other solutions, e.g., reference encoder/decoder.

Proposal 7: RAN4 not to discuss the TE verification/validation.

References

[1] R4-2410570, WF on AI/ML, Qualcomm Incorporated, RAN4#111.

[2] AI/ML Model for Option 3 feasibility study, Qualcomm Incorporated, RAN4#111.

[3] R4-2408944, Topic summary for [111][133], NR_AIML_air, RAN4#111.