3GPP RAN WG4 Meeting #112	R4-2411344

Maastricht, NL, August 19 – August 23, 2024

Title: 	Views on SBFD RRM requirements

Source: 	CATT

Agenda item:	8.19.3

Document for:	Discussion

Introduction

In RAN#104 meeting, the scope of R19 SBFD is updated. The objectives that have RAN4 impacts are captured below:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we will present our views on the potential impacts on RRM requirements.

Discussion

SBFD impacts on core requirements

CLI measurements due to SBFD

According to the objectives in WID [1], RAN4 need to specify RRM core requirements for CLI handling mechanism and other possible procedures, if identified.

After analyzing the scope of SBFD and legacy CLI requirements, at least the following requirements will be affected for now. Other requirements may also be affected, depending on RAN1 agreements.

- CLI reporting requirements

- CLI measurement period requirements

For CLI report requirements, legacy CLI requirements support periodic, event-triggered periodic and event triggered reporting, while according to the agreements in RAN1#117 meeting [2], the aperiodic reporting is supported by SBFD, and more discussions are needed on the support of periodic and semi-persistent reporting. Hence RAN4 need wait for more RAN1 progress on reporting scheme design.

Observation 1: Aperiodic reporting of CLI measurements is supported by RAN1. Periodic and semi-persistent reporting can be considered.

Proposal 1: More RAN1 progress on reporting design is needed for defining CLI reporting requirements.

As for CLI measurement period requirements, the period is related to the resource configurations. Currently RAN1 has no clear agreements on how to configure the resources for CLI measurements. Therefore RAN4 also need wait for more RAN1 progress.

Proposal 2: More RAN1 progress on resource configurations for CLI measurements is needed for defining CLI measurement period requirements.

The CLI measurement requirements in RRM specification are introduced in R16. And based on the above analysis which does not show significant impacts on CLI requirements, therefore the R16 legacy CLI requirements, in our understanding, can be used as a baseline for defining CLI requirements for SBFD operation.

Proposal 3: Legacy CLI requirements can be used as a baseline for defining CLI requirements for SBFD.

Scheduling restrictions and interruptions due to SBFD

Regarding scheduling restrictions and interruptions, RAN4 need to wait for more RAN1 agreements on SBFD in CA scenarios, since RAN1 have been discussing single carrier scenarios. But we would like to initially analyse the possible scenario where scheduling restrictions and/or interruptions may exist.

Assuming a UE capable of SBFD is configured with two carriers which are activated and synchronous. One carrier is configured with SBFD mode (carrier 2) and the other is non-SBFD carrier (carrier 1). A figure is illustrated below:

Figure1. SBFD in a CA scenario

In this scenario, carrier 1 is not configured SBFD while carrier 2 works in SBFD mode and we further assume that the 3rd and 4th DL OFDM symbols are SBFD symbols. In this case, if UE is scheduled to transmit UL data in SFBD symbols in carrier 2, then whether or not UE can be scheduled to receive DL data in SBFD symbols in carrier 1 needs further RAN1 discussions. If UE cannot receive DL data in carrier 1 in SBFD symbols, then RAN4 need to define scheduling restrictions.

Proposal 4: Defining scheduling restrictions/interruptions needs more RAN1 progress.

SBFD impacts on performance requirements

In this section, we focus on the impacts of SBFD on the measurement accuracy requirements.

According to the WID [1], new reporting quantities will be introduced, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or RS indices, and at least wideband reporting is supported. For RS index reporting, no performance requirements need to be defined. For L1-SRS-RSRP and L1-CLI-RSSI, if legacy CLI measurement accuracy requirements can be reused depends on the resource configurations which have not been discussed in RAN1. Therefore RAN4 need wait for more progress.

Proposal 5: No performance requirements need to be defined for RS index reporting.

Proposal 6: For L1-SRS-RSRP and L1-CLI-RSSI reporting, more RAN1 progress on resource configurations is needed to decide if legacy CLI measurement accuracy requirements can be reused.

Conclusions

This contribution discussed the potential impacts of SBFD on RRM requirements and some proposals are provided:

Observation 1: Aperiodic reporting of CLI measurements is supported by RAN1. Periodic and semi-persistent reporting can be considered.

Proposal 1: More RAN1 progress on reporting design is needed for defining CLI reporting requirements.

Proposal 2: More RAN1 progress on resource configurations for CLI measurements is needed for defining CLI measurement period requirements.

Proposal 3: Legacy CLI requirements can be used as a baseline for defining CLI requirements for SBFD.

Proposal 4: Defining scheduling restrictions/interruptions needs more RAN1 progress.

Proposal 5: No performance requirements need to be defined for RS index reporting.

Proposal 6: For L1-SRS-RSRP and L1-CLI-RSSI reporting, more RAN1 progress on resource configurations is needed to decide if legacy CLI measurement accuracy requirements can be reused.

References

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, RAN#104.

[2] RAN1 chair’s notes RAN1#117 eom1.

[3] 3GPP TR 38.858, Study on Evolution of NR Duplex Operation, V18.1.0.