3GPP TSG-RAN WG4 Meeting # 112	             R4-2411348

# Maastricht, NL, Aug 19 – 23, 2024

Agenda Item:	5.24.1

Source: 	CATT

Title: 	Discussion on RRM core requirements maintenance for LTM

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on L1/L2 inter-cell mobility was approved [1]. In this contribution, some open issues on RRM core requirements maintenance for L1/L2 based inter-cell mobility are further discussed.

2. Discussion

Early Candidate cell’s TCI state activation

Issue 1-3-1: Whether to consider early TCI state activation for multiple cells at the same time

The WF on whether to consider early TCI state activation for multiple cells at the same time in the last meeting is duplicated as follows [1]:

< Way Forward > - Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

Considering that a single candidate cell TCI states activation/deactivation MAC CE can only activate/deactivate TCI states of a single candidate cell, so the early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell. Therefore, we think that it is not necessary to define the early TCI state activation requirements for multiple cells at the same time.

Proposal 1: Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

L1-RSRP measurement requirements

Issue 2-2-1: Measurement period of serving cell L1-RSRP measurement

In the last meeting, the measurement period of serving cell L1-RSRP measurement was discussed and there was no consensus. The Way Forward is duplicated below [1]:

<Way Forward >: Further discuss the following options: - Option 1 (Apple): - introduce NLayer in serving cell L1 RSRP measurement requirement and clarify that it is for UE capable of RTD>CP configured with L1 RSRP measurement on neighbour cell. - Option 2 (OPPO): - Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement. - Option 3 (Ericsson, QC): - RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

The above issue mainly emphasizes there is a misalignment between intra-frequency neighbour cell measurement and serving cell measurement for UE capable of RTD>CP in FR1. Both Option 1 and Option 2 are to align the neighbour cell L1-RSRP measurement with the serving cell L1-RSRP measurement. Option 1 is proposed to follow the logic of neighbour cell L1-RSRP measurement for serving cell, and Option 2 is proposed to follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement for neighbour cell. However, we believe that there is no need to align the behaviour of neighbour cell measurement and serving cell measurement. Since there will always be some cases where UE need to measure the serving cell, but not the neighbour cell, so we prefer to support not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Proposal 2: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Known conditions

Known TCI state conditions

In the last meeting, RAN4 has agreed the related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells in FR1 under certain conditions [1]. Since the current cell switch delay requirements are only applicable to known TCI, and known TCI state conditions are related to L1-RSRP report, RAN4 should further discuss how to make the current cell switch delay requirements applicable to the case without L1 measurement and report.

The Way Forward for known TCI state conditions is as follows [1]:

<Way Forward >: Further discuss the following options in FR1: - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions. - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB - Condition 1: TCI state has been activated and - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command <Way Forward >: Further discuss the following options in FR2: - Condition 3: TCI state has been activated and - Condition 3A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 3B: the TCI state activation command was received not more than 160 ms before the cell switch command - Condition 3C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 3D: - Beam-level L3 measurement results have been reported in 1280ms - L1-RSRP measurement period is no larger than 1280ms in FR2 - Condition 4: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

For Option 1 in FR1, we don’t prefer to change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report. Considering that the known TCI state conditions are also defined in clause 8.25.2 in TS 38.133 [2], we don’t suggest using different definitions for known TCI state conditions in different requirements. For Option 2, if RAN4 allow cell switch delay requirements applicable to unknown TCI state case with conditions, we also can agree to directly clarify all applicable conditions for cell switch delay requirements in the spec, rather than defining known TCI state conditions.

For the conditions when SNR of the TCI state≥ -3dB, we believe that activated TCI state should not be a prerequisite for cell switch delay requirements, so we support that UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command.

Proposal 3: If RAN4 allow cell switch delay requirements applicable to unknown TCI state case with conditions in FR1 and FR2, we also can agree to directly clarify all applicable conditions for cell switch delay requirements in the spec, rather than defining known TCI state conditions.

Proposal 4: For the conditions when SNR of the TCI state≥ -3dB, it is supported that UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command.

UE feature

Optimization and revision on the UE capability

According to the current UE feature list, the capability of supporting SSB based L1-RSRP measurements for multiple cells with RTD > CP is duplicated as follows [3]:

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP. | 45-1 from RAN1 Rel-18 feature list or 39-2 or 39-2a |

In the last meeting, RAN4 has agreed the related LTM requirements are applicable to the case without L1 measurement and reporting of LTM candidate cells in FR1 and replied the LS to RAN2.

For the related LTM requirements defined in RAN4, including the requirements for early TCI state activation, PDCCH ordered RACH and LTM cell switch delay. For the LTM capability, since the capabilities of early T/F tracking and PDCCH order RACH are also related to the capability of handling multiple cells with RTD > CP, the capability of handling multiple cells with RTD > CP also should be decoupled from SSB based L1-RSRP measurements. Otherwise, for the case without SSB based L1-RSRP measurements, there is no capability to indicate whether UE supports RTD>CP case for early T/F tracking and/or PDCCH order RACH.

Observation: For the case without SSB based L1-RSRP measurements, there is no capability to indicate whether UE supports RTD>CP case for early T/F tracking and/or PDCCH order RACH.

Therefore, the current capability of SSB based L1-RSRP measurements for multiple cells with RTD > CP should be revised and it only needs to describe the capability of handling multiple cells with RTD > CP. This capability will be supported together with the capabilities of SSB based L1-RSRP measurements and/or early T/F tracking and/or PDCCH order RACH.

Proposal 5: The current capability of SSB based L1-RSRP measurements for multiple cells with RTD > CP should be revised.

- It only needs to describe the capability of handling multiple cells with RTD > CP.

- This capability will be supported together with the capabilities of SSB based L1-RSRP measurements and/or early T/F tracking and/or PDCCH order RACH.

3. Conclusion

In this paper, we provide our views on RRM core requirements maintenance for L1/L2 based inter-cell mobility. From this discussion we have derived the following observation and proposals:

Proposal 1: Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

Proposal 2: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Proposal 3: If RAN4 allow cell switch delay requirements applicable to unknown TCI state case with conditions in FR1 and FR2, we also can agree to directly clarify all applicable conditions for cell switch delay requirements in the spec, rather than defining known TCI state conditions.

Proposal 4: For the conditions when SNR of the TCI state≥ -3dB, it is supported that UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command.

Observation: For the case without SSB based L1-RSRP measurements, there is no capability to indicate whether UE supports RTD>CP case for early T/F tracking and/or PDCCH order RACH.

Proposal 5: The current capability of SSB based L1-RSRP measurements for multiple cells with RTD > CP should be revised.

- It only needs to describe the capability of handling multiple cells with RTD > CP.

- This capability will be supported together with the capabilities of SSB based L1-RSRP measurements and/or early T/F tracking and/or PDCCH order RACH.

4. Reference

- R4-2410302 WF on NR mobility enhancements (part 1), MediaTek Inc.

- TS 38.133 NR Requirements for support of radio resource management (V18.6.0)

- R4-2406680 Rel-18 RAN4 UE feature list for NR, CMCC