3GPP TSG-RAN WG4 Meeting # 112	R4-2411349

# Maastricht, NL, Aug 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4637 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement report for fast CA/DC setup in RRC_IDLE/RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following requirements of measurement report for fast CA/DC setup in RRC_IDLE/RRC_INACTIVE should be further revised: - 4.7 Measurement report for fast CA/DC setup - 5.8 Measurement report for fast CA/DC setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise the requirements of measurement report for fast CA/DC setup in RRC_IDLE mode. - Revise the requirements of measurement report for fast CA/DC setup in RRC_INACTIVE mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for measurement report for fast CA/DC setup in Idle/ Inactive mode would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 4.7 Measurement report for fast CA/DC setup

### 4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-    UE supporting [Measurement validation based on EMR measurement] and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

-    UE supporting [Measurement validation based on non-EMR measurement] and configured with measReselectionCarrierListNR-r18 higher layers.

A UE supporting [Measurement validation based on EMR measurement] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the idle mode measurement specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3.

A UE supporting [Measurement validation based on non-EMR measurement] shall report based on the idle mode measurement specified in clause 4.2.2, and according to the measurement reporting requirements specified in clause 4.7.3.

UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

### 4.7.2	Void

### 4.7.3	Measurement Report Requirements

A UE shall perform validity check and report valid measurement results:

-    if the UE is supporting [Measurement validation based on EMR measurement] and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16,

-    if the UE is supporting [Measurement validation based on non-EMR measurement] and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18

The measurement results are considered valid if the following conditions are met for the validity check:

-	the measurements are performed before msg1 transmission for RRC resume/setup request within the last:

-	 measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or

-	 measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18,

-	the measurement results satisfy measurement accuracy requirements at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured.

If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirements at the measurement instance.

RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified inclauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

The UE may report measurement results given the measurement results satisfy measurement accuracy requirements at the measurement instance.

<End of Change 1>

<Start of Change 2>

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-    UE supporting [Measurement validation based on EMR measurement] and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

-    UE supporting [Measurement validation based on non-EMR measurement] and configured with measReselectionCarrierListNR-r18 higher layers.

A UE supporting [Measurement validation based on EMR measurement] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the inactive mode measurement specified in clause 5.4 , and according to the measurement reporting requirements specified in clause 5.8.3.

A UE supporting [Measurement validation based on non-EMR measurement] shall report based on the inactive-mode cell re-selection requirements specified in clause 5.1.2, and according to the measurement reporting requirements specified inclause 5.8.3.

UE is not required to report for fast CA/DC setup from any measurements during INACTIVE mode for carriers which are not configured by higher layers.

### 5.8.2	Void

### 5.8.3	Measurement Report Requirements

The requirements in clause 4.7.3 shall apply.

<End of Change 2>