# 3GPP TSG-RAN WG4 Meeting #112				                                               R4-2411351

# Maastricht , NL, Aug 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR | 4639 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on eDRX requirements in INACTIVE mode for eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The numbering of reference 3GPP TS 24.008 is wrong and the reference 3GPP TS 24.008 is missed in Section 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new reference 3GPP TS 24.008 cited by the eDRX requirements for eRedCap UE. - Revise the numbering of reference 3GPP TS 24.008 from ‘[34]’ to ‘[42]’ for eRedCap UE in INACTIVE mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference of eDRX requirements in INACTIVE mode for (e)RedCap UE would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.2.2, 5.1.2.3, 5.1.2.4, 5.1.2.5, 5.1B.2.2, 5.1B.2.3, 5.1B.2.4, 5.1B.2.5, 5.1B.2.9, 5.1B.2.10, 5.1B.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[4]	3GPP TS 38.215: "NR; Physical layer measurements".

[5]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)".

[6]	3GPP TS 38.211: "NR; Physical channels and modulation”.

[7]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[8]	3GPP TS 38.212 "NR; Multiplexing and channel coding".

[9]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer".

[10]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[11]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[12]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[13]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[14]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[15]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[17]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[20]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[21]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[22]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[23]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[24]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA); Overall description".

[25]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[26]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[27]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[28]	Void.

[29]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[30]	3GPP TS 25.302: "Services provided by the Physical Layer".

[31]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and Next Generation Radio Access; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[32]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[33]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[34]	3GPP TS 37.355: "LTE Positioning Protocol (LPP) ".

[35]	3GPP TS 38.455	: "NG-RAN; NR Positioning Protocol A (NRPPa) ".

[36]	3GPP TS 37.106: "User Equipment (UE) requirements for shared spectrum channel access".

[37]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP) ".

[38]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[39]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA): Physical layer procedures"

[40]	3GPP TS 38.521-2: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[41]	3GPP TS 38.108: “NR; Satellite Access Node radio transmission and reception”.

[42]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

<End of Change 1>

<Start of Change 2>

5.1.2.2	Measurement and evaluation of serving cell

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.2 shall apply.

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1* T for FR1 and N1*T for FR2; where:

-	T is determined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.

When UE is configured with eDRX_IDLE and eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, within a single eDRX INACTIVE PTW, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1* T for FR1 and N1*T for FR2; where:

-	T is dertermined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least T/2.

If UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1 and the UE has evaluated according to Table 5.1.2.2-1 or and Table 5.1.2.2-2 in Nserv consecutive T cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1 and the UE has evaluated according to Table 5.1.2.2-3 and Table 5.1.2.2-4 in Nserv consecutive T within a single eDRX INACTIVE PTW that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Table 5.1.2.2-1: Nserv for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Nserv [number of T ] |
| --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 4*M1 |
|  |  | 0.64 | 4*M1 |
|  |  | 1.28 | 2 |
|  |  | 2.56 | 2 |
|  |  | 5.12 | 2 |
|  |  | 10.24 | 2 |
| Note1: T is dertermined according to clause 7.1 in [1]. |  |  |  |

Table 5.1.2.2-2: Nserv for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Scaling Factor (N1) | Nserv [number of T] |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 8 | 4* N1 |
|  |  | 0.64 | 5 | 4* N1 |
|  |  | 1.28 | 4 | 2* N1 |
|  |  | 2.56 | 3 | 2* N1 |
|  |  | 5.12 | 3 | 2* N1 |
|  |  | 10.24 | 3 | 2* N1 |
| Note1: T is dertermined according to clause 7.1 in [1]. |  |  |  |  |

Table 5.1.2.2-3: Nserv for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | T [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Nserv [number of T Note 3] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥ 1.28 (1) | M1*2 |
|  | 0.64 | ≥ 1.28 (1) (M1=1) or ≥ 2.56 (2) (M1=2) | M1*2 |
|  | 1.28 | ≥2.56 (2) | 2 |
|  | 2.56 | ≥5.12 (4) | 2 |
| NOTE 1: eDRX INACTIVE PTW in this table is RAN configured PTW. NOTE 2: Nserv requirements are only defined within a single eDRX INACTIVE PTW duration. NOTE 3: T is determined according to clause 7.1 in [1]. NOTE 4: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 5: The lower bound of PTW length is derived based on Nserv *DRX_cycle1.28*1.28. |  |  |  |

Table 5.1.2.2-4: Nserv for for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR2)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | T [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Nserv [number of T Note 3] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥5.12 (4) | 8 | N1*2 |
|  | 0.64 | ≥6.4 (5) | 5 | N1*2 |
|  | 1.28 | ≥10.24 (8) | 4 | N1*2 |
|  | 2.56 | ≥15.36 (12) | 3 | N1*2 |
| NOTE 1: Applies for UE of all FR2 power class.. NOTE 2: eDRX INACTIVE PTW in this table is RAN configured PTW. NOTE 3: Nserv requirements are only defined within a single eDRX INACTIVE PTW duration. NOTE 4: T is determined according to clause 7.1 in [1]. NOTE 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 6: The lower bound of PTW length is derived based on Nserv *DRX_cycle1.28*1.28. NOTE 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |

If UE is not configured with eDRX_INACTIVE ≥ 20.48s, when UE transitions from measurements within eDRX_IDLE PTW to measurements outside eDRX_IDLE PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement period requirements before or after the transition.

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

If the UE in RRC_INACTIVE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T’, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where:

-	T’= 10 s, if the UE is not configured with eDRX_inactive cycle, or

-	T’= MAX (10 s, one eDRX_INACTIVE cycle) if the UE is configured with eDRX_INACTIVE cycle for FR1, or

-	T’= MAX (10 s, N1* eDRX_INACTIVE cycle) if the UE is configured with eDRX_INACTIVE cycle for FR2.

5.1.2.3	Measurements of intra-frequency NR cells

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.3 shall apply. The requirements in clause 4.2.2.9 apply for UE configured with relaxed measurement criterion when UE is not configured with eDRX_IDLE.

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.1.2.3-1 and Table 5.1.2.3-2.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.1.2.3-3 and Table 5.1.2.3-4.

Table 5.1.2.3-1: Tdetect, NR_Intra, Tmeasure, NR_Intra and Tevaluate, NR_Intra for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 (36 x M2) | 1.28 x M2 (4 x M2) | 5.12 x M2 (16 x M2) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52 (23) | 10.24 (1) | 30.72 (3) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |

Table 5.1.2.3-2: Tdetect,NR_Intra, Tmeasure, NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1.2.3-3: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RANDRX cycle length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra [s] (number of RAN DRX cycles Note 3) | Tevaluate,NR_Intra [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥[1.28] ([1]) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥[1.28] ([1]) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥[2.56] ([2]) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥[5.12] ([4]) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 5: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. Note 6: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1.2.3-4: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR2)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RAN DRX length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra [s] (number of RAN DRX cycles Note 5) | Tevaluate,NR_Intra [s] (number of RAN DRX cycles Note 5) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: Applies for UE of all power class. Note 3: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 4: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 5: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. Note 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

5.1.2.4	Measurements of inter-frequency NR cells

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.4 shall apply with Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in Table 5.1.2.4-1 and Table 5.1.2.4-2, and with Kcarrier defined as follows.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2.2.4 shall apply with Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in Table 5.1.2.4-3 and Table 5.1.2.4-4.

Table 5.1.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1.2.4-2: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

Table 5.1.2.4-3: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 4: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 5.1.2.4-4: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR2)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: Applies for UE of all power class. Note 2: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 3: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. Note 5: When eDRX_INACTIVE=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX _INACTIVE cycles. Note 6: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 7: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 8: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. |  |  |  |  |  |  |

When the UE transitions between any two states when being configured with eDRX_INACTIVE, being configured with eDRX_INACTIVE cycle, changing eDRX_INACTIVE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier in clause 4.2.2.4 shall apply regardless of whether the serving cell is subject to CCA or not.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, Kcarrier is replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply with Kcarrier being replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

5.1.2.5	Measurements of inter-RAT E-UTRAN cells

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.5 shall apply with Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN defined in Table 5.1.2.5-1.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.5 shall apply with Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN  defined in Table 5.1.2.5-2.

Table 5.1.2.5-1: Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN for inactive UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1.2.5-2: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE ≥ 20.48s, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_Inactive cycle length [s] | RAN DRX cycle length [s] | eDRX Inactive PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN [s] (number of RAN DRX cycles Note 3) | Tmeasure,EUTRAN [s] (number of RAN DRX cycles Note 3) | Tevaluate,E-UTRAN [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 2: The number of RAN DRX cycles in this table is given for the RAN DRX cycles within RAN configured PTWs. NOTE 3: eDRX Inactive PTW in this table is RAN configured PTW [1]. NOTE 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 5: The eDRX_INACITVE cycle lengths are ran-ExtendedPagingCycle-r18 as specified in [2] NOTE 6: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN*DRX_cycle1.28*1.28. NOTE 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

<End of Change 2>

<Start of Change 3>

5.1B.2.2	Measurement and evaluation of serving cell

The requirements in clause 4.2B.2.2 shall apply when UE is not configured with eDRX_IDLE.

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1* T for FR1 and N1*T for FR2; where:

-	T is dertermined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.

When UE is configured with eDRX_IDLE and eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, within a single eDRX INACTIVE PTW, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1* T for FR1 and N1*T for FR2; where:

-	T is dertermined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least T/2.

If UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1 and the UE has evaluated according to Table 5.1B.2.2-1or and Table 5.1B.2.2-2 in Nserv_RedCap consecutive T that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1 and the UE has evaluated according to Table 5.1B.2.2-3 and Table 5.1B.2.2-4 in Nserv_RedCap consecutive T within a single eDRX INACTIVE PTW that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Table 5.1B.2.2-1: Nserv_RedCap for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Nserv _RedCap [number of T ] |
| --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 4*M1 |
|  |  | 0.64 | 4*M1 |
|  |  | 1.28 | 2 |
|  |  | 2.56 | 2 |
|  |  | 5.12 | 2 |
|  |  | 10.24 | 2 |
| Note1: T is dertermined according to clause 7.1 in [1]. Note2: M1=2 if SMTC periodicity (TSMTC) > 20 ms and T≤ 0.64 second, otherwise M1=1. |  |  |  |

Table 5.1B.2.2-2: Nserv_RedCap for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Scaling Factor (N1) | Nserv_RedCap [number of T] |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 8 | 4* N1 |
|  |  | 0.64 | 5 | 4* N1 |
|  |  | 1.28 | 4 | 2* N1 |
|  |  | 2.56 | 3 | 2* N1 |
|  |  | 5.12 | 3 | 2* N1 |
|  |  | 10.24 | 3 | 2* N1 |
| Note1: T is dertermined according to clause 7.1 in [1]. |  |  |  |  |

Table 5.1B.2.2-3: Nserv_RedCap for inactive Redcap UE, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | T [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Nserv_RedCap [number of T Note 3] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_ INACTIVE cycle length ≤10485.76 | 0.32 | ≥ 1.28 (1) | M1*2 |
|  | 0.64 | ≥ 1.28 (1) (M1=1) or ≥ 2.56 (2) (M1=2) | M1*2 |
|  | 1.28 | ≥2.56 (2) | 2 |
|  | 2.56 | ≥5.12 (4) | 2 |
| NOTE 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. NOTE 3: eDRX INACTIVE PTW in this table is RAN configured PTW. NOTE 4: Nserv_RedCap requirements are only defined within a single eDRX INACTIVE PTW duration. NOTE 5: T is determined according to clause 7.1 in [1]. NOTE 6: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 7: The lower bound of PTW length is derived based on Nserv_RedCap *DRX_cycle1.28*1.28. |  |  |  |

Table 5.1B.2.2-4: Nserv_RedCap for inactive Redcap UE, (Frequency range FR2)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | T [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Nserv_RedCap [number of T Note 3] |
| --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_ INACTIVE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | N1*2 |
|  | 0.64 | ≥6.4 (5) | 5 | N1*2 |
|  | 1.28 | ≥10.24 (8) | 4 | N1*2 |
|  | 2.56 | ≥15.36 (12) | 3 | N1*2 |
| NOTE 1: Applies for RedCap UE of all FR2 power class. NOTE 2: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 3: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. NOTE 4: eDRX INACTIVE PTW in this table is RAN configured PTW. NOTE 5: Nserv_RedCap requirements are only defined within a single eDRX INACTIVE PTW duration. NOTE 6: T is determined according to clause 7.1 in [1]. NOTE 7: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 8: The lower bound of PTW length is derived based on Nserv_RedCap *DRX_cycle1.28*1.28. NOTE 9: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |

If UE is not configured with eDRX_INACTIVE ≥ 20.48s, when UE transitions from measurements within eDRX_IDLE PTW and to measurements outside eDRX_IDLE PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement period requirements before or after the transition.

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

If the UE in RRC_INACTIVE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T’, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where

-	T’= 10 s, if the UE is not configured with eDRX_inactive cycle, or

-	T’= MAX (10 s, one eDRX_inactive cycle) if the UE is configured with eDRX_inactive cycle for FR1, or

-	T’= MAX (10 s, N1* eDRX_inactive cycle) if the UE is configured with eDRX_inactive cycle for FR2.

5.1B.2.3	Measurements of intra-frequency NR cells

The requirements in clause 4.2B.2.3 shall apply when UE is not configured with eDRX_IDLE. When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2B.2.3 shall apply with Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap defined in Table 5.1B.2.3-1 and Table 5.1B.2.3-2.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.3 shall apply with Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap defined in Table 5.1B.2.3-3 and Table 5.1B.2.3-4.

Table 5.1B.2.3-1: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 (36 x M2) | 1.28 x M2 (4 x M2) | 5.12 x M2 (16 x M2) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52 (23) | 10.24 (1) | 30.72 (3) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |

Table 5.1B.2.3-2: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1B.2.3-3: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RANDRX cycle length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra_RedCap [s] (number of RAN DRX cycles Note 3) | Tevaluate,NR_Intra_RedCap [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥[1.28] ([1]) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥[1.28] ([1]) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥[2.56] ([2]) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥[5.12] ([4]) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 5: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 6: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1B.2.3-4: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR2)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RAN DRX length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_Intra_RedCap [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra_RedCap [s] (number of RAN DRX cycles Note 5) | Tevaluate,NR_Intra_RedCap [s] (number of RAN DRX cycles Note 5) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: Applies for RedCap UE of all power class. Note 3: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 4: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 5: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

5.1B.2.4	Measurements of inter-frequency NR cells

The requirements in clause 4.2B.2.4 shall apply when UE is not configured with eDRX_IDLE. When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2B.2.4 shall apply with Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined in Table 5.1B.2.4-1 and Table 5.1B.2.4-2.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.4 shall apply with Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined in Table 5.1B.2.4-3 and Table 5.1B.2.4-4.

Table 5.1B.2.4-1: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

Table 5.1B.2.4-3: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 4: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 5.1B.2.4-4: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR2)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: Applies for RedCap UE of all power class. Note 2: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 3: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: When eDRX_INACTIVE=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX _ INACTIVE cycles. Note 6: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 7: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 8: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. |  |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells

The requirements in clause 4.2B.2.5 shall apply when UE is not configured with eDRX_IDLE. When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2B.2.5 shall apply with Tdetect, EUTRAN_RedCap, Tmeasure, EUTRAN _RedCap and Tevaluate, EUTRAN _RedCap defined in Table 5.1B.2.5-1.

Table 5.1B.2.5-1: Tdetect, EUTRAN_RedCap, Tmeasure, EUTRAN _RedCap and Tevaluate, EUTRAN _RedCap for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN_RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN _RedCap [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN _RedCap [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.5 shall apply with Tdetect, EUTRAN_RedCap, Tmeasure, EUTRAN _RedCap and Tevaluate, EUTRAN _RedCap defined in Table 5.1B.2.5-2.

Table 5.1B.2.5-2: Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap, and Tevaluate,EUTRAN_RedCap for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE ≥ 20.48s, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_Inactive cycle length [s] | RAN DRX cycle length [s] | eDRX Inactive PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap [s] (number of RAN DRX cycles Note 3) | Tmeasure,EUTRAN_RedCap [s] (number of RAN DRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 2: The number of RAN DRX cycles in this table is given for the RAN DRX cycles within RAN configured PTWs. NOTE 3: eDRX Inactive PTW in this table is RAN configured PTW [1]. NOTE 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 5: The eDRX_INACITVE cycle lengths are ran-ExtendedPagingCycle-r18 as specified in [2] NOTE 6: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. NOTE 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

<End of Change 3>

<Start of Change 4>

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2B.2.9 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion, and

-	when UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, then the requirements in Table Table 5.1B.2.9-1 and Table 5.1B.2.9-2 respectively apply provided that eDRX_IDLE cycle is ≤ 10485.76  sec, or

-	when UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.9 shall apply with Tdetect, NR_intra_RedCap_Relax, Tmeasure, NR_intra _RedCap_Relax and Tevaluate, NR_intra _RedCap_Relax defined in Table 5.1B.2.9-3 and Table 5.1B.2.9-4.

Table 5.1B.2.9-1: detect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for inactive Redcap UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 11.52 x M2 x K4 (36 x M2 x K4) | 1.28 x M2 x K4 (4 x M2 x K4) | 5.12 x M2 x K4 (16 x M2 x K4) |
|  | 0.64 | 17.92 x K4 (28 x K4) | 1.28 x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
|  | 1.28 | 32 x K4 (25 x K4) | 1.28 x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
|  | 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
|  | 5.12 | 117.76 x K4 (23 x K4) | 5.12 x K4 (1 x K4) | 15.36 x K4 (3 x K4) |
|  | 10.24 | 235.52 x K4 (23 x K4) | 10.24 x K4 (1 x K4) | 30.72 x K4 (3 x K4) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. K4 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

Table 5.1B.2.9-2: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 8 | 11.52 x N1 x M2 x K4 (36 x N1 x M2 x K4) | 1.28 x N1 x M2 x K4 (4 x N1 x M2 x K4) | 5.12 x N1 x M2 x K4 (16 x N1 x M2 x K4) |
|  | 0.64 | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
|  | 1.28 | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
|  | 2.56 | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
|  | 5.12 | 3 | 117.76 x N1 x K4 (23 x N1 x K4) | 5.12 x N1 x K4 (1 x N1 x K4) | 15.36 x N1 x K4 (3 x N1 x K4) |
|  | 10.24 | 3 | 235.52 x N1 x K4 (23 x N1 x K4) | 10.24 x N1 x K4 (1 x N1 x K4) | 30.72 x N1 x K4 (3 x N1 x K4) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. K4 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

Table 5.1B.2.9-3: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤163.84 20.48 ≤eDRX_INACTIVE cycle length ≤ 163.84 | 0.32 | ≥1.28 (1) | eDRXcyclelength×23PTWDRXcyclelength×Kx (23 x Kx) | 0.32 x M2 x Kx (1 x 1.5 x Kx) | 0.64 x M2 x Kx (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x Kx (1 x Kx) | 1.28 x Kx (2 x Kx) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x Kx (1 x Kx) | 2.56 x Kx (2 x Kx) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x Kx (1 x Kx) | 5.12 x Kx (2 x Kx) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 4: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 7: Kx = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. Kx = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. Note 8: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1B.2.9-4: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR2)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤163.84 20.48 ≤eDRX_INACTIVE cycle length ≤163.84 | 0.32 | ≥5.12 (4) | 8 | eDRXcyclelength×23×N1PTWDRXcyclelength x Kx (23 x N1 x Kx) | 0.32 x N1 x Kx (1 x N1 x Kx) | 0.64 x N1 x Kx (2 x N1 x Kx) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 x Kx (1 x N1 x Kx) | 1.28 x N1 x Kx (2 x N1 x Kx) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 x Kx (1 x N1 x Kx) | 2.56 x N1 x Kx (2 x N1 x Kx) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 x Kx (1 x N1 x Kx) | 5.12 x N1 x Kx (2 x N1 x Kx) |
| Note 1: Applies for RedCap UE of all power class. Note 2: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 3: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: When eDRX_INACTIVE=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX _INACTIVE cycles. Note 6: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 7: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 8: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 9: Kx = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. Kx = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |  |

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2B.2.10 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion, and

-	when UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, then the requirements in Table Table 5.1B.2.10-1 and Table 5.1B.2.10-2 respectively apply provided that eDRX_IDLE cycle is ≤ 10485.76 sec, or

-	when UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.10 shall apply with Tdetect, NR_inter_RedCap_Relax, Tmeasure, NR_inter _RedCap_Relax and Tevaluate, NR_inter _RedCap_Relax defined in Table 5.1B.2.10-3 and Table 5.1B.2.10-4.

Table 5.1B.2.10-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for inactive Redcap UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 x K4 (36 x M2 x K4) | 1.28 x M2 x K4 (4 x M2 x K4) | 5.12 x M2 x K4 (16 x M2 x K4) |
|  | 0.64 | 17.92 x K4 (28 x K4) | 1.28 x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
|  | 1.28 | 32 x K4 (25 x K4) | 1.28 x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
|  | 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
|  | 5.12 | 117.76 x K4 (23 x K4) | 5.12 x K4 (1 x K4) | 15.36 x K4 (3 x K4) |
|  | 10.24 | 235.52 x K4 (23) | 10.24 x K4 (1 x K4) | 30.72 x K4 (3 x K4) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. K4 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

Table 5.1B.2.10-2: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 8 | 11.52 x N1 x M2 x K4 (36 x N1 x M2 x K4) | 1.28 x N1 x M2 x K4 (4 x N1 x M2 x K4) | 5.12 x N1 x M2 x K4 (16 x N1 x M2 x K4) |
|  | 0.64 | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
|  | 1.28 | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
|  | 2.56 | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
|  | 5.12 | 3 | 117.76 x N1 x K4 (23 x N1 x K4) | 5.12 x N1 x K4 (1 x N1 x K4) | 15.36 x N1 x K4 (3 x N1 x K4) |
|  | 10.24 | 3 | 235.52 x N1 x K4 (23 x N1 x K4) | 10.24 x N1 x K4 (1 x N1 x K4) | 30.72 x N1 x K4 (3 x N1 x K4) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. K4 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

Table 5.1B.2.10-3: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤163.84 20.48 ≤eDRX_INACTIVE cycle length ≤ 163.84 | 0.32 | ≥1.28 (1) | eDRXcyclelength×23PTWDRXcyclelength×Kx (23 x Kx) | 0.32 x 1.5 x Kx (1 x 1.5 x Kx) | 0.64 x 1.5 x Kx (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x Kx (1 x Kx) | 1.28 x Kx (2 x Kx) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x Kx (1 x Kx) | 2.56 x Kx (2 x Kx) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x Kx (1 x Kx) | 5.12 x Kx (2 x Kx) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 4: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 7: Kx = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. Kx = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

Table 5.1B.2.10-4: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for Redcap UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR2)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤163.84 20.48 ≤eDRX_INACTIVE cycle length ≤163.84 | 0.32 | ≥5.12 (4) | 8 | eDRXcyclelength×23×N1PTWDRXcyclelength x Kx (23 x N1 x Kx) | 0.32 x N1 x Kx (1 x N1 x Kx) | 0.64 x N1 x Kx (2 x N1 x Kx) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 x Kx (1 x N1 x Kx) | 1.28 x N1 x Kx (2 x N1 x Kx) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 x Kx (1 x N1 x Kx) | 2.56 x N1 x Kx (2 x N1 x Kx) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 x Kx (1 x N1 x Kx) | 5.12 x N1 x Kx (2 x N1 x Kx) |
| Note 1: Applies for RedCap UE of all power class. Note 2: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 3: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: When eDRX_INACTIVE=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX _INACTIVE cycles. Note 6: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 7: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 8: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 9: Kx = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. Kx = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |  |

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2B.2.11 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled stationary criterion or low mobility criterion or not-at-cell edge criterion, and

-	when UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, then the requirements in Table Table 5.1B.2.11-1 apply provided that eDRX_IDLE cycle is ≤ 10485.76 sec, or

-	when UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.11 shall apply with Tdetect, EUTRAN_RedCap_Relax, Tmeasure, EUTRAN _RedCap_Relax and Tevaluate, EUTRAN _RedCap_Relax defined in Table 5.1B.2.11-2.

Table 5.1B.2.11-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for inactive Redcap UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN_RedCap_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN_RedCap_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 x K4 (36 x M2 x K4) | 1.28 x M2 x K4 (4 x M2 x K4) | 5.12 x M2 x K4 (16 x M2 x K4) |
|  | 0.64 | 17.92 x K4 (28 x K4) | 1.28 x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
|  | 1.28 | 32 x K4 (25 x K4) | 1.28 x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
|  | 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
|  | 5.12 | 117.76 x K4 (23 x K4) | 5.12 x K4 (1 x K4) | 15.36 x K4 (3 x K4) |
|  | 10.24 | 235.52 x K4 (23 x K4) | 10.24 x K4 (1 x K4) | 30.72 x K4 (3 x K4) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. K4 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

Table 5.1B.2.11-2: Tdetect,EUTRAN_RedCap_Relax, Tmeasure,EUTRAN_RedCap_Relax, and Tevaluate,EUTRAN_RedCap_Relax for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE ≥ 20.48s, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_Inactive cycle length [s] | RAN DRX cycle length [s] | eDRX Inactive PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap_Relax [s] (number of RAN DRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_Relax [s] (number of RAN DRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_Relax [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 163.84 20.48 ≤ eDRX_INACTIVE cycle length ≤ 163.84 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx Kx (23 x Kx) | 0.32 x Kx (1 x Kx) | 0.64 x Kx (2 x Kx) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x Kx (1 x Kx) | 1.28 x Kx (2 x Kx) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x Kx (1 x Kx) | 2.56 x Kx (2 x Kx) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x Kx (1 x Kx) | 5.12 x Kx (2 x Kx) |
| NOTE 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 2: The number of RAN DRX cycles in this table is given for the RAN DRX cycles within RAN configured PTWs. NOTE 3: eDRX Inactive PTW in this table is RAN configured PTW [1]. NOTE 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 5: The eDRX_INACITVE cycle lengths are ran-ExtendedPagingCycle-r18 as specified in [2] NOTE 6: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. NOTE 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. NOTE 8: Kx = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. Kx = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

<End of Change 4>